| Table of Content |  |  |  |  |
| --- | --- | --- | --- | --- |
| Table of Content |  |  |  |  |
| Index | Component | Default | Address | Page |
| 1 | WAVE677DV_ARCH |  | 0x000 |  |
| 1.1 | WAVE6_HOST_IF |  | 0x000-0x7FF |  |
| 1.1.1 | WAVE6_HOST_IF_groups |  | 0x000-0x7FF |  |
| 1.1.1.69 | CMD_INIT_SEQ_OPTION | 0x00000000 | 0x204 |  |
| 1.1.1.70 | CMD_OPTION | 0x00000000 | 0x204 |  |
| 1.1.1.71 | CMD_SET_FB_OPTION | 0x00000000 | 0x204 |  |
| 1.1.1.72 | CMD_SET_PARAM_OPTION | 0x00000000 | 0x204 |  |
| 1.1.1.93 | CMD_BS_RD_PTR_DEC_PIC | 0x00000000 | 0x300 |  |
| 1.1.1.94 | CMD_BS_START | 0x00000000 | 0x300 |  |
| 1.1.1.95 | CMD_CREATE_INST_ADDR_WORK_BUF | 0x00000000 | 0x300 |  |
| 1.1.1.96 | CMD_ENC_SEQ_SET_PARAM_ENABLE | 0x00000000 | 0x300 |  |
| 1.1.1.97 | CMD_SET_FB_COMMON_PIC_INFO | 0x00000000 | 0x300 |  |
| 1.1.1.98 | RET_DEC_ADDR_USERDATA_BASE | 0x00000000 | 0x300 |  |
| 1.1.1.99 | RET_ENC_RD_PTR | 0x00000000 | 0x300 |  |
| 1.1.1.100 | RET_ENC_RD_PTR_QUERY_ENC_FW_STATUS | 0x00000000 | 0x300 |  |
| 1.1.1.101 | RET_QUERY_ENC_BS_RD_PTR | 0x00000000 | 0x300 |  |
| 1.1.1.103 | CMD_BS_WR_PTR | 0x00000000 | 0x304 |  |
| 1.1.1.104 | CMD_BS_WR_PTR_DEC_PIC | 0x00000000 | 0x304 |  |
| 1.1.1.105 | CMD_CREATE_INST_WORK_BUF_SIZE | 0x00000000 | 0x304 |  |
| 1.1.1.106 | CMD_ENC_SEQ_SRC_SIZE | 0x00000000 | 0x304 |  |
| 1.1.1.107 | CMD_SET_FB_PIC_SIZE | 0x00000000 | 0x304 |  |
| 1.1.1.108 | RET_DEC_USERDATA_SIZE | 0x00000000 | 0x304 |  |
| 1.1.1.109 | RET_ENC_WR_PTR | 0x00000000 | 0x304 |  |
| 1.1.1.110 | RET_ENC_WR_PTR_QUERY_ENC_FW_STATUS | 0x00000000 | 0x304 |  |
| 1.1.1.111 | RET_QUERY_ENC_BS_WR_PTR | 0x00000000 | 0x304 |  |
| 1.1.1.113 | CMD_BS_OPTIONS | 0x00000000 | 0x308 |  |
| 1.1.1.114 | CMD_BS_OPTIONS_ENC_PIC | 0x00000000 | 0x308 |  |
| 1.1.1.115 | CMD_ENC_SEQ_CUSTOM_MAP_ENDIAN | 0x00000000 | 0x308 |  |
| 1.1.1.116 | CMD_SET_FB_NUM_FB | 0x00000000 | 0x308 |  |
| 1.1.1.117 | RET_ENC_NUM_REQUIRED_FB | 0x00000000 | 0x308 |  |
| 1.1.1.119 | CMD_SET_FB_FBC_STRIDE | 0x00000000 | 0x30C |  |
| 1.1.1.120 | CMD_USE_SEC_AXI | 0x00000000 | 0x30C |  |
| 1.1.1.121 | CMD_USE_SEC_AXI_ENC_PIC | 0x00000000 | 0x30C |  |
| 1.1.1.122 | RET_DEC_BS_RD_PTR | 0x00000000 | 0x30C |  |
| 1.1.1.123 | RET_MIN_SRC_BUF_NUM | 0x00000000 | 0x30C |  |
| 1.1.1.125 | CMD_ENC_REPORT_PARAM | 0x00000000 | 0x310 |  |
| 1.1.1.126 | CMD_ENC_SEQ_PPS_PARAM | 0x00000000 | 0x310 |  |
| 1.1.1.127 | CMD_SEQ_CHANGE_ENABLE_FLAG | 0x00000000 | 0x310 |  |
| 1.1.1.128 | CMD_SET_FB_ADDR_LUMA_BASE0 | 0x00000000 | 0x310 |  |
| 1.1.1.129 | RET_DEC_SEQ_PARAM | 0x00000000 | 0x310 |  |
| 1.1.1.130 | RET_ENC_PIC_TYPE | 0x00000000 | 0x310 |  |
| 1.1.1.132 | CMD_ENC_SEQ_GOP_PARAM | 0x00000000 | 0x314 |  |
| 1.1.1.133 | CMD_SET_FB_ADDR_CB_BASE0 | 0x00000000 | 0x314 |  |
| 1.1.1.134 | RET_DEC_COLOR_SAMPLE_INFO | 0x00000000 | 0x314 |  |
| 1.1.1.135 | RET_ENC_PIC_POC | 0x00000000 | 0x314 |  |
| 1.1.1.137 | CMD_DEC_TEMPORAL_ID_PLUS1 | 0x00000000 | 0x318 |  |
| 1.1.1.138 | CMD_ENC_MV_HISTO_CLASS0 | 0x00000000 | 0x318 |  |
| 1.1.1.139 | CMD_ENC_SEQ_INTRA_PARAM | 0x00000000 | 0x318 |  |
| 1.1.1.140 | CMD_ENC_SEQ_INTRA_PARAM_AVC | 0x00000000 | 0x318 |  |
| 1.1.1.141 | CMD_SET_FB_ADDR_CR_BASE0 | 0x00000000 | 0x318 |  |
| 1.1.1.142 | CMD_SET_FB_ADDR_FBC_Y_OFFSET0 | 0x00000000 | 0x318 |  |
| 1.1.1.143 | RET_DEC_ASPECT_RATIO | 0x00000000 | 0x318 |  |
| 1.1.1.144 | RET_ENC_PIC_IDX | 0x00000000 | 0x318 |  |
| 1.1.1.146 | CMD_ENC_MV_HISTO_CLASS1 | 0x00000000 | 0x31C |  |
| 1.1.1.147 | CMD_ENC_SEQ_CONF_WIN_TOP_BOT | 0x00000000 | 0x31C |  |
| 1.1.1.148 | CMD_SET_FB_ADDR_FBC_C_OFFSET0 | 0x00000000 | 0x31C |  |
| 1.1.1.149 | RET_DEC_BIT_RATE | 0x00000000 | 0x31C |  |
| 1.1.1.150 | RET_ENC_PIC_SLICE_NUM | 0x00000000 | 0x31C |  |
| 1.1.1.152 | CMD_ENC_SEQ_CONF_WIN_LEFT_RIGHT | 0x00000000 | 0x320 |  |
| 1.1.1.153 | CMD_SET_FB_ADDR_COL0 | 0x00000000 | 0x320 |  |
| 1.1.1.154 | RET_DEC_FRAME_RATE_NR | 0x00000000 | 0x320 |  |
| 1.1.1.155 | RET_ENC_PIC_SKIP | 0x00000000 | 0x320 |  |
| 1.1.1.157 | CMD_ENC_SEQ_RDO_PARAM | 0x00000000 | 0x324 |  |
| 1.1.1.158 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE0 | 0x00000000 | 0x324 |  |
| 1.1.1.159 | RET_DEC_FRAME_RATE_DR | 0x00000000 | 0x324 |  |
| 1.1.1.160 | RET_ENC_PIC_NUM_INTRA | 0x00000000 | 0x324 |  |
| 1.1.1.162 | CMD_SET_FB_ADDR_LUMA_BASE1 | 0x00000000 | 0x328 |  |
| 1.1.1.163 | RET_DEC_NUM_REQURED_FB | 0x00000000 | 0x328 |  |
| 1.1.1.164 | RET_ENC_PIC_NUM_MERGE | 0x00000000 | 0x328 |  |
| 1.1.1.166 | CMD_ENC_SRC_PIC_IDX | 0x00000000 | 0x32C |  |
| 1.1.1.167 | CMD_SET_FB_ADDR_CB_BASE1 | 0x00000000 | 0x32C |  |
| 1.1.1.168 | RET_DEC_NUM_REORDER_DELAY | 0x00000000 | 0x32C |  |
| 1.1.1.169 | RET_ENC_PIC_NON_REF_PIC_FLAG | 0x00000000 | 0x32C |  |
| 1.1.1.171 | CMD_ENC_SEQ_INTRA_MIN_MAX_QP | 0x00000000 | 0x330 |  |
| 1.1.1.172 | CMD_ENC_SRC_ADDR_Y | 0x00000000 | 0x330 |  |
| 1.1.1.173 | CMD_SET_FB_ADDR_CR_BASE1 | 0x00000000 | 0x330 |  |
| 1.1.1.174 | CMD_SET_FB_ADDR_FBC_Y_OFFSET1 | 0x00000000 | 0x330 |  |
| 1.1.1.175 | RET_DEC_SUB_LAYER_INFO | 0x00000000 | 0x330 |  |
| 1.1.1.176 | RET_ENC_PIC_NUM_SKIP | 0x00000000 | 0x330 |  |
| 1.1.1.178 | CMD_ENC_SEQ_RC_FRAME_RATE | 0x00000000 | 0x334 |  |
| 1.1.1.179 | CMD_ENC_SRC_ADDR_U | 0x00000000 | 0x334 |  |
| 1.1.1.180 | CMD_SET_FB_ADDR_FBC_C_OFFSET1 | 0x00000000 | 0x334 |  |
| 1.1.1.181 | RET_DEC_NOTIFICATION | 0x00000000 | 0x334 |  |
| 1.1.1.182 | RET_ENC_PIC_AVG_CTU_QP | 0x00000000 | 0x334 |  |
| 1.1.1.184 | CMD_ENC_SRC_ADDR_V | 0x00000000 | 0x338 |  |
| 1.1.1.185 | CMD_SET_FB_ADDR_COL1 | 0x00000000 | 0x338 |  |
| 1.1.1.186 | RET_DEC_USERDATA_IDC | 0x00000000 | 0x338 |  |
| 1.1.1.187 | RET_ENC_PIC_BYTE | 0x00000000 | 0x338 |  |
| 1.1.1.189 | CMD_ENC_SRC_STRIDE | 0x00000000 | 0x33C |  |
| 1.1.1.190 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE1 | 0x00000000 | 0x33C |  |
| 1.1.1.191 | RET_DEC_PIC_SIZE | 0x00000000 | 0x33C |  |
| 1.1.1.192 | RET_ENC_GOP_PIC_IDX | 0x00000000 | 0x33C |  |
| 1.1.1.194 | CMD_ENC_SRC_FORMAT | 0x00000000 | 0x340 |  |
| 1.1.1.195 | CMD_SET_FB_ADDR_LUMA_BASE2 | 0x00000000 | 0x340 |  |
| 1.1.1.196 | RET_DEC_CROP_TOP_BOTTOM | 0x00000000 | 0x340 |  |
| 1.1.1.197 | RET_ENC_USED_SRC_IDX | 0x00000000 | 0x340 |  |
| 1.1.1.199 | CMD_SET_FB_ADDR_CB_BASE2 | 0x00000000 | 0x344 |  |
| 1.1.1.200 | RET_DEC_CROP_LEFT_RIGHT | 0x00000000 | 0x344 |  |
| 1.1.1.201 | RET_ENC_PIC_NUM | 0x00000000 | 0x344 |  |
| 1.1.1.203 | CMD_ENC_SEQ_RC_VBV_BUFFER_SIZE | 0x00000000 | 0x348 |  |
| 1.1.1.204 | CMD_ENC_SRC_AXI_SEL | 0x00000000 | 0x348 |  |
| 1.1.1.205 | CMD_SET_FB_ADDR_CR_BASE2 | 0x00000000 | 0x348 |  |
| 1.1.1.206 | CMD_SET_FB_ADDR_FBC_Y_OFFSET2 | 0x00000000 | 0x348 |  |
| 1.1.1.207 | RET_DEC_AU_START_POS | 0x00000000 | 0x348 |  |
| 1.1.1.208 | RET_ENC_VCL_NUT | 0x00000000 | 0x348 |  |
| 1.1.1.210 | CMD_ENC_CODE_OPTION | 0x00000000 | 0x34C |  |
| 1.1.1.211 | CMD_ENC_SEQ_INTER_MIN_MAX_QP | 0x00000000 | 0x34C |  |
| 1.1.1.212 | CMD_SET_FB_ADDR_FBC_C_OFFSET2 | 0x00000000 | 0x34C |  |
| 1.1.1.213 | RET_DEC_AU_END_POS | 0x00000000 | 0x34C |  |
| 1.1.1.214 | RET_ENC_PIC_PADD_BYTE | 0x00000000 | 0x34C |  |
| 1.1.1.216 | CMD_ENC_PIC_PARAM | 0x00000000 | 0x350 |  |
| 1.1.1.217 | CMD_ENC_SEQ_ROT_PARAM | 0x00000000 | 0x350 |  |
| 1.1.1.218 | CMD_SET_FB_ADDR_COL2 | 0x00000000 | 0x350 |  |
| 1.1.1.219 | RET_DEC_PIC_TYPE | 0x00000000 | 0x350 |  |
| 1.1.1.220 | RET_ENC_PIC_DIST_LOW | 0x00000000 | 0x350 |  |
| 1.1.1.222 | CMD_ENC_LONGTERM_PIC | 0x00000000 | 0x354 |  |
| 1.1.1.223 | CMD_ENC_SEQ_NUM_UNITS_IN_TICK | 0x00000000 | 0x354 |  |
| 1.1.1.224 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE2 | 0x00000000 | 0x354 |  |
| 1.1.1.225 | RET_DEC_PIC_POC | 0x00000000 | 0x354 |  |
| 1.1.1.226 | RET_ENC_PIC_DIST_HIGH | 0x00000000 | 0x354 |  |
| 1.1.1.228 | CMD_ENC_PREFIX_SEI_NAL_ADDR | 0x00000000 | 0x358 |  |
| 1.1.1.229 | CMD_ENC_SEQ_TIME_SCALE | 0x00000000 | 0x358 |  |
| 1.1.1.230 | CMD_SET_FB_ADDR_LUMA_BASE3 | 0x00000000 | 0x358 |  |
| 1.1.1.231 | RET_DEC_RECOVERY_POINT | 0x00000000 | 0x358 |  |
| 1.1.1.232 | RET_ENC_MAX_LATENCY_PICTURES | 0x00000000 | 0x358 |  |
| 1.1.1.234 | CMD_ENC_SEQ_NUM_TICKS_POC_DIFF_ONE | 0x00000000 | 0x35C |  |
| 1.1.1.235 | CMD_SET_FB_ADDR_CB_BASE3 | 0x00000000 | 0x35C |  |
| 1.1.1.236 | RET_DEC_DEBUG_INDEX | 0x00000000 | 0x35C |  |
| 1.1.1.238 | CMD_SET_FB_ADDR_CR_BASE3 | 0x00000000 | 0x360 |  |
| 1.1.1.239 | CMD_SET_FB_ADDR_FBC_Y_OFFSET3 | 0x00000000 | 0x360 |  |
| 1.1.1.240 | RET_DEC_DECODED_INDEX | 0x00000000 | 0x360 |  |
| 1.1.1.241 | RET_ENC_HISTO_CNT_0 | 0x00000000 | 0x360 |  |
| 1.1.1.243 | CMD_SET_FB_ADDR_FBC_C_OFFSET3 | 0x00000000 | 0x364 |  |
| 1.1.1.244 | RET_DEC_DISPLAY_INDEX | 0x00000000 | 0x364 |  |
| 1.1.1.245 | RET_ENC_HISTO_CNT_1 | 0x00000000 | 0x364 |  |
| 1.1.1.247 | RET_DEC_REALLOC_INDEX | 0x00000000 | 0x368 |  |
| 1.1.1.248 | RET_ENC_HISTO_CNT_2 | 0x00000000 | 0x368 |  |
| 1.1.1.250 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE3 | 0x00000000 | 0x36C |  |
| 1.1.1.251 | RET_DEC_DISP_IDC | 0x00000000 | 0x36C |  |
| 1.1.1.252 | RET_ENC_HISTO_CNT_3 | 0x00000000 | 0x36C |  |
| 1.1.1.254 | CMD_SET_FB_ADDR_LUMA_BASE4 | 0x00000000 | 0x370 |  |
| 1.1.1.255 | RET_DEC_NUM_ERR_CTB | 0x00000000 | 0x370 |  |
| 1.1.1.256 | RET_ENC_HISTO_CNT_4 | 0x00000000 | 0x370 |  |
| 1.1.1.258 | CMD_ENC_SEQ_VUI_RBSP_ADDR | 0x00000000 | 0x374 |  |
| 1.1.1.259 | CMD_SET_FB_ADDR_CB_BASE4 | 0x00000000 | 0x374 |  |
| 1.1.1.260 | RET_ENC_NUM_TILE_COL | 0x00000000 | 0x374 |  |
| 1.1.1.262 | CMD_ENC_SEQ_HRD_RBSP_ADDR | 0x00000000 | 0x378 |  |
| 1.1.1.263 | CMD_SET_FB_ADDR_CR_BASE4 | 0x00000000 | 0x378 |  |
| 1.1.1.264 | CMD_SET_FB_ADDR_FBC_Y_OFFSET4 | 0x00000000 | 0x378 |  |
| 1.1.1.265 | RET_ENC_NUM_TILE_ROW | 0x00000000 | 0x378 |  |
| 1.1.1.267 | CMD_SET_FB_ADDR_FBC_C_OFFSET4 | 0x00000000 | 0x37C |  |
| 1.1.1.268 | RET_ENC_PIC_WIDTH | 0x00000000 | 0x37C |  |
| 1.1.1.270 | CMD_ENC_SEQ_QROUND_OFFSET | 0x00000000 | 0x380 |  |
| 1.1.1.271 | CMD_SET_FB_ADDR_COL4 | 0x00000000 | 0x380 |  |
| 1.1.1.272 | ENC_PIC_HEIGHT | 0x00000000 | 0x380 |  |
| 1.1.1.274 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE4 | 0x00000000 | 0x384 |  |
| 1.1.1.276 | CMD_SET_FB_ADDR_LUMA_BASE5 | 0x00000000 | 0x388 |  |
| 1.1.1.278 | CMD_SET_FB_ADDR_CB_BASE5 | 0x00000000 | 0x38C |  |
| 1.1.1.280 | CMD_SET_FB_ADDR_CR_BASE5 | 0x00000000 | 0x390 |  |
| 1.1.1.281 | CMD_SET_FB_ADDR_FBC_Y_OFFSET5 | 0x00000000 | 0x390 |  |
| 1.1.1.283 | CMD_SET_FB_ADDR_FBC_C_OFFSET5 | 0x00000000 | 0x394 |  |
| 1.1.1.284 | RET_DEC_NUM_REQURED_COL_FB | 0x00000000 | 0x394 |  |
| 1.1.1.286 | CMD_SET_FB_ADDR_COL5 | 0x00000000 | 0x398 |  |
| 1.1.1.288 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE5 | 0x00000000 | 0x39C |  |
| 1.1.1.290 | CMD_SET_FB_ADDR_LUMA_BASE6 | 0x00000000 | 0x3A0 |  |
| 1.1.1.291 | RET_CORE_IDC | 0x00000000 | 0x3A0 |  |
| 1.1.1.293 | CMD_SET_FB_ADDR_CB_BASE6 | 0x00000000 | 0x3A4 |  |
| 1.1.1.294 | RET_DEC_PIC_PARAM | 0x00000000 | 0x3A4 |  |
| 1.1.1.296 | CMD_SET_FB_ADDR_CR_BASE6 | 0x00000000 | 0x3A8 |  |
| 1.1.1.297 | CMD_SET_FB_ADDR_FBC_Y_OFFSET6 | 0x00000000 | 0x3A8 |  |
| 1.1.1.298 | RET_ENC_CORE_IDC | 0x00000000 | 0x3A8 |  |
| 1.1.1.300 | CMD_SET_FB_ADDR_FBC_C_OFFSET6 | 0x00000000 | 0x3AC |  |
| 1.1.1.301 | RET_HOST_CMD_WARN_INFO | 0x00000000 | 0x3AC |  |
| 1.1.1.303 | RET_HOST_CMD_ERR_INFO | 0x00000000 | 0x3B0 |  |
| 1.1.1.305 | RET_HOST_CMD_SUCCESS | 0x00000000 | 0x3B4 |  |
| 1.1.1.307 | RET_ENC_SUM_ME0_X_DIR_LOWER | 0x00000000 | 0x3B8 |  |
| 1.1.1.309 | RET_ENC_SUM_ME0_X_DIR_HIGHER | 0x00000000 | 0x3BC |  |
| 1.1.1.311 | CMD_SET_FB_ADDR_FBC_Y_OFFSET7 | 0x00000000 | 0x3C0 |  |
| 1.1.1.312 | RET_ENC_SUM_ME0_Y_DIR_LOWER | 0x00000000 | 0x3C0 |  |
| 1.1.1.314 | RET_ENC_SUM_ME0_Y_DIR_HIGHER | 0x00000000 | 0x3C4 |  |
| 1.1.1.316 | RET_ENC_SUM_ME1_X_DIR_LOWER | 0x00000000 | 0x3C8 |  |
| 1.1.1.318 | RET_DEC_WARN_INFO | 0x00000000 | 0x3CC |  |
| 1.1.1.319 | RET_ENC_SUM_ME1_X_DIR_HIGHER | 0x00000000 | 0x3CC |  |
| 1.1.1.321 | CMD_SET_FB_ADDR_LUMA_BASE8 | 0x00000000 | 0x3D0 |  |
| 1.1.1.322 | RET_DEC_ERR_INFO | 0x00000000 | 0x3D0 |  |
| 1.1.1.323 | RET_ENC_SUM_ME1_Y_DIR_LOWER | 0x00000000 | 0x3D0 |  |
| 1.1.1.325 | CMD_SET_FB_ADDR_CB_BASE8 | 0x00000000 | 0x3D4 |  |
| 1.1.1.326 | RET_DEC_DECODING_SUCCESS | 0x00000000 | 0x3D4 |  |
| 1.1.1.327 | RET_ENC_SUM_ME1_Y_DIR_HIGHER | 0x00000000 | 0x3D4 |  |
| 1.1.1.329 | CMD_SET_FB_ADDR_CR_BASE8 | 0x00000000 | 0x3D8 |  |
| 1.1.1.330 | CMD_SET_FB_ADDR_FBC_Y_OFFSET8 | 0x00000000 | 0x3D8 |  |
| 1.1.1.332 | CMD_SET_FB_ADDR_FBC_C_OFFSET8 | 0x00000000 | 0x3DC |  |
| 1.1.1.334 | CMD_SET_FB_ADDR_COL8 | 0x00000000 | 0x3E0 |  |
| 1.1.1.336 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE8 | 0x00000000 | 0x3E4 |  |
| 1.1.1.338 | CMD_SET_FB_ADDR_LUMA_BASE9 | 0x00000000 | 0x3E8 |  |
| 1.1.1.340 | CMD_SET_FB_ADDR_CB_BASE9 | 0x00000000 | 0x3EC |  |
| 1.1.1.342 | CMD_SET_FB_ADDR_CR_BASE9 | 0x00000000 | 0x3F0 |  |
| 1.1.1.343 | CMD_SET_FB_ADDR_FBC_Y_OFFSET9 | 0x00000000 | 0x3F0 |  |
| 1.1.1.345 | CMD_SET_FB_ADDR_FBC_C_OFFSET9 | 0x00000000 | 0x3F4 |  |
| 1.1.1.347 | CMD_SET_FB_ADDR_COL9 | 0x00000000 | 0x3F8 |  |
| 1.1.1.348 | RET_ENC_SRC_DEBUG_0 | 0x00000000 | 0x3F8 |  |
| 1.1.1.350 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE9 | 0x00000000 | 0x3FC |  |
| 1.1.1.351 | RET_ENC_SRC_DEBUG_1 | 0x00000000 | 0x3FC |  |
| 1.1.1.353 | CMD_SET_FB_ADDR_LUMA_BASE10 | 0x00000000 | 0x400 |  |
| 1.1.1.355 | CMD_SET_FB_ADDR_CB_BASE10 | 0x00000000 | 0x404 |  |
| 1.1.1.357 | CMD_SET_FB_ADDR_CR_BASE10 | 0x00000000 | 0x408 |  |
| 1.1.1.358 | CMD_SET_FB_ADDR_FBC_Y_OFFSET10 | 0x00000000 | 0x408 |  |
| 1.1.1.360 | CMD_SET_FB_ADDR_FBC_C_OFFSET10 | 0x00000000 | 0x40C |  |
| 1.1.1.362 | CMD_SET_FB_ADDR_COL10 | 0x00000000 | 0x410 |  |
| 1.1.1.364 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE10 | 0x00000000 | 0x414 |  |
| 1.1.1.366 | CMD_SET_FB_ADDR_LUMA_BASE11 | 0x00000000 | 0x418 |  |
| 1.1.1.368 | CMD_SET_FB_ADDR_CB_BASE11 | 0x00000000 | 0x41C |  |
| 1.1.1.370 | CMD_SET_FB_ADDR_CR_BASE11 | 0x00000000 | 0x420 |  |
| 1.1.1.371 | CMD_SET_FB_ADDR_FBC_Y_OFFSET11 | 0x00000000 | 0x420 |  |
| 1.1.1.373 | CMD_SET_FB_ADDR_FBC_C_OFFSET11 | 0x00000000 | 0x424 |  |
| 1.1.1.375 | CMD_SET_FB_ADDR_COL11 | 0x00000000 | 0x428 |  |
| 1.1.1.377 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE11 | 0x00000000 | 0x42C |  |
| 1.1.1.379 | CMD_SET_FB_ADDR_LUMA_BASE12 | 0x00000000 | 0x430 |  |
| 1.1.1.381 | CMD_SET_FB_ADDR_CB_BASE12 | 0x00000000 | 0x434 |  |
| 1.1.1.383 | CMD_SET_FB_ADDR_CR_BASE12 | 0x00000000 | 0x438 |  |
| 1.1.1.384 | CMD_SET_FB_ADDR_FBC_Y_OFFSET12 | 0x00000000 | 0x438 |  |
| 1.1.1.386 | CMD_SET_FB_ADDR_FBC_C_OFFSET12 | 0x00000000 | 0x43C |  |
| 1.1.1.388 | CMD_SET_FB_ADDR_COL12 | 0x00000000 | 0x440 |  |
| 1.1.1.390 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE12 | 0x00000000 | 0x444 |  |
| 1.1.1.392 | CMD_SET_FB_ADDR_LUMA_BASE13 | 0x00000000 | 0x448 |  |
| 1.1.1.394 | CMD_SET_FB_ADDR_CB_BASE13 | 0x00000000 | 0x44C |  |
| 1.1.1.396 | CMD_SET_FB_ADDR_CR_BASE13 | 0x00000000 | 0x450 |  |
| 1.1.1.397 | CMD_SET_FB_ADDR_FBC_Y_OFFSET13 | 0x00000000 | 0x450 |  |
| 1.1.1.399 | CMD_SET_FB_ADDR_FBC_C_OFFSET13 | 0x00000000 | 0x454 |  |
| 1.1.1.401 | CMD_SET_FB_ADDR_COL13 | 0x00000000 | 0x458 |  |
| 1.1.1.403 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE13 | 0x00000000 | 0x45C |  |
| 1.1.1.405 | CMD_SET_FB_ADDR_LUMA_BASE14 | 0x00000000 | 0x460 |  |
| 1.1.1.407 | CMD_SET_FB_ADDR_CB_BASE14 | 0x00000000 | 0x464 |  |
| 1.1.1.409 | CMD_SET_FB_ADDR_CR_BASE14 | 0x00000000 | 0x468 |  |
| 1.1.1.410 | CMD_SET_FB_ADDR_FBC_Y_OFFSET14 | 0x00000000 | 0x468 |  |
| 1.1.1.412 | CMD_SET_FB_ADDR_FBC_C_OFFSET14 | 0x00000000 | 0x46C |  |
| 1.1.1.414 | CMD_SET_FB_ADDR_COL14 | 0x00000000 | 0x470 |  |
| 1.1.1.416 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE14 | 0x00000000 | 0x474 |  |
| 1.1.1.418 | CMD_SET_FB_ADDR_LUMA_BASE15 | 0x00000000 | 0x478 |  |
| 1.1.1.420 | CMD_SET_FB_ADDR_CB_BASE15 | 0x00000000 | 0x47C |  |
| 1.1.1.422 | CMD_SET_FB_ADDR_CR_BASE15 | 0x00000000 | 0x480 |  |
| 1.1.1.423 | CMD_SET_FB_ADDR_FBC_Y_OFFSET15 | 0x00000000 | 0x480 |  |
| 1.1.1.425 | CMD_SET_FB_ADDR_FBC_C_OFFSET15 | 0x00000000 | 0x484 |  |
| 1.1.1.427 | CMD_SET_FB_ADDR_COL15 | 0x00000000 | 0x488 |  |
| 1.1.1.429 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE15 | 0x00000000 | 0x48C |  |
| 1.1.1.432 | CMD_SET_FB_ADDR_DEFAULT_CDF | 0x00000000 | 0x494 |  |
| 1.1.1.434 | CMD_SET_FB_ADDR_SEGMAP | 0x00000000 | 0x498 |  |
| 1.1.1.436 | CMD_SET_FB_DEC_PP_PVRIC_CTRL | 0x00000000 | 0x49C |  |
| 1.1.1.439 | CMD_SET_FB_DEC_PP_SCL_PARAM | 0x00000000 | 0x4A4 |  |
| 1.1.1.448 | CMD_SET_FB_DEC_PP_CROP_POS | 0x00000000 | 0x4C4 |  |
| 1.1.1.450 | CMD_SET_FB_DEC_PP_CROP_SIZE | 0x00000000 | 0x4C8 |  |
| 1.1.1.452 | CMD_SET_FB_DEC_PP_AFBC_COMMON | 0x00000000 | 0x4CC |  |
| 1.1.1.457 | CMD_SET_FB_DEC_ADDR_COL_DUAL | 0x00000000 | 0x4DC |  |
| 1.1.1.1 | HOST_GLOBAL_WR | 0x00000000 | 0x000 |  |
| 1.1.1.2 | VCPU_CUR_PC | 0x00000000 | 0x004 |  |
| 1.1.1.3 | VCPU_CUR_LR | 0x00000000 | 0x008 |  |
| 1.1.1.4 | DBG_MSG_17 | 0x00000000 | 0x00C |  |
| 1.1.1.5 | DBG_MSG_18 | 0x00000000 | 0x010 |  |
| 1.1.1.6 | DBG_MSG_19 | 0x00000000 | 0x014 |  |
| 1.1.1.7 | DBG_MSG_20 | 0x00000000 | 0x018 |  |
| 1.1.1.8 | DBG_MSG_21 | 0x00000000 | 0x01C |  |
| 1.1.1.9 | VPU_FIO_ADDR | 0x00000000 | 0x020 |  |
| 1.1.1.10 | VPU_FIO_DATA | 0x00000000 | 0x024 |  |
| 1.1.1.11 | DBG_MSG_0 | 0x00000000 | 0x028 |  |
| 1.1.1.12 | DBG_MSG_1 | 0x00000000 | 0x02C |  |
| 1.1.1.13 | DBG_MSG_22 | 0x00000000 | 0x030 |  |
| 1.1.1.14 | VPU_VINT_REASON_CLR | 0x00000000 | 0x034 |  |
| 1.1.1.15 | VPU_HOST_INT_REQ | 0x00000000 | 0x038 |  |
| 1.1.1.16 | VPU_VINT_CLEAR | 0x00000000 | 0x03C |  |
| 1.1.1.17 | VPU_HINT_CLEAR | 0x00000000 | 0x040 |  |
| 1.1.1.18 | VPU_VPU_INT_STS | 0x00000000 | 0x044 |  |
| 1.1.1.19 | VPU_VINT_ENABLE | 0x00000000 | 0x048 |  |
| 1.1.1.20 | VPU_VINT_REASON | 0x00000000 | 0x04C |  |
| 1.1.1.22 | VCPU_RESTART | 0x00000000 | 0x058 |  |
| 1.1.1.24 | VPU_REMAP_CTRL | 0x00000000 | 0x060 |  |
| 1.1.1.25 | VPU_REMAP_VADDR | 0x00000000 | 0x064 |  |
| 1.1.1.26 | VPU_REMAP_PADDR | 0x00000000 | 0x068 |  |
| 1.1.1.27 | VPU_REMAP_CORE_START | 0x00000000 | 0x06C |  |
| 1.1.1.28 | VCPU_CMD_BUSY_STATUS | 0x00000000 | 0x070 |  |
| 1.1.1.29 | VPU_HALT_STATUS | 0x00000000 | 0x074 |  |
| 1.1.1.30 | VPU_VCPU_STATUS | 0x00000000 | 0x078 |  |
| 1.1.1.31 | VPU_BUSY_STATUS | 0x00000000 | 0x07C |  |
| 1.1.1.32 | RET_FIO_STATUS | 0x00000000 | 0x080 |  |
| 1.1.1.33 | DBG_MSG_3 | 0x00000000 | 0x084 |  |
| 1.1.1.34 | DBG_MSG_4 | 0x00000000 | 0x088 |  |
| 1.1.1.35 | DBG_MSG_5 | 0x00000000 | 0x08C |  |
| 1.1.1.36 | RET_PRODUCT_NAME | 0x00000000 | 0x090 |  |
| 1.1.1.37 | RET_PRODUCT_VERSION | 0x00000000 | 0x094 |  |
| 1.1.1.38 | RET_VCPU_CONFIG0 | 0x00000000 | 0x098 |  |
| 1.1.1.39 | RET_VCPU_CONFIG1 | 0x00000000 | 0x09C |  |
| 1.1.1.40 | RET_CODEC_STD | 0x00000000 | 0x0A0 |  |
| 1.1.1.41 | RET_CONF_DATE | 0x00000000 | 0x0A4 |  |
| 1.1.1.42 | RET_CONF_REVISION | 0x00000000 | 0x0A8 |  |
| 1.1.1.43 | RET_CONF_TYPE | 0x00000000 | 0x0AC |  |
| 1.1.1.44 | RET_VCORE0_CFG | 0x00000000 | 0x0B0 |  |
| 1.1.1.45 | RET_VCORE1_CFG | 0x00000000 | 0x0B4 |  |
| 1.1.1.46 | RET_VCORE2_CFG | 0x00000000 | 0x0B8 |  |
| 1.1.1.47 | RET_VCORE3_CFG | 0x00000000 | 0x0BC |  |
| 1.1.1.48 | VPU_RET_VCORE_PRESENT | 0x00000000 | 0x0C0 |  |
| 1.1.1.49 | DBG_MSG_6 | 0x00000000 | 0x0C4 |  |
| 1.1.1.50 | DBG_MSG_7 | 0x00000000 | 0x0C8 |  |
| 1.1.1.51 | DBG_MSG_8 | 0x00000000 | 0x0CC |  |
| 1.1.1.52 | DBG_MSG_9 | 0x00000000 | 0x0D0 |  |
| 1.1.1.54 | DBG_MSG_10 | 0x00000000 | 0x0D8 |  |
| 1.1.1.55 | DBG_MSG_11 | 0x00000000 | 0x0DC |  |
| 1.1.1.56 | DBG_MSG_12 | 0x00000000 | 0x0E0 |  |
| 1.1.1.57 | DBG_MSG_13 | 0x00000000 | 0x0E4 |  |
| 1.1.1.58 | DBG_MSG_14 | 0x00000000 | 0x0E8 |  |
| 1.1.1.59 | DBG_MSG_15 | 0x00000000 | 0x0EC |  |
| 1.1.1.60 | VPU_DBG_SW_UART_STATUS | 0x00000000 | 0x0F0 |  |
| 1.1.1.61 | VPU_DBG_SW_UART_TX | 0x00000000 | 0x0F4 |  |
| 1.1.1.62 | VPU_DBG_REG_0 | 0x00000000 | 0x0F8 |  |
| 1.1.1.63 | VPU_DBG_REG_1 | 0x00000000 | 0x0FC |  |
| 1.1.1.65 | VPU_SUB_FRAME_SYNC_CTRL | 0x00000000 | 0x11C |  |
| 1.1.1.67 | COMMAND | 0x00000000 | 0x200 |  |
| 1.1.1.68 | CMD_DEC_PIC_OPTION | 0x00000000 | 0x204 |  |
| 1.1.1.73 | RET_SUCCESS | 0x00000000 | 0x208 |  |
| 1.1.1.74 | RET_FAIL_REASON | 0x00000000 | 0x20C |  |
| 1.1.1.75 | CMD_INSTANCE_INFO | 0x00000000 | 0x210 |  |
| 1.1.1.76 | CMD_QUE_FULL_IDC | 0x00000000 | 0x214 |  |
| 1.1.1.77 | RET_QUE_EMPTY_IDC | 0x00000000 | 0x218 |  |
| 1.1.1.78 | CMD_DONE_INST_IDC | 0x00000000 | 0x21C |  |
| 1.1.1.79 | RET_CREATE_INSTANCE_ID | 0x00000000 | 0x220 |  |
| 1.1.1.81 | RET_CMD_CQ_IN_TICK | 0x00000000 | 0x23C |  |
| 1.1.1.82 | RET_CMD_FW_RUN_TICK | 0x00000000 | 0x240 |  |
| 1.1.1.83 | RET_CMD_HW_RUN_TICK | 0x00000000 | 0x244 |  |
| 1.1.1.84 | RET_CMD_HW_DONE_TICK | 0x00000000 | 0x248 |  |
| 1.1.1.85 | RET_CMD_FW_DONE_TICK | 0x00000000 | 0x24C |  |
| 1.1.1.86 | RET_CMD_RQ_OUT_TICK | 0x00000000 | 0x250 |  |
| 1.1.1.87 | RET_CMD_FW_RRE_RUN_TICK | 0x00000000 | 0x254 |  |
| 1.1.1.88 | RET_CMD_HW_RRE_RUN_TICK | 0x00000000 | 0x258 |  |
| 1.1.1.89 | RET_CMD_HW_RRE_DONE_TICK | 0x00000000 | 0x25C |  |
| 1.1.1.90 | RET_CMD_FW_RRE_DONE_TICK | 0x00000000 | 0x260 |  |
| 1.1.1.92 | CMD_BS_RD_PTR | 0x00000000 | 0x300 |  |
| 1.1.1.102 | CMD_BS_SIZE | 0x00000000 | 0x304 |  |
| 1.1.1.112 | CMD_BS_OPTION | 0x00000000 | 0x308 |  |
| 1.1.1.118 | CMD_ENC_SEQ_PARAM | 0x00000000 | 0x30C |  |
| 1.1.1.124 | CMD_CREATE_INST_BS_PARAM | 0x00000000 | 0x310 |  |
| 1.1.1.131 | CMD_DEC_SEI_MASK | 0x00000000 | 0x314 |  |
| 1.1.1.136 | CMD_CREATE_ADDR_EXT | 0x00000000 | 0x318 |  |
| 1.1.1.145 | CMD_DEC_FORCE_FB_LATENCY_PLUS1 | 0x00000000 | 0x31C |  |
| 1.1.1.151 | CMD_ENC_CUSTOM_MAP_OPTION_PARAM | 0x00000000 | 0x320 |  |
| 1.1.1.156 | CMD_ENC_CUSTOM_MAP_OPTION_ADDR | 0x00000000 | 0x324 |  |
| 1.1.1.161 | CMD_ENC_SEQ_SLICE_PARAM | 0x00000000 | 0x328 |  |
| 1.1.1.165 | CMD_ENC_SEQ_INTRA_REFRESH | 0x00000000 | 0x32C |  |
| 1.1.1.170 | CMD_CREATE_INST_CORE_INFO | 0x00000000 | 0x330 |  |
| 1.1.1.177 | CMD_CREATE_INST_PRIORITY | 0x00000000 | 0x334 |  |
| 1.1.1.183 | CMD_ENC_SEQ_RC_TARGET_RATE | 0x00000000 | 0x338 |  |
| 1.1.1.188 | CMD_ENC_SEQ_RC_PARAM | 0x00000000 | 0x33C |  |
| 1.1.1.193 | CMD_ENC_SEQ_HVS_PARAM | 0x00000000 | 0x340 |  |
| 1.1.1.198 | CMD_ENC_SEQ_RC_MAX_BITRATE | 0x00000000 | 0x344 |  |
| 1.1.1.202 | CMD_CREATE_INST_ADDR_TEMP_BASE | 0x00000000 | 0x348 |  |
| 1.1.1.209 | CMD_CREATE_INST_TEMP_SIZE | 0x00000000 | 0x34C |  |
| 1.1.1.215 | CMD_CREATE_INST_ADDR_SEC_AXI | 0x00000000 | 0x350 |  |
| 1.1.1.221 | CMD_CREATE_INST_SEC_AXI_SIZE | 0x00000000 | 0x354 |  |
| 1.1.1.227 | CMD_CREATE_INST_ADDR_AR_TABLE | 0x00000000 | 0x358 |  |
| 1.1.1.233 | CMD_ENC_PREFIX_SEI_INFO | 0x00000000 | 0x35C |  |
| 1.1.1.237 | CMD_ENC_SUFFIX_SEI_NAL_ADDR | 0x00000000 | 0x360 |  |
| 1.1.1.242 | CMD_ENC_SUFFIX_SEI_INFO | 0x00000000 | 0x364 |  |
| 1.1.1.246 | CMD_SET_FB_ADDR_COL3 | 0x00000000 | 0x368 |  |
| 1.1.1.249 | CMD_ENC_SEQ_BG_PARAM | 0x00000000 | 0x36C |  |
| 1.1.1.253 | CMD_ENC_SEQ_NON_VCL_PARAM | 0x00000000 | 0x370 |  |
| 1.1.1.257 | CMD_ENC_CSC_COEFF_0 | 0x00000000 | 0x374 |  |
| 1.1.1.261 | CMD_ENC_CSC_COEFF_1 | 0x00000000 | 0x378 |  |
| 1.1.1.266 | CMD_ENC_CSC_COEFF_2 | 0x00000000 | 0x37C |  |
| 1.1.1.269 | CMD_ENC_CSC_COEFF_3 | 0x00000000 | 0x380 |  |
| 1.1.1.273 | CMD_ENC_SEQ_QUANT_PARAM_0 | 0x00000000 | 0x384 |  |
| 1.1.1.275 | CMD_ENC_SEQ_QUANT_PARAM_1 | 0x00000000 | 0x388 |  |
| 1.1.1.277 | CMD_ENC_SEQ_CUSTOM_GOP_PARAM | 0x00000000 | 0x38C |  |
| 1.1.1.279 | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_0 | 0x00000000 | 0x390 |  |
| 1.1.1.282 | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_1 | 0x00000000 | 0x394 |  |
| 1.1.1.285 | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_2 | 0x00000000 | 0x398 |  |
| 1.1.1.287 | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_3 | 0x00000000 | 0x39C |  |
| 1.1.1.289 | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_4 | 0x00000000 | 0x3A0 |  |
| 1.1.1.292 | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_5 | 0x00000000 | 0x3A4 |  |
| 1.1.1.295 | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_6 | 0x00000000 | 0x3A8 |  |
| 1.1.1.299 | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_7 | 0x00000000 | 0x3AC |  |
| 1.1.1.302 | CMD_SET_FB_ADDR_COL6 | 0x00000000 | 0x3B0 |  |
| 1.1.1.304 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE6 | 0x00000000 | 0x3B4 |  |
| 1.1.1.306 | CMD_SET_FB_ADDR_LUMA_BASE7 | 0x00000000 | 0x3B8 |  |
| 1.1.1.308 | CMD_SET_FB_ADDR_CB_BASE7 | 0x00000000 | 0x3BC |  |
| 1.1.1.310 | CMD_SET_FB_ADDR_CR_BASE7 | 0x00000000 | 0x3C0 |  |
| 1.1.1.313 | CMD_SET_FB_ADDR_FBC_C_OFFSET7 | 0x00000000 | 0x3C4 |  |
| 1.1.1.315 | CMD_SET_FB_ADDR_COL7 | 0x00000000 | 0x3C8 |  |
| 1.1.1.317 | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE7 | 0x00000000 | 0x3CC |  |
| 1.1.1.320 | CMD_ENC_SEQ_TILE_PARAM | 0x00000000 | 0x3D0 |  |
| 1.1.1.324 | CMD_ENC_CUSTOM_LAMBDA_DATA_0 | 0x00000000 | 0x3D4 |  |
| 1.1.1.328 | CMD_ENC_CUSTOM_LAMBDA_DATA_1 | 0x00000000 | 0x3D8 |  |
| 1.1.1.331 | CMD_ENC_CUSTOM_LAMBDA_DATA_2 | 0x00000000 | 0x3DC |  |
| 1.1.1.333 | CMD_ENC_CUSTOM_LAMBDA_DATA_3 | 0x00000000 | 0x3E0 |  |
| 1.1.1.335 | CMD_ENC_CUSTOM_LAMBDA_DATA_4 | 0x00000000 | 0x3E4 |  |
| 1.1.1.337 | CMD_ENC_CUSTOM_LAMBDA_DATA_5 | 0x00000000 | 0x3E8 |  |
| 1.1.1.339 | CMD_ENC_CUSTOM_LAMBDA_DATA_6 | 0x00000000 | 0x3EC |  |
| 1.1.1.341 | CMD_ENC_CUSTOM_LAMBDA_DATA_7 | 0x00000000 | 0x3F0 |  |
| 1.1.1.344 | CMD_ENC_CUSTOM_LAMBDA_DATA_8 | 0x00000000 | 0x3F4 |  |
| 1.1.1.346 | CMD_ENC_CUSTOM_LAMBDA_DATA_9 | 0x00000000 | 0x3F8 |  |
| 1.1.1.349 | CMD_ENC_CUSTOM_LAMBDA_DATA_10 | 0x00000000 | 0x3FC |  |
| 1.1.1.352 | CMD_ENC_CUSTOM_LAMBDA_DATA_11 | 0x00000000 | 0x400 |  |
| 1.1.1.354 | CMD_ENC_CUSTOM_LAMBDA_DATA_12 | 0x00000000 | 0x404 |  |
| 1.1.1.356 | CMD_ENC_CUSTOM_LAMBDA_DATA_13 | 0x00000000 | 0x408 |  |
| 1.1.1.359 | CMD_ENC_CUSTOM_LAMBDA_DATA_14 | 0x00000000 | 0x40C |  |
| 1.1.1.361 | CMD_ENC_CUSTOM_LAMBDA_DATA_15 | 0x00000000 | 0x410 |  |
| 1.1.1.363 | CMD_ENC_CUSTOM_LAMBDA_DATA_16 | 0x00000000 | 0x414 |  |
| 1.1.1.365 | CMD_ENC_CUSTOM_LAMBDA_DATA_17 | 0x00000000 | 0x418 |  |
| 1.1.1.367 | CMD_ENC_CUSTOM_LAMBDA_DATA_18 | 0x00000000 | 0x41C |  |
| 1.1.1.369 | CMD_ENC_CUSTOM_LAMBDA_DATA_19 | 0x00000000 | 0x420 |  |
| 1.1.1.372 | CMD_ENC_CUSTOM_LAMBDA_DATA_20 | 0x00000000 | 0x424 |  |
| 1.1.1.374 | CMD_ENC_CUSTOM_LAMBDA_DATA_21 | 0x00000000 | 0x428 |  |
| 1.1.1.376 | CMD_ENC_CUSTOM_LAMBDA_DATA_22 | 0x00000000 | 0x42C |  |
| 1.1.1.378 | CMD_ENC_CUSTOM_LAMBDA_DATA_23 | 0x00000000 | 0x430 |  |
| 1.1.1.380 | CMD_ENC_CUSTOM_LAMBDA_DATA_24 | 0x00000000 | 0x434 |  |
| 1.1.1.382 | CMD_ENC_CUSTOM_LAMBDA_DATA_25 | 0x00000000 | 0x438 |  |
| 1.1.1.385 | CMD_ENC_CUSTOM_LAMBDA_DATA_26 | 0x00000000 | 0x43C |  |
| 1.1.1.387 | CMD_ENC_CUSTOM_LAMBDA_DATA_27 | 0x00000000 | 0x440 |  |
| 1.1.1.389 | CMD_ENC_CUSTOM_LAMBDA_DATA_28 | 0x00000000 | 0x444 |  |
| 1.1.1.391 | CMD_ENC_CUSTOM_LAMBDA_DATA_29 | 0x00000000 | 0x448 |  |
| 1.1.1.393 | CMD_ENC_CUSTOM_LAMBDA_DATA_30 | 0x00000000 | 0x44C |  |
| 1.1.1.395 | CMD_ENC_CUSTOM_LAMBDA_DATA_31 | 0x00000000 | 0x450 |  |
| 1.1.1.398 | CMD_ENC_CUSTOM_LAMBDA_DATA_32 | 0x00000000 | 0x454 |  |
| 1.1.1.400 | CMD_ENC_CUSTOM_LAMBDA_DATA_33 | 0x00000000 | 0x458 |  |
| 1.1.1.402 | CMD_ENC_CUSTOM_LAMBDA_DATA_34 | 0x00000000 | 0x45C |  |
| 1.1.1.404 | CMD_ENC_CUSTOM_LAMBDA_DATA_35 | 0x00000000 | 0x460 |  |
| 1.1.1.406 | CMD_ENC_CUSTOM_LAMBDA_DATA_36 | 0x00000000 | 0x464 |  |
| 1.1.1.408 | CMD_ENC_CUSTOM_LAMBDA_DATA_37 | 0x00000000 | 0x468 |  |
| 1.1.1.411 | CMD_ENC_CUSTOM_LAMBDA_DATA_38 | 0x00000000 | 0x46C |  |
| 1.1.1.413 | CMD_ENC_CUSTOM_LAMBDA_DATA_39 | 0x00000000 | 0x470 |  |
| 1.1.1.415 | CMD_ENC_CUSTOM_LAMBDA_DATA_40 | 0x00000000 | 0x474 |  |
| 1.1.1.417 | CMD_ENC_CUSTOM_LAMBDA_DATA_41 | 0x00000000 | 0x478 |  |
| 1.1.1.419 | CMD_ENC_CUSTOM_LAMBDA_DATA_42 | 0x00000000 | 0x47C |  |
| 1.1.1.421 | CMD_ENC_CUSTOM_LAMBDA_DATA_43 | 0x00000000 | 0x480 |  |
| 1.1.1.424 | CMD_ENC_CUSTOM_LAMBDA_DATA_44 | 0x00000000 | 0x484 |  |
| 1.1.1.426 | CMD_ENC_CUSTOM_LAMBDA_DATA_45 | 0x00000000 | 0x488 |  |
| 1.1.1.428 | CMD_ENC_CUSTOM_LAMBDA_DATA_46 | 0x00000000 | 0x48C |  |
| 1.1.1.430 | CMD_ENC_CUSTOM_LAMBDA_DATA_47 | 0x00000000 | 0x490 |  |
| 1.1.1.431 | CMD_ENC_CUSTOM_LAMBDA_DATA_48 | 0x00000000 | 0x494 |  |
| 1.1.1.433 | CMD_ENC_CUSTOM_LAMBDA_DATA_49 | 0x00000000 | 0x498 |  |
| 1.1.1.435 | CMD_ENC_CUSTOM_LAMBDA_DATA_50 | 0x00000000 | 0x49C |  |
| 1.1.1.437 | CMD_ENC_CUSTOM_LAMBDA_DATA_51 | 0x00000000 | 0x4A0 |  |
| 1.1.1.438 | CMD_ENC_SEQ_TEMPORAL_LAYER_0_QP | 0x00000000 | 0x4A4 |  |
| 1.1.1.440 | CMD_ENC_SEQ_TEMPORAL_LAYER_1_QP | 0x00000000 | 0x4A8 |  |
| 1.1.1.441 | CMD_ENC_SEQ_TEMPORAL_LAYER_2_QP | 0x00000000 | 0x4AC |  |
| 1.1.1.442 | CMD_ENC_SEQ_TEMPORAL_LAYER_3_QP | 0x00000000 | 0x4B0 |  |
| 1.1.1.443 | CMD_ENC_SEQ_SCL_SRC_SIZE | 0x00000000 | 0x4B4 |  |
| 1.1.1.444 | CMD_ENC_SEQ_SCL_PARAM | 0x00000000 | 0x4B8 |  |
| 1.1.1.445 | CMD_ENC_SEQ_Y2Y_PARAM | 0x00000000 | 0x4BC |  |
| 1.1.1.446 | CMD_SET_FB_DEC_PP_CROP_PARAM | 0x00000000 | 0x4C0 |  |
| 1.1.1.447 | CMD_ENC_SEQ_SFS_PARAM | 0x00000000 | 0x4C4 |  |
| 1.1.1.449 | CMD_ENC_SEQ_CROP_ENABLE | 0x00000000 | 0x4C8 |  |
| 1.1.1.451 | CMD_ENC_SEQ_CROP_START_POS | 0x00000000 | 0x4CC |  |
| 1.1.1.453 | CMD_ENC_SEQ_CROP_SRC_SIZE | 0x00000000 | 0x4D0 |  |
| 1.1.1.455 | CMD_ENC_SEQ_Y2Y_DATA_0 | 0x00000000 | 0x4D8 |  |
| 1.1.1.456 | CMD_ENC_SEQ_Y2Y_DATA_1 | 0x00000000 | 0x4DC |  |
| 1.1.1.458 | CMD_ENC_SEQ_Y2Y_DATA_2 | 0x00000000 | 0x4E0 |  |
| 1.1.1.459 | CMD_ENC_SEQ_INPUT_RINGBUF | 0x00000000 | 0x4E4 |  |
| 1.1.1.461 | VPU_PDBG_CTRL | 0x00000000 | 0x600 |  |
| 1.1.1.462 | VPU_PDBG_IDX_REG | 0x00000000 | 0x604 |  |
| 1.1.1.463 | VPU_PDBG_WDATA_REG | 0x00000000 | 0x608 |  |
| 1.1.1.464 | VPU_PDBG_RDATA_REG | 0x00000000 | 0x60C |  |
| 1.1.1.465 | VPU_PDBG_STEP_MASK | 0x00000000 | 0x610 |  |

| 1.0 |  |  | WAVE677DV_ARCH |  |  | 0x000 |
| --- | --- | --- | --- | --- | --- | --- |
| offset | 0 | external |  |  | size | 2048 |
| {vendor=ChipsnMedia;library=ip;version=v1.0.0} |  |  |  |  |  |  |

| 1.1.0 |  |  | signals |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| name | port type | description |  |  |  |  |
| CMD_DEC_PIC_OPTION_signal | in |  |  |  |  |  |
| CMD_BS_RD_PTR_signal | in |  |  |  |  |  |
| CMD_BS_SIZE_signal | in |  |  |  |  |  |
| CMD_BS_OPTION_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_PARAM_signal | in |  |  |  |  |  |
| CMD_CREATE_INST_BS_PARAM_signal | in |  |  |  |  |  |
| CMD_DEC_SEI_MASK_signal | in |  |  |  |  |  |
| CMD_CREATE_ADDR_EXT_signal | in |  |  |  |  |  |
| CMD_DEC_FORCE_FB_LATENCY_PLUS1_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_MAP_OPTION_PARAM_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_MAP_OPTION_ADDR_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_SLICE_PARAM_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_INTRA_REFRESH_signal | in |  |  |  |  |  |
| CMD_CREATE_INST_CORE_INFO_signal | in |  |  |  |  |  |
| CMD_CREATE_INST_PRIORITY_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_RC_TARGET_RATE_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_RC_PARAM_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_HVS_PARAM_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_RC_MAX_BITRATE_signal | in |  |  |  |  |  |
| CMD_CREATE_INST_ADDR_TEMP_BASE_signal | in |  |  |  |  |  |
| CMD_CREATE_INST_TEMP_SIZE_signal | in |  |  |  |  |  |
| CMD_CREATE_INST_ADDR_SEC_AXI_signal | in |  |  |  |  |  |
| CMD_CREATE_INST_SEC_AXI_SIZE_signal | in |  |  |  |  |  |
| CMD_CREATE_INST_ADDR_AR_TABLE_signal | in |  |  |  |  |  |
| CMD_ENC_PREFIX_SEI_INFO_signal | in |  |  |  |  |  |
| CMD_ENC_SUFFIX_SEI_NAL_ADDR_signal | in |  |  |  |  |  |
| CMD_ENC_SUFFIX_SEI_INFO_signal | in |  |  |  |  |  |
| CMD_SET_FB_ADDR_COL3_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_BG_PARAM_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_NON_VCL_PARAM_signal | in |  |  |  |  |  |
| CMD_ENC_CSC_COEFF_0_signal | in |  |  |  |  |  |
| CMD_ENC_CSC_COEFF_1_signal | in |  |  |  |  |  |
| CMD_ENC_CSC_COEFF_2_signal | in |  |  |  |  |  |
| CMD_ENC_CSC_COEFF_3_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_QUANT_PARAM_0_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_QUANT_PARAM_1_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_CUSTOM_GOP_PARAM_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_0_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_1_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_2_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_3_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_4_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_5_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_6_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_7_signal | in |  |  |  |  |  |
| CMD_SET_FB_ADDR_COL6_signal | in |  |  |  |  |  |
| CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE6_signal | in |  |  |  |  |  |
| CMD_SET_FB_ADDR_LUMA_BASE7_signal | in |  |  |  |  |  |
| CMD_SET_FB_ADDR_CB_BASE7_signal | in |  |  |  |  |  |
| CMD_SET_FB_ADDR_CR_BASE7_signal | in |  |  |  |  |  |
| CMD_SET_FB_ADDR_FBC_C_OFFSET7_signal | in |  |  |  |  |  |
| CMD_SET_FB_ADDR_COL7_signal | in |  |  |  |  |  |
| CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE7_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_TILE_PARAM_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_0_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_1_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_2_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_3_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_4_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_5_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_6_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_7_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_8_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_9_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_10_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_11_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_12_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_13_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_14_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_15_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_16_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_17_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_18_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_19_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_20_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_21_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_22_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_23_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_24_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_25_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_26_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_27_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_28_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_29_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_30_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_31_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_32_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_33_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_34_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_35_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_36_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_37_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_38_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_39_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_40_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_41_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_42_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_43_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_44_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_45_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_46_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_48_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_49_signal | in |  |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_50_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_TEMPORAL_LAYER_0_QP_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_SFS_PARAM_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_CROP_ENABLE_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_CROP_START_POS_signal | in |  |  |  |  |  |
| CMD_ENC_SEQ_Y2Y_DATA_1_signal | in |  |  |  |  |  |

| 1.1.1.0 |  |  |  | WAVE6_HOST_IF |  |  |  |  | 0x000 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset | 0 | external |  |  | repeat |  |  | size |  | 2048 |

| 1.1.1.69.0 |  |  |  | WAVE6_HOST_IF_groups |  |  |  |  | 0x000 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset | 0 | external |  |  | repeat |  |  | size |  | 2048 |
| NOTE: Basically, the reset values of command I/O interface are pseudo-values. But it can be initialized as 0x0 when the host run INIT_VPU of ref_sw and cleared it.{range=2048} |  |  |  |  |  |  |  |  |  |  |

| 1.1.1.70.0 |  |  |  |  |  |  |  | CMD_INIT_SEQ_OPTION |  |  |  |  |  |  | 0x204 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 516 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Decode picture header option{alt_mode=CMD_DEC_PIC_OPTION_signal=1;virtualpairUid=2022-09-0709:27:31T173acdd5-3dd6-4841-8584-d31cbbc63a2c;alternate_reg=CMD_DEC_PIC_OPTION} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 5:0 | INIT_SEQ_OPTION |  |  | rw | ro |  | 0x0 |  |  | 0x01: INIT_SEQ 0x11: INIT_SEQ (w/ Thumbnail mode){has_reset=0} |  |  |  |  |  |

| 1.1.1.71.0 |  |  |  |  |  |  |  | CMD_OPTION |  |  |  |  |  |  | 0x204 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 516 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Command option{alt_mode=CMD_DEC_PIC_OPTION_signal=2;virtualpairUid=2022-09-0709:27:31T173acdd5-3dd6-4841-8584-d31cbbc63a2c;alternate_reg=CMD_DEC_PIC_OPTION} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | if COMMAND is QUERY, CMD_OPTION is as follows. 2 : ENC_GET_RESULT, DEC_GET_RESULT 3 : ENC_FW_STATUS (for debugging purpose) 6 : GET_WR_PTR otherwise, refer to description in the relevant command.{has_reset=0} |  |  |  |  |  |

| 1.1.1.72.0 |  |  |  |  |  |  |  | CMD_SET_FB_OPTION |  |  |  |  |  |  | 0x204 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 516 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_SET_FB_OPTION {alt_mode=CMD_DEC_PIC_OPTION_signal=3;virtualpairUid=2022-09-0709:27:31T173acdd5-3dd6-4841-8584-d31cbbc63a2c;alternate_reg=CMD_DEC_PIC_OPTION} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:27 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | RSVD{has_reset=0} |  |  |  |  |  |
| 26:26 | NON_REF_FBC_WRITING |  |  | rw | ro |  | 0x0 |  |  | 0: disable FBC writing for non-reference pictures. (default) 1: enable FBC writing for all pictures for verification.{has_reset=0} |  |  |  |  |  |
| 25:20 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 19:16 | FB_ENDIAN |  |  | rw | ro |  | 0x0 |  |  | Framebuffer endianness. This is applied to only decoder.{has_reset=0} |  |  |  |  |  |
| 4:4 | SETUP_DONE |  |  | rw | ro |  | 0x0 |  |  | Setup Done Please set this flag as 1 when setup for frame buffer is done{has_reset=0} |  |  |  |  |  |
| 3:3 | FB_GROUP_INDICATOR |  |  | rw | ro |  | 0x0 |  |  | shall be 1{has_reset=0} |  |  |  |  |  |
| 2:0 | SET_FB_MODE |  |  | rw | ro |  | 0x0 |  |  | *0: Set FB* 1: Update FB{has_reset=0} |  |  |  |  |  |

| 1.1.1.93.0 |  |  |  |  |  |  |  | CMD_SET_PARAM_OPTION |  |  |  |  |  |  | 0x204 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 516 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_SET_PARAM_OPTION{alt_mode=CMD_DEC_PIC_OPTION_signal=4;virtualpairUid=2022-09-0709:27:31T173acdd5-3dd6-4841-8584-d31cbbc63a2c;alternate_reg=CMD_DEC_PIC_OPTION} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:2 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 1:0 | SET_PARAM_OPTION |  |  | rw | ro |  | 0x0 |  |  | ENC_SET_PARAM command option *0x0 : SET_PARAM* 0x1 : CHANGE_PARAM{has_reset=0} |  |  |  |  |  |

| 1.1.1.94.0 |  |  |  |  |  |  |  | CMD_BS_RD_PTR_DEC_PIC |  |  |  |  |  |  | 0x300 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 768 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| read pointer on Bitstream buffer{alt_mode=CMD_BS_RD_PTR_signal=1;virtualpairUid=2022-09-0709:27:31T927ba960-7865-42b9-b066-08646b1e13ba;alternate_reg=CMD_BS_RD_PTR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | RD_PTR |  |  | rw | ro |  | 0x0 |  |  | Start address of bitstream buffer (in line buffer mode only){has_reset=0} |  |  |  |  |  |

| 1.1.1.95.0 |  |  |  |  |  |  |  | CMD_BS_START |  |  |  |  |  |  | 0x300 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 768 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Bitstream buffer start address{alt_mode=CMD_BS_RD_PTR_signal=2;virtualpairUid=2022-09-0709:27:31T927ba960-7865-42b9-b066-08646b1e13ba;alternate_reg=CMD_BS_RD_PTR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | BS_START_ADDR |  |  | rw | ro |  | 0x0 |  |  | Start address of bitstream buffer (in line buffer mode only) It should be aligned in bus width (128bit/16 byte), but we highly recommend to be aligned in 4KB{has_reset=0} |  |  |  |  |  |

| 1.1.1.96.0 |  |  |  |  |  |  |  | CMD_CREATE_INST_ADDR_WORK_BUF |  |  |  |  |  |  | 0x300 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 768 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Work buffer base address{alt_mode=CMD_BS_RD_PTR_signal=3;virtualpairUid=2022-09-0709:27:31T927ba960-7865-42b9-b066-08646b1e13ba;alternate_reg=CMD_BS_RD_PTR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | WORK_BUF_BASE |  |  | rw | ro |  | 0x0 |  |  | Base address of work buffer for each instance This address should be aligned in 4KB boundary.{has_reset=0} |  |  |  |  |  |

| 1.1.1.97.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_SET_PARAM_ENABLE |  |  |  |  |  |  | 0x300 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 768 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Enabling flags for parameter changing{alt_mode=CMD_BS_RD_PTR_signal=4;virtualpairUid=2022-09-0709:27:31T927ba960-7865-42b9-b066-08646b1e13ba;alternate_reg=CMD_BS_RD_PTR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:28 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 27:27 | ENABLE_SET_QUANT_PARAM |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_QUANT_PARAM_0 and CMD_ENC_SEQ_QUANT_PARAM_1.{has_reset=0} |  |  |  |  |  |
| 26:26 | ENABLE_SET_QROUND_OFFSET |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_QROUND_OFFSET.{has_reset=0} |  |  |  |  |  |
| 25:23 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 22:22 | ENABLE_SET_BG_PARAM |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_BG_PARAM.{has_reset=0} |  |  |  |  |  |
| 21:21 | RSVD2 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:20 | ENABLE_SET_VUI_HRD_PARAM |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_NON_VCL_PARAM and CMD_ENC_SEQ_VUI_RBSP_ADDR, CMD_ENC_SEQ_HRD_RBSP_ADDR.{has_reset=0} |  |  |  |  |  |
| 19:15 | RSVD3 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 14:14 | ENABLE_SET_RC_VBV_BUFFER_SIZE |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_RC_VBV_BUFFER_SIZE.{has_reset=0} |  |  |  |  |  |
| 13:13 | ENABLE_SET_RC_MAX_BITRATE |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_RC_MAX_BITRATE.{has_reset=0} |  |  |  |  |  |
| 12:12 | ENABLE_SET_MIN_MAX_QP |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_INTRA_MIN_MAX_QP and CMD_ENC_SEQ_INTER_MIN_MAX_QP.{has_reset=0} |  |  |  |  |  |
| 11:11 | ENABLE_SET_RC_PARAM |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_RC_PARAM.{has_reset=0} |  |  |  |  |  |
| 10:10 | ENABLE_SET_RC_TARGET_RATE |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_RC_TARGET_RATE.{has_reset=0} |  |  |  |  |  |
| 9:7 | RSVD4 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 6:6 | ENABLE_SET_SLICE_PARAM |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_SLICE.{has_reset=0} |  |  |  |  |  |
| 5:5 | RSVD5 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 4:4 | ENABLE_SET_TEMPORAL_QP_PARAM |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_TEMPORAL_QP_PARAM. - CODECAPI_AVEncVideoEncodeQp - CODECAPI_AVEncVideoEncodeFrameTypeQp{has_reset=0} |  |  |  |  |  |
| 3:3 | RSVD6 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 2:2 | ENABLE_SET_INTRA_PARAM |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_INTRA_PARAM/CMD_ENC_SEQ_INTRA_PARAM_AVC.{has_reset=0} |  |  |  |  |  |
| 1:1 | ENABLE_SET_TEMPORAL_COUNT_PARAM |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_TEMPORAL_COUNT_PARAM. - CODECAPI_AVTemporalLayerCount{has_reset=0} |  |  |  |  |  |
| 0:0 | ENABLE_SET_PPS_PARAM |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_PPS_PARAM.{has_reset=0} |  |  |  |  |  |

| 1.1.1.98.0 |  |  |  |  |  |  |  | CMD_SET_FB_COMMON_PIC_INFO |  |  |  |  |  |  | 0x300 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 768 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| DPB information{alt_mode=CMD_BS_RD_PTR_signal=5;virtualpairUid=2022-09-0709:27:31T927ba960-7865-42b9-b066-08646b1e13ba;alternate_reg=CMD_BS_RD_PTR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:31 | PVRIC_ENABLE |  |  | rw | ro |  | 0x0 |  |  | This field enables VPU to generate PVRIC format of decoded frames. 0: BWB format 1: PVRIC format [NOTE] This is valid only if PVRIC hardware is included in the product.{has_reset=0} |  |  |  |  |  |
| 30:30 | AFBC_ENABLE |  |  | rw | ro |  | 0x0 |  |  | This field enables VPU to generate AFBC(Arm Frame Buffer Compression) format of decoded frames. 0: BWB format 1: AFBC format [NOTE] This is valid only if AFBC hardware is included in the product.{has_reset=0} |  |  |  |  |  |
| 29:29 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 28:28 | BWB_ENABLE |  |  | rw | ro |  | 0x0 |  |  | This field indicates whether target frame buffers are used for uncompressed linear frame. 0: Target frame buffers are compressed frames. 1: Target frame buffers are uncompressed linear frames.{has_reset=0} |  |  |  |  |  |
| 27:27 | CSC_ENABLE |  |  | rw | ro |  | 0x0 |  |  | This field indicates whether to enable CSC (color space conversion) or not. 0 : disable CSC. (output: YCbCr) 1 : enable CSC. (output: RGB) This field is valid only if BWB_ENABLE is 1. [NOTE] This is valid only if CSC hardware is included in the product.{has_reset=0} |  |  |  |  |  |
| 26:26 | ONE_PLANE_OUTPUT_ENABLE |  |  | rw | ro |  | 0x0 |  |  | This field indicates enable 1 plane output 0 : disalbe 1-plane output 1 : enable 1-plane output [NOTE] YUV444 support 1 plane output. This field is valid only if BWB_ENABLE is 1.{has_reset=0} |  |  |  |  |  |
| 25:24 | PIXEL_FORMAT |  |  | rw | ro |  | 0x0 |  |  | This filed indicates pixel format of the bitstream 0 : YUV420 1 : YUV422 2 : YUV444 This field is valid only if BWB_ENABLE is 1.{has_reset=0} |  |  |  |  |  |
| 23:23 | PIXEL_ORDER_MODE |  |  | rw | ro |  | 0x0 |  |  | Pixel ordering in a bus word 0: decreasing ordering - pixel position is decreasing as byte address in a bus is increaing. 1: incrsing ordering - pixel position is increasing as byte address in a bus is increaing. [NOTE] Pixel position is always increasing as word address is increasing. This field is valid only if BWB_ENABLE is 1.{has_reset=0} |  |  |  |  |  |
| 22:20 | PIXEL_OUTPUT_MODE |  |  | rw | ro |  | 0x0 |  |  | [22] 0 : MSB justified pixel 1 : LSB justified pixel [21:20] 0 : BWB output mode 0 (10bit/pixel -> 8bit/pixel) 1 : BWB output mode 1 (16bit/pixel) 2 : BWB output mode 2 (32bit/pixel) PIXEL_OUTPUT_MODE cannot be 2 if Chroma Output Format (COMMON_PIC_INFO[18:16]) is set as packed mode (4~7) Valid only if BWB_ENABLE is 1.{has_reset=0} |  |  |  |  |  |
| 19:19 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 18:16 | CHROMA_OUTPUT_FORMAT |  |  | rw | ro |  | 0x0 |  |  | BWB Chroma format 0 : Planar mode - YV12 when YCbCr420, YV16 when YCbCr422 1 : Semi-planar mode, Cb/Cr interleaved - NV12 when YCbCr420, NV16 when YCbCr422 3 : Semi-planar mode, Cb/Cr interleaved - NV21 when YCbCr420, NV61 when YCbCr422 Valid only if BWB_ENABLE is 1.{has_reset=0} |  |  |  |  |  |
| 15:0 | FB_STRIDE |  |  | rw | ro |  | 0x0 |  |  | Stride for the storing alignment{has_reset=0} |  |  |  |  |  |

| 1.1.1.99.0 |  |  |  |  |  |  |  | RET_DEC_ADDR_USERDATA_BASE |  |  |  |  |  |  | 0x300 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 768 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| User Data Buffer Base Address{alt_mode=CMD_BS_RD_PTR_signal=6;virtualpairUid=2022-09-0709:27:31T927ba960-7865-42b9-b066-08646b1e13ba;alternate_reg=CMD_BS_RD_PTR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | USER_DATA_BUF_BASE |  |  | rw | ro |  | 0x0 |  |  | * support standard : H265/H264 Base address of user data buffer User data buffer holds SEI, SPS, and VUI information. This address should be aligned in 4K boundary. VPU reports user data of the decoded frame if user data exists in the decoded frame and the relevant user data report option is enabled. User data buffering/reordering is required to sync user data with display frame.{has_reset=0} |  |  |  |  |  |

| 1.1.1.100.0 |  |  |  |  |  |  |  | RET_ENC_RD_PTR |  |  |  |  |  |  | 0x300 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 768 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| bitstream buffer read pointer{alt_mode=CMD_BS_RD_PTR_signal=7;virtualpairUid=2022-09-0709:27:31T927ba960-7865-42b9-b066-08646b1e13ba;alternate_reg=CMD_BS_RD_PTR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | RD_PTR |  |  | rw | ro |  | 0x0 |  |  | The start address of CPB where bitstream reads{has_reset=0} |  |  |  |  |  |

| 1.1.1.101.0 |  |  |  |  |  |  |  | RET_ENC_RD_PTR_QUERY_ENC_FW_STATUS |  |  |  |  |  |  | 0x300 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 768 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| bitstream buffer read pointer{alt_mode=CMD_BS_RD_PTR_signal=8;virtualpairUid=2022-09-0709:27:31T927ba960-7865-42b9-b066-08646b1e13ba;alternate_reg=CMD_BS_RD_PTR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | RD_PTR |  |  | rw | ro |  | 0x0 |  |  | The start address of CPB where bitstream reads{has_reset=0} |  |  |  |  |  |

| 1.1.1.103.0 |  |  |  |  |  |  |  | RET_QUERY_ENC_BS_RD_PTR |  |  |  |  |  |  | 0x300 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 768 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| {alt_mode=CMD_BS_RD_PTR_signal=9;virtualpairUid=2022-09-0709:27:31T927ba960-7865-42b9-b066-08646b1e13ba;alternate_reg=CMD_BS_RD_PTR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | QUERY_ENC_BS_RD_PTR |  |  | ro | ro |  | 0x0 |  |  | The start position of bistream buffer for the currnetly encoded picture.{has_reset=0} |  |  |  |  |  |

| 1.1.1.104.0 |  |  |  |  |  |  |  | CMD_BS_WR_PTR |  |  |  |  |  |  | 0x304 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 772 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Bitstream buffer write pointer{alt_mode=CMD_BS_SIZE_signal=1;virtualpairUid=2022-09-0709:27:31T93109875-62e0-4860-bc83-f143828565a6;alternate_reg=CMD_BS_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | WR_PTR |  |  | rw | ro |  | 0x0 |  |  | End address of bitstream for handling current command Host can update this register anytime. If a bitstream_empty interrupt is asserted, host processor should do either feed more bitstream and update WR_PTR or set EXPLICIT_END to complete decoding anyhow.{has_reset=0} |  |  |  |  |  |

| 1.1.1.105.0 |  |  |  |  |  |  |  | CMD_BS_WR_PTR_DEC_PIC |  |  |  |  |  |  | 0x304 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 772 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| write pointer on Bitstream buffer{alt_mode=CMD_BS_SIZE_signal=2;virtualpairUid=2022-09-0709:27:31T93109875-62e0-4860-bc83-f143828565a6;alternate_reg=CMD_BS_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | WR_PTR |  |  | rw | ro |  | 0x0 |  |  | WR_PTR{has_reset=0} |  |  |  |  |  |

| 1.1.1.106.0 |  |  |  |  |  |  |  | CMD_CREATE_INST_WORK_BUF_SIZE |  |  |  |  |  |  | 0x304 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 772 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Work buffer size{alt_mode=CMD_BS_SIZE_signal=3;virtualpairUid=2022-09-0709:27:31T93109875-62e0-4860-bc83-f143828565a6;alternate_reg=CMD_BS_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | WORK_BUF_SIZE |  |  | rw | ro |  | 0x0 |  |  | Size of work buffer (4KB boundary) The size of work buffer should be larger than required minimum work buffer size. This address should be aligned in 4KB boundary.{has_reset=0} |  |  |  |  |  |

| 1.1.1.107.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_SRC_SIZE |  |  |  |  |  |  | 0x304 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 772 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| A size of source picture (MANDATORY){alt_mode=CMD_BS_SIZE_signal=4;virtualpairUid=2022-09-0709:27:31T93109875-62e0-4860-bc83-f143828565a6;alternate_reg=CMD_BS_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | SRC_HEIGHT |  |  | rw | ro |  | 0x0 |  |  | A height of source picture in pixel (128 ~ 8192){has_reset=0} |  |  |  |  |  |
| 15:0 | SRC_WIDTH |  |  | rw | ro |  | 0x0 |  |  | A width of source picture in pixel (256 ~ 8192){has_reset=0} |  |  |  |  |  |

| 1.1.1.108.0 |  |  |  |  |  |  |  | CMD_SET_FB_PIC_SIZE |  |  |  |  |  |  | 0x304 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 772 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Decoded picture size{alt_mode=CMD_BS_SIZE_signal=5;virtualpairUid=2022-09-0709:27:31T93109875-62e0-4860-bc83-f143828565a6;alternate_reg=CMD_BS_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | PIC_WIDTH |  |  | rw | ro |  | 0x0 |  |  | Picture width of DPB When SCL_EN is 1 and BWB_ENABLE is 1, following restrictions are applied - output image width (support down scaler only 1- 1/8) - size range :32 ~ 8192 - size must be 4 align{has_reset=0} |  |  |  |  |  |
| 15:0 | PIC_HEIGHT |  |  | rw | ro |  | 0x0 |  |  | Picture height of DPB When SCL_EN is 1 and BWB_ENABLE is 1, following restrictions are applied - output image height (support down scaler only 1- 1/8) - size range :32 ~ 8192 - size must be 4 align{has_reset=0} |  |  |  |  |  |

| 1.1.1.109.0 |  |  |  |  |  |  |  | RET_DEC_USERDATA_SIZE |  |  |  |  |  |  | 0x304 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 772 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| User Data Buffer Size{alt_mode=CMD_BS_SIZE_signal=6;virtualpairUid=2022-09-0709:27:31T93109875-62e0-4860-bc83-f143828565a6;alternate_reg=CMD_BS_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | USER_DATA_BUF_SIZE |  |  | rw | ro |  | 0x0 |  |  | * support standard : H265/H264 Size of user data buffer{has_reset=0} |  |  |  |  |  |

| 1.1.1.110.0 |  |  |  |  |  |  |  | RET_ENC_WR_PTR |  |  |  |  |  |  | 0x304 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 772 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| bitstream buffer write pointer{alt_mode=CMD_BS_SIZE_signal=7;virtualpairUid=2022-09-0709:27:31T93109875-62e0-4860-bc83-f143828565a6;alternate_reg=CMD_BS_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | WR_PTR |  |  | rw | ro |  | 0x0 |  |  | The end address of CPB where bitstream writes{has_reset=0} |  |  |  |  |  |

| 1.1.1.111.0 |  |  |  |  |  |  |  | RET_ENC_WR_PTR_QUERY_ENC_FW_STATUS |  |  |  |  |  |  | 0x304 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 772 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| bitstream buffer write pointer{alt_mode=CMD_BS_SIZE_signal=8;virtualpairUid=2022-09-0709:27:31T93109875-62e0-4860-bc83-f143828565a6;alternate_reg=CMD_BS_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | WR_PTR |  |  | rw | ro |  | 0x0 |  |  | The end address of CPB where bitstream writes{has_reset=0} |  |  |  |  |  |

| 1.1.1.113.0 |  |  |  |  |  |  |  | RET_QUERY_ENC_BS_WR_PTR |  |  |  |  |  |  | 0x304 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 772 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| {alt_mode=CMD_BS_SIZE_signal=9;virtualpairUid=2022-09-0709:27:31T93109875-62e0-4860-bc83-f143828565a6;alternate_reg=CMD_BS_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | QUERY_ENC_BS_WR_PTR |  |  | ro | ro |  | 0x0 |  |  | The end position of bistream buffer for the currnetly encoded picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.114.0 |  |  |  |  |  |  |  | CMD_BS_OPTIONS |  |  |  |  |  |  | 0x308 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 776 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Bitstream buffer option{alt_mode=CMD_BS_OPTION_signal=1;virtualpairUid=2022-09-0709:27:31Tf65c537b-3e95-47ab-8050-d9673202e17e;alternate_reg=CMD_BS_OPTION} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:4 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 3:2 | STREAM_FORMAT |  |  | rw | ro |  | 0x0 |  |  | Bitstream format (AV1) 0 : IVF 1 : OBU 2 : ANNEXB{has_reset=0} |  |  |  |  |  |
| 1:1 | STREAM_END |  |  | rw | ro |  | 0x0 |  |  | This field makes VPU assume Stream End when there is no stream to feed in the buffer.{has_reset=0} |  |  |  |  |  |
| 0:0 | EXPLICIT_END |  |  | rw | ro |  | 0x0 |  |  | Explicit End When this field is set to 1, VPU assumes that bitstream buffer has at least one single frame and follows the tasks below. # VPU decodes until the end of bitstream buffer (WR_PTR) even if the last 3 bytes are all 0. # VPU returns success if it has decoded a frame successfully. If bitstream is insufficient to complete decoding a frame, VPU performs what it is supposed to do with the specified task in BS_SHORTAGE_OPTION of BS_PARAM. If this flag is 0, # VPU decodes to the almost end of bitstream buffer (WR_PTR), but not to some bytes (less than 3). It intentionally does not consume some a few bytes, because VPU is not sure if the last bytes are the start code of next NAL or they might not fill the offset in the CABAC. # VPU returns success if it has decoded a frame successfully. If bitstream is insufficient to complete decoding a frame, VPU stops decoding and waits for more bitstream to be filled. Then host processor can do either feed bitstream more or set EXPLICIT_END to complete decoding anyhow. BITSTREAM_EMPTY interrupt is asserted when EXPLITCT_END = 0 or when bitstream buffer is near empty (not real empty) for seamless decoding. CAUTION: Host processor can set this register any time, but cannot clear during command processing.{has_reset=0} |  |  |  |  |  |

| 1.1.1.115.0 |  |  |  |  |  |  |  | CMD_BS_OPTIONS_ENC_PIC |  |  |  |  |  |  | 0x308 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 776 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Bitstream buffer option{alt_mode=CMD_BS_OPTION_signal=2;virtualpairUid=2022-09-0709:27:31Tf65c537b-3e95-47ab-8050-d9673202e17e;alternate_reg=CMD_BS_OPTION} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 5:4 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 3:0 | BS_ENDIAN |  |  | rw | ro |  | 0x0 |  |  | Endianness of bitstream buffer{has_reset=0} |  |  |  |  |  |

| 1.1.1.116.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_CUSTOM_MAP_ENDIAN |  |  |  |  |  |  | 0x308 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 776 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Custom map endian (MANDATORY){alt_mode=CMD_BS_OPTION_signal=3;virtualpairUid=2022-09-0709:27:31Tf65c537b-3e95-47ab-8050-d9673202e17e;alternate_reg=CMD_BS_OPTION} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CUSTOM_MAP_ENDIAN |  |  | rw | ro |  | 0x0 |  |  | Endianness{has_reset=0} |  |  |  |  |  |

| 1.1.1.117.0 |  |  |  |  |  |  |  | CMD_SET_FB_NUM_FB |  |  |  |  |  |  | 0x308 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 776 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| The number of start/end frame buffer to set (max. 8 frame buffers can be set for one SET_FB command{alt_mode=CMD_BS_OPTION_signal=4;virtualpairUid=2022-09-0709:27:31Tf65c537b-3e95-47ab-8050-d9673202e17e;alternate_reg=CMD_BS_OPTION} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:29 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 28:24 | BWB_FB_START_IDX |  |  | rw | ro |  | 0x0 |  |  | The start frame buffer index to set{has_reset=0} |  |  |  |  |  |
| 20:16 | BWB_FB_END_IDX |  |  | rw | ro |  | 0x0 |  |  | The end frame buffer index to set{has_reset=0} |  |  |  |  |  |
| 15:10 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 9:5 | COL_FB_START_IDX |  |  | rw | ro |  | 0x0 |  |  | The start frame buffer index to set for Codec dedicated information buffer{has_reset=0} |  |  |  |  |  |
| 4:0 | COL_FB_END_IDX |  |  | rw | ro |  | 0x0 |  |  | The end frame buffer index to set for Codec dedicated information buffer{has_reset=0} |  |  |  |  |  |

| 1.1.1.119.0 |  |  |  |  |  |  |  | RET_ENC_NUM_REQUIRED_FB |  |  |  |  |  |  | 0x308 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 776 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Minimum number of reference frame buffer required for encoding{alt_mode=CMD_BS_OPTION_signal=5;virtualpairUid=2022-09-0709:27:31Tf65c537b-3e95-47ab-8050-d9673202e17e;alternate_reg=CMD_BS_OPTION} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | RET_MIN_FB_BUF_NUM |  |  | rw | ro |  | 0x0 |  |  | The minimum number of reference frame buffer for encoding (1 ~ 16){has_reset=0} |  |  |  |  |  |

| 1.1.1.120.0 |  |  |  |  |  |  |  | CMD_SET_FB_FBC_STRIDE |  |  |  |  |  |  | 0x30C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 780 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Frame buffer setting for compressed frame{alt_mode=CMD_ENC_SEQ_PARAM_signal=1;virtualpairUid=2022-09-0709:27:31Ta8acfb72-9691-4023-9e43-d4626272c979;alternate_reg=CMD_ENC_SEQ_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | FBC_LUMA_STRIDE |  |  | rw | ro |  | 0x0 |  |  | A stride of FBC for luma component{has_reset=0} |  |  |  |  |  |
| 15:0 | FBC_CHROMA_STRIDE |  |  | rw | ro |  | 0x0 |  |  | A stride of FBC for chroma component{has_reset=0} |  |  |  |  |  |

| 1.1.1.121.0 |  |  |  |  |  |  |  | CMD_USE_SEC_AXI |  |  |  |  |  |  | 0x30C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 780 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Secondary AXI usage option{alt_mode=CMD_ENC_SEQ_PARAM_signal=2;virtualpairUid=2022-09-0709:27:31Ta8acfb72-9691-4023-9e43-d4626272c979;alternate_reg=CMD_ENC_SEQ_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 1:1 | SEC_AXI_LF_ROW |  |  | rw | ro |  | 0x0 |  |  | Use 2nd AXI temp buffer for Read channel of RDO row buffer.{has_reset=0} |  |  |  |  |  |
| 0:0 | SEC_AXI_RDO_ENABLE |  |  | rw | ro |  | 0x0 |  |  | Use 2nd AXI temp buffer for Read channel of LF row buffer.{has_reset=0} |  |  |  |  |  |

| 1.1.1.122.0 |  |  |  |  |  |  |  | CMD_USE_SEC_AXI_ENC_PIC |  |  |  |  |  |  | 0x30C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 780 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Secondary AXI usage option{alt_mode=CMD_ENC_SEQ_PARAM_signal=3;virtualpairUid=2022-09-0709:27:31Ta8acfb72-9691-4023-9e43-d4626272c979;alternate_reg=CMD_ENC_SEQ_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 1:1 | SEC_AXI_RDO_ENABLE |  |  | rw | ro |  | 0x0 |  |  | Use 2nd AXI temp buffer for Read channel of RDO row buffer.{has_reset=0} |  |  |  |  |  |
| 0:0 | SEC_AXI_LF_ROW |  |  | rw | ro |  | 0x0 |  |  | Use 2nd AXI temp buffer for Read channel of LF row buffer.{has_reset=0} |  |  |  |  |  |

| 1.1.1.123.0 |  |  |  |  |  |  |  | RET_DEC_BS_RD_PTR |  |  |  |  |  |  | 0x30C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 780 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Bitstream buffer read pointer{alt_mode=CMD_ENC_SEQ_PARAM_signal=4;virtualpairUid=2022-09-0709:27:31Ta8acfb72-9691-4023-9e43-d4626272c979;alternate_reg=CMD_ENC_SEQ_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | BS_RD_PTR |  |  | rw | ro |  | 0x0 |  |  | VPU read address in the CPB. Host can feed bitstream data up to this address.{has_reset=0} |  |  |  |  |  |

| 1.1.1.125.0 |  |  |  |  |  |  |  | RET_MIN_SRC_BUF_NUM |  |  |  |  |  |  | 0x30C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 780 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Minimum number of source frame buffer required for encoding{alt_mode=CMD_ENC_SEQ_PARAM_signal=5;virtualpairUid=2022-09-0709:27:31Ta8acfb72-9691-4023-9e43-d4626272c979;alternate_reg=CMD_ENC_SEQ_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | RET_MIN_SRC_BUF_NUM |  |  | rw | ro |  | 0x0 |  |  | The minimum number of source frame buffer for encoding (1 ~ 16){has_reset=0} |  |  |  |  |  |

| 1.1.1.126.0 |  |  |  |  |  |  |  | CMD_ENC_REPORT_PARAM |  |  |  |  |  |  | 0x310 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 784 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Report parameters{alt_mode=CMD_CREATE_INST_BS_PARAM_signal=1;virtualpairUid=2022-09-0709:27:32Te06ff2d0-7434-4684-be12-3c276b70f371;alternate_reg=CMD_CREATE_INST_BS_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:2 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 1:1 | ENABLE_HISTO_EN |  |  | rw | ro |  | 0x0 |  |  | It eanbles to report MV histogram count.{has_reset=0} |  |  |  |  |  |
| 0:0 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |

| 1.1.1.127.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_PPS_PARAM |  |  |  |  |  |  | 0x310 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 784 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Encoder picture parameters (MANDATORY){alt_mode=CMD_CREATE_INST_BS_PARAM_signal=2;virtualpairUid=2022-09-0709:27:32Te06ff2d0-7434-4684-be12-3c276b70f371;alternate_reg=CMD_CREATE_INST_BS_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:31 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 30:30 | USE_ENTROPY_CODING_MODE |  |  | rw | ro |  | 0x0 |  |  | It enables CABAC. (AVC only) 0 : CAVLC is used for entropy coding 1 : CABAC is used for entropy coding{has_reset=0} |  |  |  |  |  |
| 29:29 | USE_TRANSFORM_8x8 |  |  | rw | ro |  | 0x0 |  |  | It enables transform 8x8.{has_reset=0} |  |  |  |  |  |
| 28:24 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved It should be 0.{has_reset=0} |  |  |  |  |  |
| 23:19 | CR_QP_OFFSET |  |  | rw | ro |  | 0x0 |  |  | HEVC/AVC: QP offset for Cr color component{has_reset=0} |  |  |  |  |  |
| 18:14 | CB_QP_OFFSET |  |  | rw | ro |  | 0x0 |  |  | HEVC/AVC: QP offset for Cb color component{has_reset=0} |  |  |  |  |  |
| 13:10 | TC_OFFSET_DIV2 |  |  | rw | ro |  | 0x0 |  |  | HEVC/AVC: TcOffsetDiv2 for deblocking filter{has_reset=0} |  |  |  |  |  |
| 9:6 | BETA_OFFSET_DIV2 |  |  | rw | ro |  | 0x0 |  |  | HEVC/AVC: BetaOffsetDiv2 for deblocking filter AV1: LF sharpness{has_reset=0} |  |  |  |  |  |
| 5:5 | DISABLE_DBK |  |  | rw | ro |  | 0x0 |  |  | It disables the in-loop deblocking filter.{has_reset=0} |  |  |  |  |  |
| 4:3 | RSVD2 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 2:2 | USE_LF_CROSS_SLICE_BOUNDARY |  |  | rw | ro |  | 0x0 |  |  | It enables the use of in-loop filtering across slice boundaries. (HEVC/AVC){has_reset=0} |  |  |  |  |  |
| 1:1 | CONSTRAINED_INTRA_PRED |  |  | rw | ro |  | 0x0 |  |  | It enables constrained intra prediction. (HEVC/AVC){has_reset=0} |  |  |  |  |  |
| 0:0 | RSVD3 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |

| 1.1.1.128.0 |  |  |  |  |  |  |  | CMD_SEQ_CHANGE_ENABLE_FLAG |  |  |  |  |  |  | 0x310 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 784 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Sequence change enable flag If any bit flag of SEQ_CHANGE_ENABLE_FLAG is 1 and the syntax value corresponding the bit flag is changed with a newly activated SPS, VPU regards it as sequence change. VPU reports sequence change to RET_QUERY_DEC_NOTIFICATION register. Then VPU bumps out all of the pictures that remain in DPB and clears the reference flag. However, there is an exception. If RET_QUERY_DEC_NOTIFICATION[17] is 1, VPU only reports the DPB index to be replaced wihtin the same sequence.{alt_mode=CMD_CREATE_INST_BS_PARAM_signal=3;virtualpairUid=2022-09-0709:27:32Te06ff2d0-7434-4684-be12-3c276b70f371;alternate_reg=CMD_CREATE_INST_BS_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:20 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 19:19 | DEC_TEMP_ID_MODE |  |  | rw | ro |  | 0x0 |  |  | A DPB count change enable flag{has_reset=0} |  |  |  |  |  |
| 18:18 | SEQ_CHANGE_ENABLE_FLAG_BITDEPTH |  |  | rw | ro |  | 0x0 |  |  | A bit-depth change enable flag{has_reset=0} |  |  |  |  |  |
| 17:17 | SEQ_CHANGE_ENABLE_FLAG_INTER_RES_CHANGE |  |  | rw | ro |  | 0x0 |  |  | An inter resolution change enable flag for VP9/AV1{has_reset=0} |  |  |  |  |  |
| 16:16 | SEQ_CHANGE_ENABLE_FLAG_SIZE |  |  | rw | ro |  | 0x0 |  |  | A picture size change enable flag{has_reset=0} |  |  |  |  |  |
| 15:15 | SEQ_CHANGE_ENABLE_FLAG_CHROMA_FORMAT |  |  | rw | ro |  | 0x0 |  |  | A chroma format change enable flag{has_reset=0} |  |  |  |  |  |
| 14:6 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 5:5 | SEQ_CHANGE_ENABLE_FLAG_PROFILE |  |  | rw | ro |  | 0x0 |  |  | A profile change enable flag{has_reset=0} |  |  |  |  |  |
| 4:0 | RSVD2 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |

| 1.1.1.129.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE0 |  |  |  |  |  |  | 0x310 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 784 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 0{alt_mode=CMD_CREATE_INST_BS_PARAM_signal=4;virtualpairUid=2022-09-0709:27:32Te06ff2d0-7434-4684-be12-3c276b70f371;alternate_reg=CMD_CREATE_INST_BS_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE0 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 0 Compressed Luma base address of DPB index 0{has_reset=0} |  |  |  |  |  |

| 1.1.1.130.0 |  |  |  |  |  |  |  | RET_DEC_SEQ_PARAM |  |  |  |  |  |  | 0x310 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 784 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Bitstream sequence parameter information{alt_mode=CMD_CREATE_INST_BS_PARAM_signal=5;virtualpairUid=2022-09-0709:27:32Te06ff2d0-7434-4684-be12-3c276b70f371;alternate_reg=CMD_CREATE_INST_BS_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | PROFILE_SPACE |  |  | rw | ro |  | 0x0 |  |  | H.265 : the value of general_profile_space of active SPS. See H.265 annex A for detail information. AVS2 : the value of sample_precision 8{has_reset=0} |  |  |  |  |  |
| 29:29 | PROFILE_TIER_FLAG |  |  | rw | ro |  | 0x0 |  |  | H.265 : the value of general_tire_flag of active SPS{has_reset=0} |  |  |  |  |  |
| 28:24 | PROFILE_IDC |  |  | rw | ro |  | 0x0 |  |  | H.265/H.264 : the value of general_profile_idc of active SPS VP9 : the value of a profile of VP9 AVS2: the value of profile_id of active SPS AV1 : the value of profile_ of sequence header{has_reset=0} |  |  |  |  |  |
| 23:21 | SPS_MAX_SUB_LAYER |  |  | rw | ro |  | 0x0 |  |  | H.265/H.264 : the value of maximum number of temporal sub-layers, sps_max_sub_layer_minus1+1, of active SPS AVS2 : the value of maximum number of temporal sub-layers if temporal_id_enable is 1. Then it returns 7. Otherwise, it is 0.{has_reset=0} |  |  |  |  |  |
| 20:20 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 19:12 | PROFILE_COMPATIBILITY_FLAG |  |  | rw | ro |  | 0x0 |  |  | H.265 : the value of first 8 bits out of general_profile_compatiblity_flag[32] of active SPS. PROFILE_COMPATIBILITY_FLAG[i] is equal to general_profile_compatiblity_flag[i] with i = 0 to 7.{has_reset=0} |  |  |  |  |  |
| 11:11 | PROGRESS_SOURCE_FLAG |  |  | rw | ro |  | 0x0 |  |  | H.265 : the value of general_progressive_source_flag of active SPS AVS2 : the value of progressive_sequence of active SPS{has_reset=0} |  |  |  |  |  |
| 10:10 | INTERLACE_SOURCE_FLAG |  |  | rw | ro |  | 0x0 |  |  | H.265 : the value of general_interlaced_source_flag of active SPS AVS2 : the value of field_coded_sequence of active SPS{has_reset=0} |  |  |  |  |  |
| 9:9 | NON_PACKED_CONSTRAINT_FLAG |  |  | rw | ro |  | 0x0 |  |  | H.256 : the value of general_non_packed_constraint_flag of active SPS{has_reset=0} |  |  |  |  |  |
| 8:8 | FRAME_ONLY_CONSTRAINT_FLAG |  |  | rw | ro |  | 0x0 |  |  | H.265 : the value of general_frame_only_constraint_flag of active SPS AVS2 : the value of progressive_sequence of active SPS{has_reset=0} |  |  |  |  |  |
| 7:0 | LEVEL_IDC |  |  | rw | ro |  | 0x0 |  |  | H.265/H.264 : the value of general_level_idc of active SPS AVS2 : the value of level_id of active SPS AV1 : the value of level_id of sequence header{has_reset=0} |  |  |  |  |  |

| 1.1.1.132.0 |  |  |  |  |  |  |  | RET_ENC_PIC_TYPE |  |  |  |  |  |  | 0x310 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 784 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Encoded picture type{alt_mode=CMD_CREATE_INST_BS_PARAM_signal=6;virtualpairUid=2022-09-0709:27:32Te06ff2d0-7434-4684-be12-3c276b70f371;alternate_reg=CMD_CREATE_INST_BS_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 15:0 | PIC_TYPE |  |  | rw | ro |  | 0x0 |  |  | The encoded picture type 0 : I slice 1 : P slice 2 : B slice{has_reset=0} |  |  |  |  |  |

| 1.1.1.133.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_GOP_PARAM |  |  |  |  |  |  | 0x314 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 788 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| GOP structure designation (MANDATORY){alt_mode=CMD_DEC_SEI_MASK_signal=1;virtualpairUid=2022-09-0709:27:32Tcbc23275-2bfa-4d9e-b933-c6d6fb79bb1e;alternate_reg=CMD_DEC_SEI_MASK} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:20 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 19:16 | TEMPORAL_LAYER_COUNT |  |  | rw | ro |  | 0x0 |  |  | Temporal layer count ??number of temporal layer{has_reset=0} |  |  |  |  |  |
| 15:12 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 11:11 | EN_TEMPORAL_LAYER_3_QP |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_TEMPORAL_LAYER_3_QP.{has_reset=0} |  |  |  |  |  |
| 10:10 | EN_TEMPORAL_LAYER_2_QP |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_TEMPORAL_LAYER_2_QP.{has_reset=0} |  |  |  |  |  |
| 9:9 | EN_TEMPORAL_LAYER_1_QP |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_TEMPORAL_LAYER_1_QP.{has_reset=0} |  |  |  |  |  |
| 8:8 | EN_TEMPORAL_LAYER_0_QP |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_SEQ_TEMPORAL_LAYER_0_QP.{has_reset=0} |  |  |  |  |  |
| 7:0 | GOP_PRESET_IDX |  |  | rw | ro |  | 0x0 |  |  | A GOP structure option 0: Custom GOP 1 : I-I-I-I,..I (all intra, gop_size=1) 2 : I-P-P-P,... P (consecutive P, gop_size=1) 3 : I-B-B-B,...B (consecutive B, gop_size=1) 4 : I-B-P-B-P,... (gop_size=2) 5 : I-B-B-B-P,... (gop_size=4) 6 : I-P-P-P-P,... (consecutive P, gop_size=4) 7 : I-B-B-B-B,... (consecutive B, gop_size=4) 8 : I-B-B-B-B-B-B-B-B,... (random access, gop_size=8){has_reset=0} |  |  |  |  |  |

| 1.1.1.134.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE0 |  |  |  |  |  |  | 0x314 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 788 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index 0{alt_mode=CMD_DEC_SEI_MASK_signal=2;virtualpairUid=2022-09-0709:27:32Tcbc23275-2bfa-4d9e-b933-c6d6fb79bb1e;alternate_reg=CMD_DEC_SEI_MASK} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE0 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 0 Compressed Cb and Cr base address of DPB index 0{has_reset=0} |  |  |  |  |  |

| 1.1.1.135.0 |  |  |  |  |  |  |  | RET_DEC_COLOR_SAMPLE_INFO |  |  |  |  |  |  | 0x314 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 788 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Color Sample Information{alt_mode=CMD_DEC_SEI_MASK_signal=3;virtualpairUid=2022-09-0709:27:32Tcbc23275-2bfa-4d9e-b933-c6d6fb79bb1e;alternate_reg=CMD_DEC_SEI_MASK} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 23:16 | ASPECT_RATIO_IDC |  |  | rw | ro |  | 0x0 |  |  | H.265 : The value of aspect_ratio_idc of H.265. See Table E-1 of Annex E.2 of VUI of H.265 specification. If not decoded in the bitstream, the value of ASPECT_RATIO_IDC is equal to 0. AVS2 : The value of aspect_ratio of AVS2. See Table-39 of AVS2 specification.{has_reset=0} |  |  |  |  |  |
| 12:12 | LF_PIC_DBK_DISABLE |  |  | rw | ro |  | 0x0 |  |  | H.265 : 1 when both sao and loop filter are off. 0 otherwise. VP9 : 1 when loop filter is off. 0 otherwise.{has_reset=0} |  |  |  |  |  |
| 11:8 | COLOR_FORMAT_IDC |  |  | rw | ro |  | 0x0 |  |  | H.265/H.264 : Chroma sampling, chroma_format_idc. See H.265 spec clause 6.2 for more information AVS2 : The chroma sampling chroma_format_idc. See Table-36 of AVS2 specification.{has_reset=0} |  |  |  |  |  |
| 7:4 | BIT_DEPTH_CHROMA |  |  | rw | ro |  | 0x0 |  |  | Bit depth of chroma sample{has_reset=0} |  |  |  |  |  |
| 3:0 | BIT_DEPTH_LUMA |  |  | rw | ro |  | 0x0 |  |  | Bit depth of luma sample{has_reset=0} |  |  |  |  |  |

| 1.1.1.137.0 |  |  |  |  |  |  |  | RET_ENC_PIC_POC |  |  |  |  |  |  | 0x314 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 788 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| A POC value of encoded picture{alt_mode=CMD_DEC_SEI_MASK_signal=4;virtualpairUid=2022-09-0709:27:32Tcbc23275-2bfa-4d9e-b933-c6d6fb79bb1e;alternate_reg=CMD_DEC_SEI_MASK} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | PIC_POC |  |  | rw | ro |  | 0x0 |  |  | A POC value of the currently encoded picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.138.0 |  |  |  |  |  |  |  | CMD_DEC_TEMPORAL_ID_PLUS1 |  |  |  |  |  |  | 0x318 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 792 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Max Decode Temporal ID{alt_mode=CMD_CREATE_ADDR_EXT_signal=1;virtualpairUid=2022-09-0709:27:32T38f35ffa-5d2c-443b-a1a3-7b811f6c6cdc;alternate_reg=CMD_CREATE_ADDR_EXT} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 11:9 | TARGET_DEC_SPATIAL_ID_PLUS1 |  |  | rw | ro |  | 0x0 |  |  | Set an absolute target spatial ID (TARGET_DEC_SPATIAL_ID_PLUS1). (for AV1){has_reset=0} |  |  |  |  |  |
| 8:8 | DEC_TEMP_ID_MODE |  |  | rw | ro |  | 0x0 |  |  | Set the mode of temporal ID selection. 0: use the target temporal_id as absolute value. @* TARGET_DEC_TEMP_ID = TARGET_DEC_TEMP_ID_PLUS1 -1 @* When use of absolute value for temporal target, decoder can keep the decoding layer ID. If the SPS_MAX_SUB_LAYER is changed in the bitstream, the temporal skip ratio can be changed. 1: use the target temporal _id as relative value. @* TARGET_DEC_TEMP_ID = SPS_MAX_SUB_LAYER - REL_TARGET_TEMP_ID @* SPS_MAX_SUB_LAYER is signalled from bitstream. @* When use of relatvie value, decoder can keep the skip(discard) ratio regardless the change of SPS_MAX_SUB_LAYER in the bitstream{has_reset=0} |  |  |  |  |  |
| 7:0 | TARGET_DEC_TEMP_ID_PLUS1 |  |  | rw | ro |  | 0x0 |  |  | If DEC_TEMP_ID_MODE is 0, this field is used as an absolute target temporal ID (TARGET_DEC_TEMP_ID_PLUS1). @* TARGET_DEC_TEMP_ID = TARGET_DEC_TEMP_ID_PLUS1 -1. @* Based on TARGET_DEC_TEMP_ID, @** 0x0 : it decodes a picture of all ranges of temporal ID, which means temporal ID decoding constraint off. @** 0x1 ~ 0x6 : it decodes a picture if the temporal ID is less than or equal to TARGET_DEC_TEMP_ID. It discards a picture when its temporal ID is greater than TARGET_DEC_TEMP_ID. If DEC_TEMP_ID_MODE is 1, this field is used as a relative target temporal ID (REL_TARGET_TEMP_ID). @* TARGET_DEC_TEMP_ID = SPS_MAX_SUB_LAYER - REL_TARGET_TEMP_ID @* It discards a picture when its temporal ID is greater than TARGET_DEC_TEMP_ID.{has_reset=0} |  |  |  |  |  |

| 1.1.1.139.0 |  |  |  |  |  |  |  | CMD_ENC_MV_HISTO_CLASS0 |  |  |  |  |  |  | 0x318 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 792 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_MV_HISTO_CLASS0{alt_mode=CMD_CREATE_ADDR_EXT_signal=2;virtualpairUid=2022-09-0709:27:32T38f35ffa-5d2c-443b-a1a3-7b811f6c6cdc;alternate_reg=CMD_CREATE_ADDR_EXT} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | MV_CLASS_THRESHOLD_0 |  |  | rw | ro |  | 0x0 |  |  | MV threshold value for MV histogram class 0 (MV precison = 1/4-pel){has_reset=0} |  |  |  |  |  |
| 15:0 | MV_CLASS_THRESHOLD_1 |  |  | rw | ro |  | 0x0 |  |  | MV threshold value for MV histogram class 1 (MV precison = 1/4-pel){has_reset=0} |  |  |  |  |  |

| 1.1.1.140.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_INTRA_PARAM |  |  |  |  |  |  | 0x318 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 792 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Intra picture coding parameters for HEVC, AV1 (MANDATORY) {alt_mode=CMD_CREATE_ADDR_EXT_signal=3;virtualpairUid=2022-09-0709:27:32T38f35ffa-5d2c-443b-a1a3-7b811f6c6cdc;alternate_reg=CMD_CREATE_ADDR_EXT} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | INTRA_PERIOD |  |  | rw | ro |  | 0x0 |  |  | A period of intra picture (0 ~ 1024) 0 - implies an infinite period{has_reset=0} |  |  |  |  |  |
| 15:11 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 10:9 | EN_FORCED_IDR_HEADER |  |  | rw | ro |  | 0x0 |  |  | 0 : ForcedIDRHeader off 1 : Add Header at every IDR frame 2 : Add Header at every I frame{has_reset=0} |  |  |  |  |  |
| 8:3 | INTRA_QP |  |  | rw | ro |  | 0x0 |  |  | A quantization parameter of intra picture (0 ~ 51){has_reset=0} |  |  |  |  |  |
| 2:0 | DECODING_REFRESH_TYPE |  |  | rw | ro |  | 0x0 |  |  | A decoding refresh type of intra picture 0 : Non-IRAP 1 : CRA 2 : IDR [NOTE] This parameter cannot be changed within the same sequence.�{has_reset=0} |  |  |  |  |  |

| 1.1.1.141.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_INTRA_PARAM_AVC |  |  |  |  |  |  | 0x318 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 792 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Intra picture coding parameters for AVC (MANDATORY){alt_mode=CMD_CREATE_ADDR_EXT_signal=4;virtualpairUid=2022-09-0709:27:32T38f35ffa-5d2c-443b-a1a3-7b811f6c6cdc;alternate_reg=CMD_CREATE_ADDR_EXT} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:28 | EN_FORCED_IDR_HEADER |  |  | rw | ro |  | 0x0 |  |  | 0: No forced header(SPS/PPS) 1: Forced header before IDR frame 2: Forced header before key frame{has_reset=0} |  |  |  |  |  |
| 27:17 | IDR_PERIOD |  |  | rw | ro |  | 0x0 |  |  | A period of IDR picture (0 ~ 2047) 0 - implies an infinite period [NOTE] This parameter cannot be changed within the same sequence.{has_reset=0} |  |  |  |  |  |
| 16:6 | INTRA_PERIOD |  |  | rw | ro |  | 0x0 |  |  | A period of intra picture (0 ~ 2047) 0 - implies an infinite period [NOTE] This parameter cannot be changed within the same sequence.{has_reset=0} |  |  |  |  |  |
| 5:0 | INTRA_QP |  |  | rw | ro |  | 0x0 |  |  | A quantization parameter of intra picture (0 ~ 51) {has_reset=0} |  |  |  |  |  |

| 1.1.1.142.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE0 |  |  |  |  |  |  | 0x318 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 792 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 0{alt_mode=CMD_CREATE_ADDR_EXT_signal=5;virtualpairUid=2022-09-0709:27:32T38f35ffa-5d2c-443b-a1a3-7b811f6c6cdc;alternate_reg=CMD_CREATE_ADDR_EXT} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE0 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 0{has_reset=0} |  |  |  |  |  |

| 1.1.1.143.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET0 |  |  |  |  |  |  | 0x318 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 792 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 0{alt_mode=CMD_CREATE_ADDR_EXT_signal=6;virtualpairUid=2022-09-0709:27:32T38f35ffa-5d2c-443b-a1a3-7b811f6c6cdc;alternate_reg=CMD_CREATE_ADDR_EXT} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE0 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB index 0 when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.144.0 |  |  |  |  |  |  |  | RET_DEC_ASPECT_RATIO |  |  |  |  |  |  | 0x318 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 792 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Sample Aspect Ratio{alt_mode=CMD_CREATE_ADDR_EXT_signal=7;virtualpairUid=2022-09-0709:27:32T38f35ffa-5d2c-443b-a1a3-7b811f6c6cdc;alternate_reg=CMD_CREATE_ADDR_EXT} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | SAR_WIDTH |  |  | rw | ro |  | 0x0 |  |  | H.265 : the horizontal size of the sample aspect ratio. If it is not decoded in bitstream (VUI in case of H.265), the value of SarWidth is equal to 0. VP9 : N/A{has_reset=0} |  |  |  |  |  |
| 15:0 | SAR_HEIGHT |  |  | rw | ro |  | 0x0 |  |  | H.265 : the vertical size of the sample aspect ratio. If it is not decoded in bitstream(VUI in case of H.265), the value of SarHeight is equal to 0.{has_reset=0} |  |  |  |  |  |

| 1.1.1.146.0 |  |  |  |  |  |  |  | RET_ENC_PIC_IDX |  |  |  |  |  |  | 0x318 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 792 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| A frame buffer index of the encoded picture {alt_mode=CMD_CREATE_ADDR_EXT_signal=8;virtualpairUid=2022-09-0709:27:32T38f35ffa-5d2c-443b-a1a3-7b811f6c6cdc;alternate_reg=CMD_CREATE_ADDR_EXT} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | PIC_IDX |  |  | rw | ro |  | 0x0 |  |  | Greater than or equal to 0 : encoded picture buffer index -1 : encoding end -2 : encoding delay (default) -3 : header encoded only (no VCL) -4 : change of encoding parameter{has_reset=0} |  |  |  |  |  |

| 1.1.1.147.0 |  |  |  |  |  |  |  | CMD_ENC_MV_HISTO_CLASS1 |  |  |  |  |  |  | 0x31C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 796 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_MV_HISTO_CLASS1{alt_mode=CMD_DEC_FORCE_FB_LATENCY_PLUS1_signal=1;virtualpairUid=2022-09-0709:27:32T1b95734c-2e2e-4e4b-8df1-d6dcd5e28ac5;alternate_reg=CMD_DEC_FORCE_FB_LATENCY_PLUS1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | MV_CLASS_THRESHOLD_2 |  |  | rw | ro |  | 0x0 |  |  | MV threshold value for MV histogram class 2 (MV precison = 1/4-pel){has_reset=0} |  |  |  |  |  |
| 15:0 | MV_CLASS_THRESHOLD_3 |  |  | rw | ro |  | 0x0 |  |  | tile_y_meta_stride{has_reset=0} |  |  |  |  |  |

| 1.1.1.148.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_CONF_WIN_TOP_BOT |  |  |  |  |  |  | 0x31C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 796 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Top and bottom size for conformance window {alt_mode=CMD_DEC_FORCE_FB_LATENCY_PLUS1_signal=2;virtualpairUid=2022-09-0709:27:32T1b95734c-2e2e-4e4b-8df1-d6dcd5e28ac5;alternate_reg=CMD_DEC_FORCE_FB_LATENCY_PLUS1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | CONFORMACE_WINDOW_SIZE_BOTTOM |  |  | rw | ro |  | 0x0 |  |  | A conformance bottom window size (0 ~ 8192){has_reset=0} |  |  |  |  |  |
| 15:0 | CONFORMACE_WINDOW_SIZE_TOP |  |  | rw | ro |  | 0x0 |  |  | A conformance top window size (0 ~ 8192){has_reset=0} |  |  |  |  |  |

| 1.1.1.149.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET0 |  |  |  |  |  |  | 0x31C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 796 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index 0{alt_mode=CMD_DEC_FORCE_FB_LATENCY_PLUS1_signal=3;virtualpairUid=2022-09-0709:27:32T1b95734c-2e2e-4e4b-8df1-d6dcd5e28ac5;alternate_reg=CMD_DEC_FORCE_FB_LATENCY_PLUS1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE0 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 0. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.150.0 |  |  |  |  |  |  |  | RET_DEC_BIT_RATE |  |  |  |  |  |  | 0x31C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 796 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Maximum Bit Rate{alt_mode=CMD_DEC_FORCE_FB_LATENCY_PLUS1_signal=4;virtualpairUid=2022-09-0709:27:32T1b95734c-2e2e-4e4b-8df1-d6dcd5e28ac5;alternate_reg=CMD_DEC_FORCE_FB_LATENCY_PLUS1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | MAX_BIT_RATE |  |  | rw | ro |  | 0x0 |  |  | The maximum input bit rate for maxNumSubLayer, as specified in sub layer HRD parameter of H.265. The maximum input bit rate specified in active SPS of AVS2{has_reset=0} |  |  |  |  |  |

| 1.1.1.152.0 |  |  |  |  |  |  |  | RET_ENC_PIC_SLICE_NUM |  |  |  |  |  |  | 0x31C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 796 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Number of slice segments{alt_mode=CMD_DEC_FORCE_FB_LATENCY_PLUS1_signal=5;virtualpairUid=2022-09-0709:27:32T1b95734c-2e2e-4e4b-8df1-d6dcd5e28ac5;alternate_reg=CMD_DEC_FORCE_FB_LATENCY_PLUS1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | reserved |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 15:0 | PIC_SLICE_NUM |  |  | rw | ro |  | 0x0 |  |  | The number of slices in the encoded picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.153.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_CONF_WIN_LEFT_RIGHT |  |  |  |  |  |  | 0x320 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 800 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Left and right size for conformance window{alt_mode=CMD_ENC_CUSTOM_MAP_OPTION_PARAM_signal=1;virtualpairUid=2022-09-0709:27:32Tfda4929d-ec37-4c3a-a063-12c2a1966d63;alternate_reg=CMD_ENC_CUSTOM_MAP_OPTION_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | CONFORMANCE_WINDOW_SIZE_RIGHT |  |  | rw | ro |  | 0x0 |  |  | A conformance right window size (0 ~ 8192){has_reset=0} |  |  |  |  |  |
| 15:0 | CONFORMANCE_WINDOW_SIZE_LFET |  |  | rw | ro |  | 0x0 |  |  | A conformance left window size (0 ~ 8192){has_reset=0} |  |  |  |  |  |

| 1.1.1.154.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL0 |  |  |  |  |  |  | 0x320 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 800 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index 0{alt_mode=CMD_ENC_CUSTOM_MAP_OPTION_PARAM_signal=2;virtualpairUid=2022-09-0709:27:32Tfda4929d-ec37-4c3a-a063-12c2a1966d63;alternate_reg=CMD_ENC_CUSTOM_MAP_OPTION_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE0 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.155.0 |  |  |  |  |  |  |  | RET_DEC_FRAME_RATE_NR |  |  |  |  |  |  | 0x320 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 800 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Frame Rate Numerator{alt_mode=CMD_ENC_CUSTOM_MAP_OPTION_PARAM_signal=3;virtualpairUid=2022-09-0709:27:32Tfda4929d-ec37-4c3a-a063-12c2a1966d63;alternate_reg=CMD_ENC_CUSTOM_MAP_OPTION_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FRAME_RATE_NR |  |  | rw | ro |  | 0x0 |  |  | It returns a frame rate numerator. This field is valid for H.265 and AVS2. If frame rate syntax is not decoded in bitstream, the value of FrameRateNr is equal to -1. If FRAME_RATE_NR and FRAME_RATE_DR are not-zero values, FrameRate is derived as follows. FrameRate =FRAME_RATE_NR/FRAME_RATE_DR. Otherwise if FRAME_RATE_NR or FRAME_RATE_DR are zero values, the value of FrameRate is invalid.{has_reset=0} |  |  |  |  |  |

| 1.1.1.157.0 |  |  |  |  |  |  |  | RET_ENC_PIC_SKIP |  |  |  |  |  |  | 0x320 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 800 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| A picture skip flag {alt_mode=CMD_ENC_CUSTOM_MAP_OPTION_PARAM_signal=4;virtualpairUid=2022-09-0709:27:32Tfda4929d-ec37-4c3a-a063-12c2a1966d63;alternate_reg=CMD_ENC_CUSTOM_MAP_OPTION_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 7:1 | PIC_SKIP |  |  | rw | ro |  | 0x0 |  |  | A picture skip flag{has_reset=0} |  |  |  |  |  |

| 1.1.1.158.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_RDO_PARAM |  |  |  |  |  |  | 0x324 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 804 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| RDO coding options (MANDATORY){alt_mode=CMD_ENC_CUSTOM_MAP_OPTION_ADDR_signal=1;virtualpairUid=2022-09-0709:27:32Tce3f12d5-fc35-45e7-8b86-f1d2d4a4b6b3;alternate_reg=CMD_ENC_CUSTOM_MAP_OPTION_ADDR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:23 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 22:22 | USE_CUSTOM_LAMBDA |  |  | rw | ro |  | 0x0 |  |  | It enables to change CMD_ENC_CUSTOM_LAMBDA_ADDR.{has_reset=0} |  |  |  |  |  |
| 21:21 | ME_SEARCH_CENTER |  |  | rw | ro |  | 0x0 |  |  | It specifies a search center mode for motion estimation. 0 : the ME search center is fixed as (0, 0). 1 : the ME search center is decided by MVP (motion vertor prediction).{has_reset=0} |  |  |  |  |  |
| 20:20 | USE_CUSTOM_QP_MAP |  |  | rw | ro |  | 0x0 |  |  | It enables custom QP map.{has_reset=0} |  |  |  |  |  |
| 19:19 | USE_CUSTOM_MODE_MAP |  |  | rw | ro |  | 0x0 |  |  | It enables custom mode map.{has_reset=0} |  |  |  |  |  |
| 18:18 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 17:17 | USE_CUSTOM_QROUND_OFFSET |  |  | rw | ro |  | 0x0 |  |  | It enables custom QRoundIntra and QRoundInter{has_reset=0} |  |  |  |  |  |
| 16:10 | RSVD2 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 9:8 | INTRA_NXN |  |  | rw | ro |  | 0x0 |  |  | It enables intra NxN Pu mode{has_reset=0} |  |  |  |  |  |
| 7:5 | RSVD3 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 4:4 | DISABLE_COEF_CLEAR |  |  | rw | ro |  | 0x0 |  |  | Disables the transform coefficient clearing algorithm in P or B picture 0 : All-zero coefficient block is evaluated additionally in RDO 1 : All-zero coefficient block is not evaluated in RDO{has_reset=0} |  |  |  |  |  |
| 3:3 | USE_ADAPTIVE_ROUNDING |  |  | rw | ro |  | 0x0 |  |  | It enables adaptive rounding and ignore QRoundIntra and QRoundInter{has_reset=0} |  |  |  |  |  |
| 2:2 | EN_HVS_QP |  |  | rw | ro |  | 0x0 |  |  | It enables CU QP adjustment for subjective quality enhancement.{has_reset=0} |  |  |  |  |  |
| 1:0 | RSVD4 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |

| 1.1.1.159.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE0 |  |  |  |  |  |  | 0x324 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 804 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 0{alt_mode=CMD_ENC_CUSTOM_MAP_OPTION_ADDR_signal=2;virtualpairUid=2022-09-0709:27:32Tce3f12d5-fc35-45e7-8b86-f1d2d4a4b6b3;alternate_reg=CMD_ENC_CUSTOM_MAP_OPTION_ADDR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SUB_SAMPLED_FB_BASE0 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frame 0{has_reset=0} |  |  |  |  |  |

| 1.1.1.160.0 |  |  |  |  |  |  |  | RET_DEC_FRAME_RATE_DR |  |  |  |  |  |  | 0x324 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 804 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Frame Rate Denominator{alt_mode=CMD_ENC_CUSTOM_MAP_OPTION_ADDR_signal=3;virtualpairUid=2022-09-0709:27:32Tce3f12d5-fc35-45e7-8b86-f1d2d4a4b6b3;alternate_reg=CMD_ENC_CUSTOM_MAP_OPTION_ADDR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FRAME_RATE_DR |  |  | rw | ro |  | 0x0 |  |  | It returns a frame rate denominator. This field is valid for H.265 and AVS2. If frame rate syntax is not decoded in bitstream, the value of FrameRateDr is equal to -1.{has_reset=0} |  |  |  |  |  |

| 1.1.1.162.0 |  |  |  |  |  |  |  | RET_ENC_PIC_NUM_INTRA |  |  |  |  |  |  | 0x324 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 804 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Number of intra block{alt_mode=CMD_ENC_CUSTOM_MAP_OPTION_ADDR_signal=4;virtualpairUid=2022-09-0709:27:32Tce3f12d5-fc35-45e7-8b86-f1d2d4a4b6b3;alternate_reg=CMD_ENC_CUSTOM_MAP_OPTION_ADDR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | PIC_NUM_INTRA |  |  | rw | ro |  | 0x0 |  |  | The number of intra block in 8x8{has_reset=0} |  |  |  |  |  |

| 1.1.1.163.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE1 |  |  |  |  |  |  | 0x328 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 808 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 1{alt_mode=CMD_ENC_SEQ_SLICE_PARAM_signal=1;virtualpairUid=2022-09-0709:27:32T82a2a29b-f5f5-412f-9f20-e6fc01c64e82;alternate_reg=CMD_ENC_SEQ_SLICE_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE1 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 1 Compressed Luma base address of DPB index 1{has_reset=0} |  |  |  |  |  |

| 1.1.1.164.0 |  |  |  |  |  |  |  | RET_DEC_NUM_REQURED_FB |  |  |  |  |  |  | 0x328 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 808 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Required Number of Minimum DPB {alt_mode=CMD_ENC_SEQ_SLICE_PARAM_signal=2;virtualpairUid=2022-09-0709:27:32T82a2a29b-f5f5-412f-9f20-e6fc01c64e82;alternate_reg=CMD_ENC_SEQ_SLICE_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 4:0 | MIN_DPB_NUM |  |  | rw | ro |  | 0x0 |  |  | Required number of frame buffers for decoding sequence {has_reset=0} |  |  |  |  |  |

| 1.1.1.166.0 |  |  |  |  |  |  |  | RET_ENC_PIC_NUM_MERGE |  |  |  |  |  |  | 0x328 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 808 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Number of merge block{alt_mode=CMD_ENC_SEQ_SLICE_PARAM_signal=3;virtualpairUid=2022-09-0709:27:32T82a2a29b-f5f5-412f-9f20-e6fc01c64e82;alternate_reg=CMD_ENC_SEQ_SLICE_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | PIC_NUM_MERGE |  |  | rw | ro |  | 0x0 |  |  | The number of merge block in 8x8{has_reset=0} |  |  |  |  |  |

| 1.1.1.167.0 |  |  |  |  |  |  |  | CMD_ENC_SRC_PIC_IDX |  |  |  |  |  |  | 0x32C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 812 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| GOP structure designation (MANDATORY){alt_mode=CMD_ENC_SEQ_INTRA_REFRESH_signal=1;virtualpairUid=2022-09-0709:27:32T2fdf6ba9-44df-4557-9994-3046c65d5372;alternate_reg=CMD_ENC_SEQ_INTRA_REFRESH} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SRC_IDX |  |  | rw | ro |  | 0x0 |  |  | A buffer index of a source picture (-2 : no more source picture){has_reset=0} |  |  |  |  |  |

| 1.1.1.168.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE1 |  |  |  |  |  |  | 0x32C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 812 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index1{alt_mode=CMD_ENC_SEQ_INTRA_REFRESH_signal=2;virtualpairUid=2022-09-0709:27:32T2fdf6ba9-44df-4557-9994-3046c65d5372;alternate_reg=CMD_ENC_SEQ_INTRA_REFRESH} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE1 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 1 Compressed Cb and Cr base address of DPB index 1{has_reset=0} |  |  |  |  |  |

| 1.1.1.169.0 |  |  |  |  |  |  |  | RET_DEC_NUM_REORDER_DELAY |  |  |  |  |  |  | 0x32C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 812 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Reorder frame number{alt_mode=CMD_ENC_SEQ_INTRA_REFRESH_signal=3;virtualpairUid=2022-09-0709:27:32T2fdf6ba9-44df-4557-9994-3046c65d5372;alternate_reg=CMD_ENC_SEQ_INTRA_REFRESH} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 4:0 | REORDER_DELAY_NUM |  |  | rw | ro |  | 0x0 |  |  | The value of REORDER_DELAY_NUM is reorder picture number of H.265. No picture reordering in VP9. The value of output_reorder_delay of active SPS of AVS2. {has_reset=0} |  |  |  |  |  |

| 1.1.1.171.0 |  |  |  |  |  |  |  | RET_ENC_PIC_NON_REF_PIC_FLAG |  |  |  |  |  |  | 0x32C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 812 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Non reference picture flag{alt_mode=CMD_ENC_SEQ_INTRA_REFRESH_signal=4;virtualpairUid=2022-09-0709:27:32T2fdf6ba9-44df-4557-9994-3046c65d5372;alternate_reg=CMD_ENC_SEQ_INTRA_REFRESH} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | PIC_NON_REF_PIC_FLAG |  |  | rw | ro |  | 0x0 |  |  | Non reference picture flag 0 : reference picture 1 : non reference picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.172.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_INTRA_MIN_MAX_QP |  |  |  |  |  |  | 0x330 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 816 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_INTRA_MIN_MAX_QP{alt_mode=CMD_CREATE_INST_CORE_INFO_signal=1;virtualpairUid=2022-09-0709:27:32Te6ec9a82-4986-41ae-95c6-164a717fa1d4;alternate_reg=CMD_CREATE_INST_CORE_INFO} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:12 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 11:6 | I_MAX_QP |  |  | rw | ro |  | 0x0 |  |  | A maximum QP of I pictures. (0 ~ 51){has_reset=0} |  |  |  |  |  |
| 5:0 | I_MIN_QP |  |  | rw | ro |  | 0x0 |  |  | A minimum QP of I pictures. (0 ~ 51){has_reset=0} |  |  |  |  |  |

| 1.1.1.173.0 |  |  |  |  |  |  |  | CMD_ENC_SRC_ADDR_Y |  |  |  |  |  |  | 0x330 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 816 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Y component source address {alt_mode=CMD_CREATE_INST_CORE_INFO_signal=2;virtualpairUid=2022-09-0709:27:32Te6ec9a82-4986-41ae-95c6-164a717fa1d4;alternate_reg=CMD_CREATE_INST_CORE_INFO} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SRC_ADDR_Y |  |  | rw | ro |  | 0x0 |  |  | An address of luma component of a source picture buffer An address of luma compressed source buffer when PVRIC_EN is 1{has_reset=0} |  |  |  |  |  |

| 1.1.1.174.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE1 |  |  |  |  |  |  | 0x330 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 816 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 1{alt_mode=CMD_CREATE_INST_CORE_INFO_signal=3;virtualpairUid=2022-09-0709:27:32Te6ec9a82-4986-41ae-95c6-164a717fa1d4;alternate_reg=CMD_CREATE_INST_CORE_INFO} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE1 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 1{has_reset=0} |  |  |  |  |  |

| 1.1.1.175.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET1 |  |  |  |  |  |  | 0x330 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 816 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 1{alt_mode=CMD_CREATE_INST_CORE_INFO_signal=4;virtualpairUid=2022-09-0709:27:32Te6ec9a82-4986-41ae-95c6-164a717fa1d4;alternate_reg=CMD_CREATE_INST_CORE_INFO} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE1 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB index 1 when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.176.0 |  |  |  |  |  |  |  | RET_DEC_SUB_LAYER_INFO |  |  |  |  |  |  | 0x330 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 816 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Sub-layer information{alt_mode=CMD_CREATE_INST_CORE_INFO_signal=5;virtualpairUid=2022-09-0709:27:32Te6ec9a82-4986-41ae-95c6-164a717fa1d4;alternate_reg=CMD_CREATE_INST_CORE_INFO} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 25:24 | MAX_SPATIAL_ID |  |  | rw | ro |  | 0x0 |  |  | Max spatial ID in current coded video sequence. (for AV1){has_reset=0} |  |  |  |  |  |
| 20:19 | SPATIAL_ID |  |  | rw | ro |  | 0x0 |  |  | Spatial ID of current picture. (for AV1){has_reset=0} |  |  |  |  |  |
| 18:18 | SPATIAL_SVC_LAYER |  |  | rw | ro |  | 0x0 |  |  | Spatial SVC layer (SVAC decoder only) 0 : base layer picture 1 : enhance layer picture{has_reset=0} |  |  |  |  |  |
| 17:17 | SPATIAL_SVC_MODE |  |  | rw | ro |  | 0x0 |  |  | Spatial SVC mode (SVAC decoder only) 0 : disable inter-layer prediction, simulcast. 1 : enable inter-layer prediction in which EL picture is predicted with motion vector information from the base layer picture.{has_reset=0} |  |  |  |  |  |
| 16:16 | SPATIAL_SVC_FLAG |  |  | rw | ro |  | 0x0 |  |  | Spatial SVC flag (SVAC/AV1 decoder only){has_reset=0} |  |  |  |  |  |
| 15:8 | MAX_TEMPORAL_ID |  |  | rw | ro |  | 0x0 |  |  | Max temporal ID (num_of_temporal_level_minus1 + 1)_x000D_ VP9 : N/A{has_reset=0} |  |  |  |  |  |
| 7:0 | TEMPORAL_ID |  |  | rw | ro |  | 0x0 |  |  | Temporal ID VP9 : N/A{has_reset=0} |  |  |  |  |  |

| 1.1.1.178.0 |  |  |  |  |  |  |  | RET_ENC_PIC_NUM_SKIP |  |  |  |  |  |  | 0x330 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 816 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Number of skip block{alt_mode=CMD_CREATE_INST_CORE_INFO_signal=6;virtualpairUid=2022-09-0709:27:32Te6ec9a82-4986-41ae-95c6-164a717fa1d4;alternate_reg=CMD_CREATE_INST_CORE_INFO} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | PIC_NUM_SKIP |  |  | rw | ro |  | 0x0 |  |  | The number of skip block in 8x8{has_reset=0} |  |  |  |  |  |

| 1.1.1.179.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_RC_FRAME_RATE |  |  |  |  |  |  | 0x334 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 820 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_RC_FRAME_RATE{alt_mode=CMD_CREATE_INST_PRIORITY_signal=1;virtualpairUid=2022-09-0709:27:32T71f10ab9-3169-4b2d-86da-421bc28f21b1;alternate_reg=CMD_CREATE_INST_PRIORITY} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | RC_FRAME_RATE |  |  | rw | ro |  | 0x0 |  |  | A frame rate indicator for rate control For example, the value 30 of frameRateInfo represents 30 frames/sec.{has_reset=0} |  |  |  |  |  |

| 1.1.1.180.0 |  |  |  |  |  |  |  | CMD_ENC_SRC_ADDR_U |  |  |  |  |  |  | 0x334 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 820 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb component source address {alt_mode=CMD_CREATE_INST_PRIORITY_signal=2;virtualpairUid=2022-09-0709:27:32T71f10ab9-3169-4b2d-86da-421bc28f21b1;alternate_reg=CMD_CREATE_INST_PRIORITY} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SRC_ADDR_U |  |  | rw | ro |  | 0x0 |  |  | An address of Cb component of a source picture buffer An address of Cb compressed source buffer when PVRIC_EN is 1{has_reset=0} |  |  |  |  |  |

| 1.1.1.181.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET1 |  |  |  |  |  |  | 0x334 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 820 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index 1{alt_mode=CMD_CREATE_INST_PRIORITY_signal=3;virtualpairUid=2022-09-0709:27:32T71f10ab9-3169-4b2d-86da-421bc28f21b1;alternate_reg=CMD_CREATE_INST_PRIORITY} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE1 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 1. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.182.0 |  |  |  |  |  |  |  | RET_DEC_NOTIFICATION |  |  |  |  |  |  | 0x334 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 820 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Sequence change flag If NOTI_FLAG is equal to 1, VPU detects a new sequence. It may require different decoding resources such as DPB size, DPB number, and etc. Otherwise, VPU decodes the subsequent AU of the sequence.{alt_mode=CMD_CREATE_INST_PRIORITY_signal=4;virtualpairUid=2022-09-0709:27:32T71f10ab9-3169-4b2d-86da-421bc28f21b1;alternate_reg=CMD_CREATE_INST_PRIORITY} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:20 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 19:19 | NOTI_FLAG_DPB_COUNT |  |  | rw | ro |  | 0x0 |  |  | If NOTI_FLAG_DPB_COUNT is equal to 1, the current picture needs the different number of DPB.{has_reset=0} |  |  |  |  |  |
| 18:18 | NOTI_FLAG_BITDEPTH |  |  | rw | ro |  | 0x0 |  |  | If NOTI_FLAG_BITDEPTH is equal to 1, the bitdepth of the currently decoded picture is different from the bitdepth of the previously decoded picture.{has_reset=0} |  |  |  |  |  |
| 17:17 | NOTI_FLAG_INTER_RES_CHANGE |  |  | rw | ro |  | 0x0 |  |  | If NOTI_FLAG_INTER_RES_CHANGE is equal to 1, one DPB needs to be reallocated. (VP9/AV1 decoder only){has_reset=0} |  |  |  |  |  |
| 16:16 | NOTI_FLAG_SIZE |  |  | rw | ro |  | 0x0 |  |  | If NOTI_FLAG_SIZE is equal to 1, the decoded size of the current picture is different from the size of the previously decoded picture. Otherwise, the decoded size of current picture is equal to the size of previously decoded picture.{has_reset=0} |  |  |  |  |  |
| 15:15 | NOTI_FLAG_CHROMA_FORMAT_CHANGE |  |  | rw | ro |  | 0x0 |  |  | If it is equal to 1, the chroma format (4:2:0 etc.) is different from the chroma format of the previously decoded picture.{has_reset=0} |  |  |  |  |  |
| 14:6 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 5:5 | NOTI_FLAG_PROFILE |  |  | rw | ro |  | 0x0 |  |  | If NOTI_FLAG_PROFILE is equal to 1, the profile of the current picture is different from the profile of the previously decoded picture. Otherwise, the profile of the current picture is equal to the profile of the previously decoded picture.{has_reset=0} |  |  |  |  |  |
| 4:0 | RSVD2 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |

| 1.1.1.184.0 |  |  |  |  |  |  |  | RET_ENC_PIC_AVG_CTU_QP |  |  |  |  |  |  | 0x334 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 820 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CTU QP on average{alt_mode=CMD_CREATE_INST_PRIORITY_signal=5;virtualpairUid=2022-09-0709:27:32T71f10ab9-3169-4b2d-86da-421bc28f21b1;alternate_reg=CMD_CREATE_INST_PRIORITY} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | PIC_AVG_CTU_QP |  |  | rw | ro |  | 0x0 |  |  | An average value of CTU QPs{has_reset=0} |  |  |  |  |  |

| 1.1.1.185.0 |  |  |  |  |  |  |  | CMD_ENC_SRC_ADDR_V |  |  |  |  |  |  | 0x338 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 824 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr component source address {alt_mode=CMD_ENC_SEQ_RC_TARGET_RATE_signal=1;virtualpairUid=2022-09-0709:27:32T5e2fe86d-1d70-4964-8c14-ec30876729a2;alternate_reg=CMD_ENC_SEQ_RC_TARGET_RATE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SRC_ADDR_V |  |  | rw | ro |  | 0x0 |  |  | An address of Cr component of a source picture buffer An address of Cr compressed source buffer when EN_CFRAME50 is 1{has_reset=0} |  |  |  |  |  |

| 1.1.1.186.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL1 |  |  |  |  |  |  | 0x338 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 824 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index 1{alt_mode=CMD_ENC_SEQ_RC_TARGET_RATE_signal=2;virtualpairUid=2022-09-0709:27:32T5e2fe86d-1d70-4964-8c14-ec30876729a2;alternate_reg=CMD_ENC_SEQ_RC_TARGET_RATE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE1 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.187.0 |  |  |  |  |  |  |  | RET_DEC_USERDATA_IDC |  |  |  |  |  |  | 0x338 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 824 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Each bit field of USER_DATA_FLAG specifies the report result to USER_DATA_BUF_BASE. (HEVC/AVC only){alt_mode=CMD_ENC_SEQ_RC_TARGET_RATE_signal=3;virtualpairUid=2022-09-0709:27:32T5e2fe86d-1d70-4964-8c14-ec30876729a2;alternate_reg=CMD_ENC_SEQ_RC_TARGET_RATE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:31 | USERDATA_FLAG0 |  |  | rw | ro |  | 0x0 |  |  | A flag of the 3rd user_data_registered_itu_t_t35 suffix sei{has_reset=0} |  |  |  |  |  |
| 30:30 | USERDATA_FLAG1 |  |  | rw | ro |  | 0x0 |  |  | A flag of the 2nd user_data_registered_itu_t_t35 suffix sei{has_reset=0} |  |  |  |  |  |
| 29:29 | USERDATA_FLAG2 |  |  | rw | ro |  | 0x0 |  |  | A flag of the 3rd user_data_registered_itu_t_t35 prefix sei{has_reset=0} |  |  |  |  |  |
| 28:28 | USERDATA_FLAG3 |  |  | rw | ro |  | 0x0 |  |  | A flag of the 2nd user_data_registered_itu_t_t35 prefix sei{has_reset=0} |  |  |  |  |  |
| 27:17 | USERDATA_FLAG4 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 16:16 | USERDATA_FLAG5 |  |  | rw | ro |  | 0x0 |  |  | A flag of colour_remapping_info prefix sei{has_reset=0} |  |  |  |  |  |
| 15:15 | USERDATA_FLAG6 |  |  | rw | ro |  | 0x0 |  |  | A flag of content_light_level_info prefix sei{has_reset=0} |  |  |  |  |  |
| 14:14 | USERDATA_FLAG7 |  |  | rw | ro |  | 0x0 |  |  | A flag of film_grain_characteristics_info prefix sei{has_reset=0} |  |  |  |  |  |
| 13:13 | USERDATA_FLAG8 |  |  | rw | ro |  | 0x0 |  |  | A flag of tone_mapping_info prefix sei{has_reset=0} |  |  |  |  |  |
| 12:12 | USERDATA_FLAG9 |  |  | rw | ro |  | 0x0 |  |  | A flag of knee_function_info sei{has_reset=0} |  |  |  |  |  |
| 11:11 | USERDATA_FLAG10 |  |  | rw | ro |  | 0x0 |  |  | A flag of chroma_resampling_filter_hint prefix sei{has_reset=0} |  |  |  |  |  |
| 10:10 | USERDATA_FLAG11 |  |  | rw | ro |  | 0x0 |  |  | A flag of mastering_display_color_volume prefix sei{has_reset=0} |  |  |  |  |  |
| 9:9 | RSVD12 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 8:8 | USERDATA_FLAG13 |  |  | rw | ro |  | 0x0 |  |  | A flag of user_data_unregistered suffix sei.{has_reset=0} |  |  |  |  |  |
| 7:7 | USERDATA_FLAG14 |  |  | rw | ro |  | 0x0 |  |  | A flag of the 1st user_data_registered_itu_t_t35 suffix sei.{has_reset=0} |  |  |  |  |  |
| 6:6 | USERDATA_FLAG15 |  |  | rw | ro |  | 0x0 |  |  | A flag of user_data_unregistered prefix sei{has_reset=0} |  |  |  |  |  |
| 5:5 | USERDATA_FLAG16 |  |  | rw | ro |  | 0x0 |  |  | A flag of the 1st user_data_registered_itu_t_t35 prefix sei{has_reset=0} |  |  |  |  |  |
| 4:4 | USERDATA_FLAG17 |  |  | rw | ro |  | 0x0 |  |  | A flag of pic_timing sei{has_reset=0} |  |  |  |  |  |
| 3:3 | RSVD18 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 2:2 | USERDATA_FLAG19 |  |  | rw | ro |  | 0x0 |  |  | A flag of vui parameter.{has_reset=0} |  |  |  |  |  |
| 1:1 | USERDATA_FLAG20 |  |  | rw | ro |  | 0x0 |  |  | A flag of user data buffer full. User data buffer full flag equal to 1 specifies that decoded frame has more user data size than VPU internal buffer. VPU only dumps the internal buffer size of user data to USER_DATA_BUF_BASE buffer. In other words, VPU is unable to report the rest of the user data to USER_DATA_BUF_BASE buffer after the internal buffer fullness happens.{has_reset=0} |  |  |  |  |  |
| 0:0 | RSVD21 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |

| 1.1.1.189.0 |  |  |  |  |  |  |  | RET_ENC_PIC_BYTE |  |  |  |  |  |  | 0x338 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 824 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Byte size of encoded picture{alt_mode=CMD_ENC_SEQ_RC_TARGET_RATE_signal=4;virtualpairUid=2022-09-0709:27:32T5e2fe86d-1d70-4964-8c14-ec30876729a2;alternate_reg=CMD_ENC_SEQ_RC_TARGET_RATE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | PIC_BYTE |  |  | rw | ro |  | 0x0 |  |  | The size of encoded picture in byte{has_reset=0} |  |  |  |  |  |

| 1.1.1.190.0 |  |  |  |  |  |  |  | CMD_ENC_SRC_STRIDE |  |  |  |  |  |  | 0x33C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 828 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Stride of source picture{alt_mode=CMD_ENC_SEQ_RC_PARAM_signal=1;virtualpairUid=2022-09-0709:27:32T26638be9-8879-4a66-8008-fb3c88d6f9ab;alternate_reg=CMD_ENC_SEQ_RC_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | SRC_Y_STRIDE |  |  | rw | ro |  | 0x0 |  |  | A luma stride of source picture in pixel{has_reset=0} |  |  |  |  |  |
| 15:0 | SRC_C_STRIDE |  |  | rw | ro |  | 0x0 |  |  | A chroma stride of source picture in pixel. In case of FORMAT_422, it should be doubled.{has_reset=0} |  |  |  |  |  |

| 1.1.1.191.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE1 |  |  |  |  |  |  | 0x33C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 828 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 1{alt_mode=CMD_ENC_SEQ_RC_PARAM_signal=2;virtualpairUid=2022-09-0709:27:32T26638be9-8879-4a66-8008-fb3c88d6f9ab;alternate_reg=CMD_ENC_SEQ_RC_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SUB_SAMPLED_FB_BASE1 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frames 1{has_reset=0} |  |  |  |  |  |

| 1.1.1.192.0 |  |  |  |  |  |  |  | RET_DEC_PIC_SIZE |  |  |  |  |  |  | 0x33C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 828 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Decoded Picture Size{alt_mode=CMD_ENC_SEQ_RC_PARAM_signal=3;virtualpairUid=2022-09-0709:27:32T26638be9-8879-4a66-8008-fb3c88d6f9ab;alternate_reg=CMD_ENC_SEQ_RC_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | DECODED_PIC_WIDTH |  |  | rw | ro |  | 0x0 |  |  | Decoded Picture Width The value of DECODED_PIC_WIDTH is eqaul to the value of pic_width_in_luma_samples in H.265 active SPS. The value of horizontal_size in AVS2 active SPS.{has_reset=0} |  |  |  |  |  |
| 15:0 | DECODED_PIC_HEIGHT |  |  | rw | ro |  | 0x0 |  |  | Decoded Picture Height The value of DECODED_PIC_HEIGHT is eqaul to the value of pic_height_in_luma_samples in H.265 active SPS. The value of vertical_size in AVS2 active SPS.{has_reset=0} |  |  |  |  |  |

| 1.1.1.194.0 |  |  |  |  |  |  |  | RET_ENC_GOP_PIC_IDX |  |  |  |  |  |  | 0x33C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 828 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| A picture index in GOP {alt_mode=CMD_ENC_SEQ_RC_PARAM_signal=4;virtualpairUid=2022-09-0709:27:32T26638be9-8879-4a66-8008-fb3c88d6f9ab;alternate_reg=CMD_ENC_SEQ_RC_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | GOP_PIC_IDX |  |  | rw | ro |  | 0x0 |  |  | A picture index in GOP{has_reset=0} |  |  |  |  |  |

| 1.1.1.195.0 |  |  |  |  |  |  |  | CMD_ENC_SRC_FORMAT |  |  |  |  |  |  | 0x340 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 832 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Format of source picture{alt_mode=CMD_ENC_SEQ_HVS_PARAM_signal=1;virtualpairUid=2022-09-0709:27:32T94a5ed0e-4d23-40d5-b894-8376d608dea1;alternate_reg=CMD_ENC_SEQ_HVS_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:25 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 15:12 | src_endian |  |  | rw | ro |  | 0x0 |  |  | Endianness of source picture{has_reset=0} |  |  |  |  |  |
| 11:8 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 6:6 | src_sample_left_aligned |  |  | rw | ro |  | 0x0 |  |  | (Only for 10bit) 0 : source data bits located MSB 1 : source data bits located LSB{has_reset=0} |  |  |  |  |  |
| 5:5 | src_sample_special_flag |  |  | rw | ro |  | 0x0 |  |  | (Only for 10bit) 0 : 1 pixels packing in 2-byte 1 : 3 pixels packing in 4-byte{has_reset=0} |  |  |  |  |  |
| 4:4 | src_sample_10bit |  |  | rw | ro |  | 0x0 |  |  | 0 : 8bit 1 : 10bit{has_reset=0} |  |  |  |  |  |
| 3:3 | src_plane_special_flag |  |  | rw | ro |  | 0x0 |  |  | When source is 1-plane, 0 : YUYV, YVYU 1 : UYUV, VYUY{has_reset=0} |  |  |  |  |  |
| 2:2 | src_plane_cr_first |  |  | rw | ro |  | 0x0 |  |  | For 1-plane, 0 : YUYV 1 : YVYU For 2-plane, 0 : NV12, NV16 1 : NV21, NV61 For 3-plane, there is no effect because cb_base and cr_base have a seperated register.{has_reset=0} |  |  |  |  |  |
| 1:0 | src_plane_num |  |  | rw | ro |  | 0x0 |  |  | 0 : prohibited but hardware retrieves it as 3 plane. 1 : 1-plane (Packed - only for YUV422) 2 : 2-plane (one of NV12/21/16/61) 0, 3 : 3-plane (Planar){has_reset=0} |  |  |  |  |  |

| 1.1.1.196.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE2 |  |  |  |  |  |  | 0x340 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 832 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 2{alt_mode=CMD_ENC_SEQ_HVS_PARAM_signal=2;virtualpairUid=2022-09-0709:27:32T94a5ed0e-4d23-40d5-b894-8376d608dea1;alternate_reg=CMD_ENC_SEQ_HVS_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE2 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 2 Compressed Luma base address of DPB index 2{has_reset=0} |  |  |  |  |  |

| 1.1.1.197.0 |  |  |  |  |  |  |  | RET_DEC_CROP_TOP_BOTTOM |  |  |  |  |  |  | 0x340 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 832 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Display Crop Offset Top/Bottom{alt_mode=CMD_ENC_SEQ_HVS_PARAM_signal=3;virtualpairUid=2022-09-0709:27:32T94a5ed0e-4d23-40d5-b894-8376d608dea1;alternate_reg=CMD_ENC_SEQ_HVS_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | DISPLAY_TOP_OFFSET |  |  | rw | ro |  | 0x0 |  |  | Display top offset DISPLAY_TOP_OFFSET is equal to leftOffset. topOffset = conf_win_top_offset + def_disp_win_top_offset. See Annex E.3 of H.265 specification for more information.{has_reset=0} |  |  |  |  |  |
| 15:0 | DISPLAY_BOTTOM_OFFSET |  |  | rw | ro |  | 0x0 |  |  | Display bottom offset DISPLAY_BOTTOM_OFFSET is equal to bottomOffset. bottomOffset = conf_win_bottom_offset + def_disp_win_bottom_offset{has_reset=0} |  |  |  |  |  |

| 1.1.1.199.0 |  |  |  |  |  |  |  | RET_ENC_USED_SRC_IDX |  |  |  |  |  |  | 0x340 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 832 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Buffer index of source picture that is used for encoding{alt_mode=CMD_ENC_SEQ_HVS_PARAM_signal=4;virtualpairUid=2022-09-0709:27:32T94a5ed0e-4d23-40d5-b894-8376d608dea1;alternate_reg=CMD_ENC_SEQ_HVS_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | USED_SRC_PIC_IDX |  |  | rw | ro |  | 0x0 |  |  | A source buffer index of the encoded picture (-2 : encoding delay){has_reset=0} |  |  |  |  |  |

| 1.1.1.200.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE2 |  |  |  |  |  |  | 0x344 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 836 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index 2{alt_mode=CMD_ENC_SEQ_RC_MAX_BITRATE_signal=1;virtualpairUid=2022-09-0709:27:32T5e4acd37-0329-4cfc-bdcf-6ae569421836;alternate_reg=CMD_ENC_SEQ_RC_MAX_BITRATE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE2 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 2 Compressed Cb and Cr base address of DPB index 2{has_reset=0} |  |  |  |  |  |

| 1.1.1.201.0 |  |  |  |  |  |  |  | RET_DEC_CROP_LEFT_RIGHT |  |  |  |  |  |  | 0x344 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 836 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Display Crop Offset Left/Right{alt_mode=CMD_ENC_SEQ_RC_MAX_BITRATE_signal=2;virtualpairUid=2022-09-0709:27:32T5e4acd37-0329-4cfc-bdcf-6ae569421836;alternate_reg=CMD_ENC_SEQ_RC_MAX_BITRATE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | DISPLAY_LEFT_OFFSET |  |  | rw | ro |  | 0x0 |  |  | Display left offset DISPLAY_LEFT_OFFSET is equal to leftOffset. leftOffset = conf_win_left_offset + def_disp_win_left_offset{has_reset=0} |  |  |  |  |  |
| 15:0 | DISPLAY_RIGHT_OFFSET |  |  | rw | ro |  | 0x0 |  |  | Display right offset DISPLAY_RIGHT_OFFSET is equal to rightOffset. rightOffset = conf_win_right_offset + def_disp_win_right_offset{has_reset=0} |  |  |  |  |  |

| 1.1.1.203.0 |  |  |  |  |  |  |  | RET_ENC_PIC_NUM |  |  |  |  |  |  | 0x344 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 836 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Encoded picture number{alt_mode=CMD_ENC_SEQ_RC_MAX_BITRATE_signal=3;virtualpairUid=2022-09-0709:27:32T5e4acd37-0329-4cfc-bdcf-6ae569421836;alternate_reg=CMD_ENC_SEQ_RC_MAX_BITRATE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | PIC_NUM |  |  | rw | ro |  | 0x0 |  |  | The encoded picture number{has_reset=0} |  |  |  |  |  |

| 1.1.1.204.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_RC_VBV_BUFFER_SIZE |  |  |  |  |  |  | 0x348 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 840 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_RC_VBV_BUFFER_SIZE{alt_mode=CMD_CREATE_INST_ADDR_TEMP_BASE_signal=1;virtualpairUid=2022-09-0709:27:32Te4120e6d-9df2-4cb6-b629-7530429bdc0d;alternate_reg=CMD_CREATE_INST_ADDR_TEMP_BASE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | RC_VBV_BUFFER_SIZE |  |  | rw | ro |  | 0x0 |  |  | VBV buffer size in msec{has_reset=0} |  |  |  |  |  |

| 1.1.1.205.0 |  |  |  |  |  |  |  | CMD_ENC_SRC_AXI_SEL |  |  |  |  |  |  | 0x348 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 840 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Selection of AXI port to load source pictures {alt_mode=CMD_CREATE_INST_ADDR_TEMP_BASE_signal=2;virtualpairUid=2022-09-0709:27:32Te4120e6d-9df2-4cb6-b629-7530429bdc0d;alternate_reg=CMD_CREATE_INST_ADDR_TEMP_BASE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 0:0 | PRP_GDI_SEL |  |  | rw | ro |  | 0x0 |  |  | 0 : PRP port (default) 1 : Primary port{has_reset=0} |  |  |  |  |  |

| 1.1.1.206.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE2 |  |  |  |  |  |  | 0x348 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 840 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 2{alt_mode=CMD_CREATE_INST_ADDR_TEMP_BASE_signal=3;virtualpairUid=2022-09-0709:27:32Te4120e6d-9df2-4cb6-b629-7530429bdc0d;alternate_reg=CMD_CREATE_INST_ADDR_TEMP_BASE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE2 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 2{has_reset=0} |  |  |  |  |  |

| 1.1.1.207.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET2 |  |  |  |  |  |  | 0x348 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 840 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 2{alt_mode=CMD_CREATE_INST_ADDR_TEMP_BASE_signal=4;virtualpairUid=2022-09-0709:27:32Te4120e6d-9df2-4cb6-b629-7530429bdc0d;alternate_reg=CMD_CREATE_INST_ADDR_TEMP_BASE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE2 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB index 2 when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.208.0 |  |  |  |  |  |  |  | RET_DEC_AU_START_POS |  |  |  |  |  |  | 0x348 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 840 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| AU Bitstream Start Position{alt_mode=CMD_CREATE_INST_ADDR_TEMP_BASE_signal=5;virtualpairUid=2022-09-0709:27:32Te4120e6d-9df2-4cb6-b629-7530429bdc0d;alternate_reg=CMD_CREATE_INST_ADDR_TEMP_BASE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | AU_START_POS |  |  | rw | ro |  | 0x0 |  |  | Access unit(AU) bitstream start position{has_reset=0} |  |  |  |  |  |

| 1.1.1.210.0 |  |  |  |  |  |  |  | RET_ENC_VCL_NUT |  |  |  |  |  |  | 0x348 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 840 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Encoded NAL unit type of VCL{alt_mode=CMD_CREATE_INST_ADDR_TEMP_BASE_signal=6;virtualpairUid=2022-09-0709:27:32Te4120e6d-9df2-4cb6-b629-7530429bdc0d;alternate_reg=CMD_CREATE_INST_ADDR_TEMP_BASE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | VCL_NUT |  |  | rw | ro |  | 0x0 |  |  | The type of the VCL NAL unit which has been encoded for the current command{has_reset=0} |  |  |  |  |  |

| 1.1.1.211.0 |  |  |  |  |  |  |  | CMD_ENC_CODE_OPTION |  |  |  |  |  |  | 0x34C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 844 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CODE_OPTION{alt_mode=CMD_CREATE_INST_TEMP_SIZE_signal=1;virtualpairUid=2022-09-0709:27:32T534f9b1f-97aa-4d7b-8dfb-a0eb35e0a9ee;alternate_reg=CMD_CREATE_INST_TEMP_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:8 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 7:7 | EOB |  |  | rw | ro |  | 0x0 |  |  | A flag to encode EOB nal unit explicitly (HEVC, AVC).{has_reset=0} |  |  |  |  |  |
| 6:6 | EOS |  |  | rw | ro |  | 0x0 |  |  | A flag to encode EOS nal unit explicitly (HEVC, AVC).{has_reset=0} |  |  |  |  |  |
| 5:5 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 4:4 | PPS |  |  | rw | ro |  | 0x0 |  |  | A flag to encode PPS nal unit explicitly (HEVC, AVC). A flag to encode FRAME_HEADER_OBU explicitly (AV1).{has_reset=0} |  |  |  |  |  |
| 3:3 | SPS |  |  | rw | ro |  | 0x0 |  |  | A flag to encode SPS nal unit explicitly (HEVC, AVC). A flag to encode SEQ_HEADER_OBU explicitly (AV1).{has_reset=0} |  |  |  |  |  |
| 2:2 | VPS |  |  | rw | ro |  | 0x0 |  |  | A flag to encode VPS nal unit explicitly (HEVC only){has_reset=0} |  |  |  |  |  |
| 1:1 | VCL |  |  | rw | ro |  | 0x0 |  |  | A flag to encode VCL nal unit explicitly (HEVC, AVC). A flag to encode TILE_GROUP_OBU explicitly (AV1).{has_reset=0} |  |  |  |  |  |
| 0:0 | IMPLICITLY_HEADER_ENCODE |  |  | rw | ro |  | 0x0 |  |  | A flag to enable to encode a header or headers (SPS, PPS) implicitly for generating bitstreams conforming to specification{has_reset=0} |  |  |  |  |  |

| 1.1.1.212.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_INTER_MIN_MAX_QP |  |  |  |  |  |  | 0x34C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 844 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_INTER_MIN_MAX_QP{alt_mode=CMD_CREATE_INST_TEMP_SIZE_signal=2;virtualpairUid=2022-09-0709:27:32T534f9b1f-97aa-4d7b-8dfb-a0eb35e0a9ee;alternate_reg=CMD_CREATE_INST_TEMP_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:24 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 23:18 | B_MAX_QP |  |  | rw | ro |  | 0x0 |  |  | A maximum QP of B pictures. (0 ~ 51){has_reset=0} |  |  |  |  |  |
| 17:12 | B_MIN_QP |  |  | rw | ro |  | 0x0 |  |  | A minimum QP of B pictures. (0 ~ 51){has_reset=0} |  |  |  |  |  |
| 11:6 | P_MAX_QP |  |  | rw | ro |  | 0x0 |  |  | A maximum QP of P pictures. (0 ~ 51){has_reset=0} |  |  |  |  |  |
| 5:0 | P_MIN_QP |  |  | rw | ro |  | 0x0 |  |  | A minimum QP of P pictures. (0 ~ 51){has_reset=0} |  |  |  |  |  |

| 1.1.1.213.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET2 |  |  |  |  |  |  | 0x34C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 844 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index 2{alt_mode=CMD_CREATE_INST_TEMP_SIZE_signal=3;virtualpairUid=2022-09-0709:27:32T534f9b1f-97aa-4d7b-8dfb-a0eb35e0a9ee;alternate_reg=CMD_CREATE_INST_TEMP_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE2 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 2. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.214.0 |  |  |  |  |  |  |  | RET_DEC_AU_END_POS |  |  |  |  |  |  | 0x34C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 844 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| AU Bitstream End Position{alt_mode=CMD_CREATE_INST_TEMP_SIZE_signal=4;virtualpairUid=2022-09-0709:27:32T534f9b1f-97aa-4d7b-8dfb-a0eb35e0a9ee;alternate_reg=CMD_CREATE_INST_TEMP_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | AU_END_POS |  |  | rw | ro |  | 0x0 |  |  | Access unit(AU) bitstream end position{has_reset=0} |  |  |  |  |  |

| 1.1.1.216.0 |  |  |  |  |  |  |  | RET_ENC_PIC_PADD_BYTE |  |  |  |  |  |  | 0x34C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 844 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Encoded NAL padding byte num{alt_mode=CMD_CREATE_INST_TEMP_SIZE_signal=5;virtualpairUid=2022-09-0709:27:32T534f9b1f-97aa-4d7b-8dfb-a0eb35e0a9ee;alternate_reg=CMD_CREATE_INST_TEMP_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | PADD_BYTE |  |  | rw | ro |  | 0x0 |  |  | The size of encoded picture padd in NAL byte{has_reset=0} |  |  |  |  |  |

| 1.1.1.217.0 |  |  |  |  |  |  |  | CMD_ENC_PIC_PARAM |  |  |  |  |  |  | 0x350 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 848 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Picture level encoding parameters {alt_mode=CMD_CREATE_INST_ADDR_SEC_AXI_signal=1;virtualpairUid=2022-09-0709:27:32Tb40b4091-1660-4f6f-91eb-7ba45d239368;alternate_reg=CMD_CREATE_INST_ADDR_SEC_AXI} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:24 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 23:21 | FORCE_PIC_TYPE |  |  | rw | ro |  | 0x0 |  |  | A force picture type (I, P, IDR) or an irap type 0 : I picture (non-IRAP picture) 1 : P picture (valid if original picture type is not an I picture) 2 : reserved 3 : IDR picture{has_reset=0} |  |  |  |  |  |
| 20:20 | USE_FORCE_PIC_TYPE |  |  | rw | ro |  | 0x0 |  |  | A flag whether to use a force picture type or an irap type{has_reset=0} |  |  |  |  |  |
| 19:14 | FORCE_PIC_QP_B |  |  | rw | ro |  | 0x0 |  |  | A force picture quantization parameter for B picture (0 ~ 51){has_reset=0} |  |  |  |  |  |
| 13:8 | FORCE_PIC_QP_P |  |  | rw | ro |  | 0x0 |  |  | A force picture quantization parameter for P picture (0 ~ 51){has_reset=0} |  |  |  |  |  |
| 7:2 | FORCE_PIC_QP_I |  |  | rw | ro |  | 0x0 |  |  | A force picture quantization parameter for I picture (0 ~ 51){has_reset=0} |  |  |  |  |  |
| 1:1 | USE_FORCE_PIC_QP |  |  | rw | ro |  | 0x0 |  |  | A flag to use a force picture quantization parameter{has_reset=0} |  |  |  |  |  |
| 0:0 | PIC_SKIP_FLAG |  |  | rw | ro |  | 0x0 |  |  | A flag to skip the current picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.218.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_ROT_PARAM |  |  |  |  |  |  | 0x350 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 848 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_ROT_PARAM{alt_mode=CMD_CREATE_INST_ADDR_SEC_AXI_signal=2;virtualpairUid=2022-09-0709:27:32Tb40b4091-1660-4f6f-91eb-7ba45d239368;alternate_reg=CMD_CREATE_INST_ADDR_SEC_AXI} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:5 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 4:1 | ROTATE_MODE |  |  | rw | ro |  | 0x0 |  |  | [3] - horizontal mirror [2] - vertical mirror [1:0] - 90 degree left rotate 1=90' 2=180' 3=270'{has_reset=0} |  |  |  |  |  |
| 0:0 | ROTATE_ENABLE |  |  | rw | ro |  | 0x0 |  |  | It enables or disables rotation.{has_reset=0} |  |  |  |  |  |

| 1.1.1.219.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL2 |  |  |  |  |  |  | 0x350 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 848 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index 2{alt_mode=CMD_CREATE_INST_ADDR_SEC_AXI_signal=3;virtualpairUid=2022-09-0709:27:32Tb40b4091-1660-4f6f-91eb-7ba45d239368;alternate_reg=CMD_CREATE_INST_ADDR_SEC_AXI} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE2 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.220.0 |  |  |  |  |  |  |  | RET_DEC_PIC_TYPE |  |  |  |  |  |  | 0x350 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 848 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Decoded picture type{alt_mode=CMD_CREATE_INST_ADDR_SEC_AXI_signal=4;virtualpairUid=2022-09-0709:27:32Tb40b4091-1660-4f6f-91eb-7ba45d239368;alternate_reg=CMD_CREATE_INST_ADDR_SEC_AXI} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:31 | OUTPUT_FLAG |  |  | rw | ro |  | 0x0 |  |  | H.265/H.264 : a picture output flag VP9 : show_frame If OUTPUT_FLAG is 0, the current decoded picture is not output for display. If OUTPUT_FLAG is 1, the current decoded picture is (will be) output for display. AVS2 : 0 if decoded picture is GB. 1 for other picture types{has_reset=0} |  |  |  |  |  |
| 30:30 | REF_PIC_FLAG |  |  | rw | ro |  | 0x0 |  |  | H.264 0 : non reference picture 1 : reference picture{has_reset=0} |  |  |  |  |  |
| 11:10 | CTU_SIZE |  |  | rw | ro |  | 0x0 |  |  | CTU size 0 : 16x16 1 : 32x32 2 : 64x64{has_reset=0} |  |  |  |  |  |
| 9:4 | VCL_NAL_UNIT_TYPE |  |  | rw | ro |  | 0x0 |  |  | VCL NAL unit type{has_reset=0} |  |  |  |  |  |
| 3:3 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 2:0 | PIC_TYPE |  |  | rw | ro |  | 0x0 |  |  | H.265/H.264 Each field shows the decoded slices for the picture. If more than one B slice are decoded, it returns 4 with the value of B_SLICE_DECODED[2] equal to 1. If there are all types of slices in a decoded picture, it returns 7. [2] B_SLICE_DECODED [1] P_SLICE_DECODED [0] I_SLICE_DECODED VP9, 1 : I picture 2 : P picture AV1 0: KEY FRAME 1: INTER FRAME 2 : INTRA ONLY FRAME 3: SWITCH FRAME AVS2, 0 : I picture 1 : P picture 2 : B picture 3 : F picture 4 : S picture 5 : G picture 6 : GB picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.222.0 |  |  |  |  |  |  |  | RET_ENC_PIC_DIST_LOW |  |  |  |  |  |  | 0x350 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 848 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Low 32bit SSD {alt_mode=CMD_CREATE_INST_ADDR_SEC_AXI_signal=5;virtualpairUid=2022-09-0709:27:32Tb40b4091-1660-4f6f-91eb-7ba45d239368;alternate_reg=CMD_CREATE_INST_ADDR_SEC_AXI} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DIST_Y_LOW |  |  | rw | ro |  | 0x0 |  |  | Low 32bit SSD between source Y picture and reconstructed Y picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.223.0 |  |  |  |  |  |  |  | CMD_ENC_LONGTERM_PIC |  |  |  |  |  |  | 0x354 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 852 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Longterm picture setting{alt_mode=CMD_CREATE_INST_SEC_AXI_SIZE_signal=1;virtualpairUid=2022-09-0709:27:32T73f57c14-4c40-49da-89f4-dba854a53f83;alternate_reg=CMD_CREATE_INST_SEC_AXI_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:2 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | RESERVED{has_reset=0} |  |  |  |  |  |
| 1:1 | cmd_enc_longterm_pic |  |  | rw | ro |  | 0x0 |  |  | A flag to use a longterm reference picture in DPB when encoding the current picture{has_reset=0} |  |  |  |  |  |
| 0:0 | USE_SRC_LONGTERM_PIC |  |  | rw | ro |  | 0x0 |  |  | A flag for the current picture to be used as a longterm reference picture later when other picture's encoding{has_reset=0} |  |  |  |  |  |

| 1.1.1.224.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_NUM_UNITS_IN_TICK |  |  |  |  |  |  | 0x354 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 852 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_NUM_UNITS_IN_TICK{alt_mode=CMD_CREATE_INST_SEC_AXI_SIZE_signal=2;virtualpairUid=2022-09-0709:27:32T73f57c14-4c40-49da-89f4-dba854a53f83;alternate_reg=CMD_CREATE_INST_SEC_AXI_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | NUM_UNITS_IN_TICK |  |  | rw | ro |  | 0x0 |  |  | It specifies the number of time units of a clock operating at the frequency time_scale Hz.{has_reset=0} |  |  |  |  |  |

| 1.1.1.225.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE2 |  |  |  |  |  |  | 0x354 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 852 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 2{alt_mode=CMD_CREATE_INST_SEC_AXI_SIZE_signal=3;virtualpairUid=2022-09-0709:27:32T73f57c14-4c40-49da-89f4-dba854a53f83;alternate_reg=CMD_CREATE_INST_SEC_AXI_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SUB_SAMPLED_FB_BASE2 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frame 2{has_reset=0} |  |  |  |  |  |

| 1.1.1.226.0 |  |  |  |  |  |  |  | RET_DEC_PIC_POC |  |  |  |  |  |  | 0x354 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 852 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Picture Order Count{alt_mode=CMD_CREATE_INST_SEC_AXI_SIZE_signal=4;virtualpairUid=2022-09-0709:27:32T73f57c14-4c40-49da-89f4-dba854a53f83;alternate_reg=CMD_CREATE_INST_SEC_AXI_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | PIC_ORDER_COUNT |  |  | rw | ro |  | 0x0 |  |  | H.265 : picture order count VP9: N/A AVS2 : display order index (POI) POI = DOI + PictureOutputDelay - OutputReorderDelay{has_reset=0} |  |  |  |  |  |

| 1.1.1.228.0 |  |  |  |  |  |  |  | RET_ENC_PIC_DIST_HIGH |  |  |  |  |  |  | 0x354 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 852 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| High 32bit SSD {alt_mode=CMD_CREATE_INST_SEC_AXI_SIZE_signal=5;virtualpairUid=2022-09-0709:27:32T73f57c14-4c40-49da-89f4-dba854a53f83;alternate_reg=CMD_CREATE_INST_SEC_AXI_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DIST_Y_HIGH |  |  | rw | ro |  | 0x0 |  |  | High 32bit SSD between source Y picture and reconstructed Y picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.229.0 |  |  |  |  |  |  |  | CMD_ENC_PREFIX_SEI_NAL_ADDR |  |  |  |  |  |  | 0x358 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 856 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_PREFIX_SEI_NAL_ADDR{alt_mode=CMD_CREATE_INST_ADDR_AR_TABLE_signal=1;virtualpairUid=2022-09-0709:27:32T36fca6a2-39fb-4b16-a005-358f3ef7c941;alternate_reg=CMD_CREATE_INST_ADDR_AR_TABLE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | PREFIX_SEI_NAL_DATA_ADDR |  |  | rw | ro |  | 0x0 |  |  | The base address of the PREFIX SEI NAL buffer (Big endian){has_reset=0} |  |  |  |  |  |

| 1.1.1.230.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_TIME_SCALE |  |  |  |  |  |  | 0x358 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 856 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_TIME_SCALE{alt_mode=CMD_CREATE_INST_ADDR_AR_TABLE_signal=2;virtualpairUid=2022-09-0709:27:32T36fca6a2-39fb-4b16-a005-358f3ef7c941;alternate_reg=CMD_CREATE_INST_ADDR_AR_TABLE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | TIME_SCALE |  |  | rw | ro |  | 0x0 |  |  | It specifies the number of time units that pass in one second.{has_reset=0} |  |  |  |  |  |

| 1.1.1.231.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE3 |  |  |  |  |  |  | 0x358 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 856 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 3{alt_mode=CMD_CREATE_INST_ADDR_AR_TABLE_signal=3;virtualpairUid=2022-09-0709:27:32T36fca6a2-39fb-4b16-a005-358f3ef7c941;alternate_reg=CMD_CREATE_INST_ADDR_AR_TABLE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE3 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 3 Compressed Luma base address of DPB index 3{has_reset=0} |  |  |  |  |  |

| 1.1.1.232.0 |  |  |  |  |  |  |  | RET_DEC_RECOVERY_POINT |  |  |  |  |  |  | 0x358 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 856 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Recovery point of H.265{alt_mode=CMD_CREATE_INST_ADDR_AR_TABLE_signal=4;virtualpairUid=2022-09-0709:27:32T36fca6a2-39fb-4b16-a005-358f3ef7c941;alternate_reg=CMD_CREATE_INST_ADDR_AR_TABLE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 18:18 | EXIST_FLAG |  |  | rw | ro |  | 0x0 |  |  | exist_flag{has_reset=0} |  |  |  |  |  |
| 17:17 | BROKEN_LINK_FLAG |  |  | rw | ro |  | 0x0 |  |  | broken_link_flag{has_reset=0} |  |  |  |  |  |
| 16:16 | EXACT_MATCH_FLAG |  |  | rw | ro |  | 0x0 |  |  | exact_match_flag{has_reset=0} |  |  |  |  |  |
| 15:0 | SIGNED_RECOVERY_POC_CNT |  |  | rw | ro |  | 0x0 |  |  | signed recovery_poc_cnt{has_reset=0} |  |  |  |  |  |

| 1.1.1.234.0 |  |  |  |  |  |  |  | RET_ENC_MAX_LATENCY_PICTURES |  |  |  |  |  |  | 0x358 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 856 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| The number of latency picture{alt_mode=CMD_CREATE_INST_ADDR_AR_TABLE_signal=5;virtualpairUid=2022-09-0709:27:32T36fca6a2-39fb-4b16-a005-358f3ef7c941;alternate_reg=CMD_CREATE_INST_ADDR_AR_TABLE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | MAX_LATENCY_PIC |  |  | rw | ro |  | 0x0 |  |  | The number of picture delayed from reordering{has_reset=0} |  |  |  |  |  |

| 1.1.1.235.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_NUM_TICKS_POC_DIFF_ONE |  |  |  |  |  |  | 0x35C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 860 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_NUM_TICKS_POC_DIFF_ONE{alt_mode=CMD_ENC_PREFIX_SEI_INFO_signal=1;virtualpairUid=2022-09-0709:27:32Tb96a9ca9-6897-498c-9eb4-f12e9b1b6422;alternate_reg=CMD_ENC_PREFIX_SEI_INFO} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | NUM_TICKS_POC_DIFF_ONE |  |  | rw | ro |  | 0x0 |  |  | It specifies the number of clock ticks corresponding to a difference of picture order count values equal to 1.{has_reset=0} |  |  |  |  |  |

| 1.1.1.236.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE3 |  |  |  |  |  |  | 0x35C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 860 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index 3{alt_mode=CMD_ENC_PREFIX_SEI_INFO_signal=2;virtualpairUid=2022-09-0709:27:32Tb96a9ca9-6897-498c-9eb4-f12e9b1b6422;alternate_reg=CMD_ENC_PREFIX_SEI_INFO} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE3 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 3 Compressed Cb and Cr base address of DPB index 3{has_reset=0} |  |  |  |  |  |

| 1.1.1.238.0 |  |  |  |  |  |  |  | RET_DEC_DEBUG_INDEX |  |  |  |  |  |  | 0x35C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 860 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC and BWB frame buffer index for internal use{alt_mode=CMD_ENC_PREFIX_SEI_INFO_signal=3;virtualpairUid=2022-09-0709:27:32Tb96a9ca9-6897-498c-9eb4-f12e9b1b6422;alternate_reg=CMD_ENC_PREFIX_SEI_INFO} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 16:16 | DEC_AV1_INTRA_BC_FLAG |  |  | rw | ro |  | 0x0 |  |  | intra BC flag for AV1 Used for FBD model on VPU sim. env.{has_reset=0} |  |  |  |  |  |
| 15:8 | DEC_FBC_FB_INDEX |  |  | rw | ro |  | 0x0 |  |  | FBC frame buffer index{has_reset=0} |  |  |  |  |  |
| 7:0 | DEC_BWB_FB_INDEX |  |  | rw | ro |  | 0x0 |  |  | Linear frame buffer index{has_reset=0} |  |  |  |  |  |

| 1.1.1.239.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE3 |  |  |  |  |  |  | 0x360 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 864 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 3{alt_mode=CMD_ENC_SUFFIX_SEI_NAL_ADDR_signal=1;virtualpairUid=2022-09-0709:27:32T9262c75b-7010-417e-9ba5-4282b881ec88;alternate_reg=CMD_ENC_SUFFIX_SEI_NAL_ADDR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE3 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 3{has_reset=0} |  |  |  |  |  |

| 1.1.1.240.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET3 |  |  |  |  |  |  | 0x360 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 864 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 3{alt_mode=CMD_ENC_SUFFIX_SEI_NAL_ADDR_signal=2;virtualpairUid=2022-09-0709:27:32T9262c75b-7010-417e-9ba5-4282b881ec88;alternate_reg=CMD_ENC_SUFFIX_SEI_NAL_ADDR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE3 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB index 3 when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.241.0 |  |  |  |  |  |  |  | RET_DEC_DECODED_INDEX |  |  |  |  |  |  | 0x360 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 864 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Decoded picture index of DPB{alt_mode=CMD_ENC_SUFFIX_SEI_NAL_ADDR_signal=3;virtualpairUid=2022-09-0709:27:32T9262c75b-7010-417e-9ba5-4282b881ec88;alternate_reg=CMD_ENC_SUFFIX_SEI_NAL_ADDR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 4:0 | DEC_PIC_INDEX |  |  | rw | ro |  | 0x0 |  |  | Decoded Picture Index for FBC buffer{has_reset=0} |  |  |  |  |  |

| 1.1.1.243.0 |  |  |  |  |  |  |  | RET_ENC_HISTO_CNT_0 |  |  |  |  |  |  | 0x360 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 864 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| MV histogram count{alt_mode=CMD_ENC_SUFFIX_SEI_NAL_ADDR_signal=4;virtualpairUid=2022-09-0709:27:32T9262c75b-7010-417e-9ba5-4282b881ec88;alternate_reg=CMD_ENC_SUFFIX_SEI_NAL_ADDR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | HISTO_CNT_0 |  |  | rw | ro |  | 0x0 |  |  | The count number of MV histogram accumulated for class 0{has_reset=0} |  |  |  |  |  |

| 1.1.1.244.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET3 |  |  |  |  |  |  | 0x364 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 868 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index3{alt_mode=CMD_ENC_SUFFIX_SEI_INFO_signal=1;virtualpairUid=2022-09-0709:27:32T97daa0b5-21a7-400d-99a6-c3ba0833176a;alternate_reg=CMD_ENC_SUFFIX_SEI_INFO} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE3 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 3. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.245.0 |  |  |  |  |  |  |  | RET_DEC_DISPLAY_INDEX |  |  |  |  |  |  | 0x364 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 868 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Display picture index of DPB{alt_mode=CMD_ENC_SUFFIX_SEI_INFO_signal=2;virtualpairUid=2022-09-0709:27:32T97daa0b5-21a7-400d-99a6-c3ba0833176a;alternate_reg=CMD_ENC_SUFFIX_SEI_INFO} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 4:0 | DISPLAY_PIC_INDEX |  |  | rw | ro |  | 0x0 |  |  | Display Picture Index for FBC buffer (by reordering){has_reset=0} |  |  |  |  |  |

| 1.1.1.247.0 |  |  |  |  |  |  |  | RET_ENC_HISTO_CNT_1 |  |  |  |  |  |  | 0x364 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 868 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| MV histogram count{alt_mode=CMD_ENC_SUFFIX_SEI_INFO_signal=3;virtualpairUid=2022-09-0709:27:32T97daa0b5-21a7-400d-99a6-c3ba0833176a;alternate_reg=CMD_ENC_SUFFIX_SEI_INFO} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | HISTO_CNT_1 |  |  | rw | ro |  | 0x0 |  |  | The count number of MV histogram accumulated for class 1{has_reset=0} |  |  |  |  |  |

| 1.1.1.248.0 |  |  |  |  |  |  |  | RET_DEC_REALLOC_INDEX |  |  |  |  |  |  | 0x368 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 872 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Display picture index of DPB{alt_mode=CMD_SET_FB_ADDR_COL3_signal=1;virtualpairUid=2022-09-0709:27:32Ta5312b75-e4e6-431c-a8f5-df5f19ff91fd;alternate_reg=CMD_SET_FB_ADDR_COL3} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 4:0 | DISPLAY_PIC_INDEX |  |  | rw | ro |  | 0x0 |  |  | Display Picture Index for FBC buffer (by reordering){has_reset=0} |  |  |  |  |  |

| 1.1.1.250.0 |  |  |  |  |  |  |  | RET_ENC_HISTO_CNT_2 |  |  |  |  |  |  | 0x368 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 872 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| MV histogram count{alt_mode=CMD_SET_FB_ADDR_COL3_signal=2;virtualpairUid=2022-09-0709:27:32Ta5312b75-e4e6-431c-a8f5-df5f19ff91fd;alternate_reg=CMD_SET_FB_ADDR_COL3} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | HISTO_CNT_2 |  |  | rw | ro |  | 0x0 |  |  | The count number of MV histogram accumulated for class 2{has_reset=0} |  |  |  |  |  |

| 1.1.1.251.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE3 |  |  |  |  |  |  | 0x36C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 876 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 3{alt_mode=CMD_ENC_SEQ_BG_PARAM_signal=1;virtualpairUid=2022-09-0709:27:32T93327c5a-5874-4032-aa29-7b96dc4b6e24;alternate_reg=CMD_ENC_SEQ_BG_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SUB_SAMPLED_FB_BASE3 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frame 3{has_reset=0} |  |  |  |  |  |

| 1.1.1.252.0 |  |  |  |  |  |  |  | RET_DEC_DISP_IDC |  |  |  |  |  |  | 0x36C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 876 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| RET_DEC_DISP_IDC{alt_mode=CMD_ENC_SEQ_BG_PARAM_signal=2;virtualpairUid=2022-09-0709:27:32T93327c5a-5874-4032-aa29-7b96dc4b6e24;alternate_reg=CMD_ENC_SEQ_BG_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DISPLAY_FLAG |  |  | rw | ro |  | 0x0 |  |  | Display frame buffer flag.{has_reset=0} |  |  |  |  |  |

| 1.1.1.254.0 |  |  |  |  |  |  |  | RET_ENC_HISTO_CNT_3 |  |  |  |  |  |  | 0x36C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 876 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| MV histogram count{alt_mode=CMD_ENC_SEQ_BG_PARAM_signal=3;virtualpairUid=2022-09-0709:27:32T93327c5a-5874-4032-aa29-7b96dc4b6e24;alternate_reg=CMD_ENC_SEQ_BG_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | HISTO_CNT_3 |  |  | rw | ro |  | 0x0 |  |  | The count number of MV histogram accumulated for class 3{has_reset=0} |  |  |  |  |  |

| 1.1.1.255.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE4 |  |  |  |  |  |  | 0x370 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 880 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 4{alt_mode=CMD_ENC_SEQ_NON_VCL_PARAM_signal=1;virtualpairUid=2022-09-0709:27:32Tf64295e9-0696-44ae-812b-68fab558ac08;alternate_reg=CMD_ENC_SEQ_NON_VCL_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE4 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 4 Compressed Luma base address of DPB index 4{has_reset=0} |  |  |  |  |  |

| 1.1.1.256.0 |  |  |  |  |  |  |  | RET_DEC_NUM_ERR_CTB |  |  |  |  |  |  | 0x370 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 880 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Number of error CTU{alt_mode=CMD_ENC_SEQ_NON_VCL_PARAM_signal=2;virtualpairUid=2022-09-0709:27:32Tf64295e9-0696-44ae-812b-68fab558ac08;alternate_reg=CMD_ENC_SEQ_NON_VCL_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | ERROR_CTU_NUMBER |  |  | rw | ro |  | 0x0 |  |  | H.265 : error number of CTU VP9 : error number of SB SVAC : error number of CTU AVS2 : error number of LCU Otherwise : error number of macroblock{has_reset=0} |  |  |  |  |  |

| 1.1.1.258.0 |  |  |  |  |  |  |  | RET_ENC_HISTO_CNT_4 |  |  |  |  |  |  | 0x370 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 880 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| MV histogram count{alt_mode=CMD_ENC_SEQ_NON_VCL_PARAM_signal=3;virtualpairUid=2022-09-0709:27:32Tf64295e9-0696-44ae-812b-68fab558ac08;alternate_reg=CMD_ENC_SEQ_NON_VCL_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | HISTO_CNT_4 |  |  | rw | ro |  | 0x0 |  |  | The count number of MV histogram accumulated for class 4{has_reset=0} |  |  |  |  |  |

| 1.1.1.259.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_VUI_RBSP_ADDR |  |  |  |  |  |  | 0x374 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 884 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_VUI_RBSP_ADDR{alt_mode=CMD_ENC_CSC_COEFF_0_signal=1;virtualpairUid=2022-09-0709:27:32T4a4fcd77-14e7-4db5-baa2-b8cf0e96ed75;alternate_reg=CMD_ENC_CSC_COEFF_0} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | VUI_RBSP_ADDR |  |  | rw | ro |  | 0x0 |  |  | It specifies the address of VUI RBSP buffer. (Little endian){has_reset=0} |  |  |  |  |  |

| 1.1.1.260.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE4 |  |  |  |  |  |  | 0x374 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 884 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index 4{alt_mode=CMD_ENC_CSC_COEFF_0_signal=2;virtualpairUid=2022-09-0709:27:32T4a4fcd77-14e7-4db5-baa2-b8cf0e96ed75;alternate_reg=CMD_ENC_CSC_COEFF_0} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE4 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 4 Compressed Cb and Cr base address of DPB index 4{has_reset=0} |  |  |  |  |  |

| 1.1.1.262.0 |  |  |  |  |  |  |  | RET_ENC_NUM_TILE_COL |  |  |  |  |  |  | 0x374 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 884 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Number of tile columns{alt_mode=CMD_ENC_CSC_COEFF_0_signal=3;virtualpairUid=2022-09-0709:27:32T4a4fcd77-14e7-4db5-baa2-b8cf0e96ed75;alternate_reg=CMD_ENC_CSC_COEFF_0} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | NUMBER_OF_TILE_COLUMNS |  |  | rw | ro |  | 0x0 |  |  | The number of tile columns considering tile constraints (both AV1 spec. and hardware) and rotation. (In case of 90 degree rotation, the number of tiles input from CMD_ENC_SEQ_TILE_PARAM need to be adjusted to consider the rotated size. the host can identify the actual number of tiles from this register.){has_reset=0} |  |  |  |  |  |

| 1.1.1.263.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_HRD_RBSP_ADDR |  |  |  |  |  |  | 0x378 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 888 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_HRD_RBSP_ADDR{alt_mode=CMD_ENC_CSC_COEFF_1_signal=1;virtualpairUid=2022-09-0709:27:32T2c095cb0-c07c-4846-947f-160ee65aa5df;alternate_reg=CMD_ENC_CSC_COEFF_1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | HRD_RBSP_ADDR |  |  | rw | ro |  | 0x0 |  |  | It specifies the address of HRD RBSP buffer. (Little endian){has_reset=0} |  |  |  |  |  |

| 1.1.1.264.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE4 |  |  |  |  |  |  | 0x378 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 888 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 4{alt_mode=CMD_ENC_CSC_COEFF_1_signal=2;virtualpairUid=2022-09-0709:27:32T2c095cb0-c07c-4846-947f-160ee65aa5df;alternate_reg=CMD_ENC_CSC_COEFF_1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE4 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 4{has_reset=0} |  |  |  |  |  |

| 1.1.1.265.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET4 |  |  |  |  |  |  | 0x378 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 888 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 4{alt_mode=CMD_ENC_CSC_COEFF_1_signal=3;virtualpairUid=2022-09-0709:27:32T2c095cb0-c07c-4846-947f-160ee65aa5df;alternate_reg=CMD_ENC_CSC_COEFF_1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE4 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB index 4 when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.267.0 |  |  |  |  |  |  |  | RET_ENC_NUM_TILE_ROW |  |  |  |  |  |  | 0x378 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 888 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Number of tile rows{alt_mode=CMD_ENC_CSC_COEFF_1_signal=4;virtualpairUid=2022-09-0709:27:32T2c095cb0-c07c-4846-947f-160ee65aa5df;alternate_reg=CMD_ENC_CSC_COEFF_1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | NUMBER_OF_TILE_ROWS |  |  | rw | ro |  | 0x0 |  |  | The number of tile rows considering tile constraints (both AV1 spec. and hardware) and rotation.(In case of 90 degree rotation, the number of tiles input from CMD_ENC_SEQ_TILE_PARAM need to be adjusted to consider the rotated size. the host can identify the actual number of tiles from this register.){has_reset=0} |  |  |  |  |  |

| 1.1.1.268.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET4 |  |  |  |  |  |  | 0x37C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 892 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index4{alt_mode=CMD_ENC_CSC_COEFF_2_signal=1;virtualpairUid=2022-09-0709:27:32T56c98a85-4560-481d-817d-28e3cf094f89;alternate_reg=CMD_ENC_CSC_COEFF_2} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE4 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 4. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.270.0 |  |  |  |  |  |  |  | RET_ENC_PIC_WIDTH |  |  |  |  |  |  | 0x37C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 892 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Encoded picture width{alt_mode=CMD_ENC_CSC_COEFF_2_signal=2;virtualpairUid=2022-09-0709:27:32T56c98a85-4560-481d-817d-28e3cf094f89;alternate_reg=CMD_ENC_CSC_COEFF_2} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | ENCODED_PICTURE_WIDTH |  |  | rw | ro |  | 0x0 |  |  | Encoded picture width considering rotation and padding by mirror and standard constraints. (For example, in case of AVC ENC, both picture width and height should be aligned with 16 pixels. So, the host can identiy the actual encoded picture size from this register.){has_reset=0} |  |  |  |  |  |

| 1.1.1.271.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_QROUND_OFFSET |  |  |  |  |  |  | 0x380 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 896 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_QROUND_OFFSET{alt_mode=CMD_ENC_CSC_COEFF_3_signal=1;virtualpairUid=2022-09-0709:27:32T571dcddb-3bfc-4a2f-8025-676627200403;alternate_reg=CMD_ENC_CSC_COEFF_3} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:24 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 23:13 | CUSTOM_QROUND_OFFSET_INTER |  |  | rw | ro |  | 0x0 |  |  | It specifies a customized quantization rounding offset for inter pictures. (0 ~ 255){has_reset=0} |  |  |  |  |  |
| 12:2 | CUSTOM_QROUND_OFFSET_INTRA |  |  | rw | ro |  | 0x0 |  |  | It specifies a customized quantization rounding offset for intra pictures(0 ~ 255){has_reset=0} |  |  |  |  |  |
| 1:0 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |

| 1.1.1.272.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL4 |  |  |  |  |  |  | 0x380 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 896 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index 4{alt_mode=CMD_ENC_CSC_COEFF_3_signal=2;virtualpairUid=2022-09-0709:27:32T571dcddb-3bfc-4a2f-8025-676627200403;alternate_reg=CMD_ENC_CSC_COEFF_3} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE4 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.274.0 |  |  |  |  |  |  |  | ENC_PIC_HEIGHT |  |  |  |  |  |  | 0x380 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 896 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Encoded picture height{alt_mode=CMD_ENC_CSC_COEFF_3_signal=3;virtualpairUid=2022-09-0709:27:32T571dcddb-3bfc-4a2f-8025-676627200403;alternate_reg=CMD_ENC_CSC_COEFF_3} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | ENCODED_PICTURE_HEIGHT |  |  | rw | ro |  | 0x0 |  |  | Encoded picture height considering rotation and padding by mirror and standard constraints. (For example, in case of AVC ENC, both picture width and height should be aligned with 16 pixels. So, the host can identiy the actual encoded picture size from this register.){has_reset=0} |  |  |  |  |  |

| 1.1.1.276.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE4 |  |  |  |  |  |  | 0x384 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 900 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 4{alt_mode=CMD_ENC_SEQ_QUANT_PARAM_0_signal=1;virtualpairUid=2022-09-0709:27:32Td61867a9-5387-48c3-a20c-5b69b5cabd1a;alternate_reg=CMD_ENC_SEQ_QUANT_PARAM_0} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SUB_SAMPLED_FB_BASE4 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frame 4{has_reset=0} |  |  |  |  |  |

| 1.1.1.278.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE5 |  |  |  |  |  |  | 0x388 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 904 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 5{alt_mode=CMD_ENC_SEQ_QUANT_PARAM_1_signal=1;virtualpairUid=2022-09-0709:27:32Tf1eb521d-8068-43d7-a3f5-b52be4a5ee7b;alternate_reg=CMD_ENC_SEQ_QUANT_PARAM_1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE5 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 5 Compressed Luma base address of DPB index 5{has_reset=0} |  |  |  |  |  |

| 1.1.1.280.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE5 |  |  |  |  |  |  | 0x38C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 908 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index 5{alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PARAM_signal=1;virtualpairUid=2022-09-0709:27:32Tbf081025-f0a8-42c7-8027-c2b1ab839db4;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE5 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 5 Compressed Cb and Cr base address of DPB index 5{has_reset=0} |  |  |  |  |  |

| 1.1.1.281.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE5 |  |  |  |  |  |  | 0x390 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 912 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 5{alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_0_signal=1;virtualpairUid=2022-09-0709:27:32T2b814b5e-12f6-4131-82b2-f42cd8b451a4;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_0} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE5 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 5{has_reset=0} |  |  |  |  |  |

| 1.1.1.283.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET5 |  |  |  |  |  |  | 0x390 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 912 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 5{alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_0_signal=2;virtualpairUid=2022-09-0709:27:32T2b814b5e-12f6-4131-82b2-f42cd8b451a4;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_0} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE5 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB index 5 when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.284.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET5 |  |  |  |  |  |  | 0x394 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 916 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index 5{alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_1_signal=1;virtualpairUid=2022-09-0709:27:32Teb77e763-fc47-434a-b2e8-d0bd11504ae9;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE5 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 5. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.286.0 |  |  |  |  |  |  |  | RET_DEC_NUM_REQURED_COL_FB |  |  |  |  |  |  | 0x394 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 916 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Required Number of Minimum Col FB {alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_1_signal=2;virtualpairUid=2022-09-0709:27:32Teb77e763-fc47-434a-b2e8-d0bd11504ae9;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 4:0 | MIN_COL_DPB_NUM |  |  | rw | ro |  | 0x0 |  |  | Required number of frame buffers for decoding sequence {has_reset=0} |  |  |  |  |  |

| 1.1.1.288.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL5 |  |  |  |  |  |  | 0x398 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 920 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index5{alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_2_signal=1;virtualpairUid=2022-09-0709:27:32Tea98b471-70f1-4681-af54-11d5f181bdee;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_2} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE5 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.290.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE5 |  |  |  |  |  |  | 0x39C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 924 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 5{alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_3_signal=1;virtualpairUid=2022-09-0709:27:32Tca0b004e-8982-41fe-a4c6-0878e69809e3;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_3} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SUB_SAMPLED_FB_BASE5 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frame 5{has_reset=0} |  |  |  |  |  |

| 1.1.1.291.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE6 |  |  |  |  |  |  | 0x3A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 928 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 6{alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_4_signal=1;virtualpairUid=2022-09-0709:27:32T3bc4aa26-bc06-4c40-8c58-616390cd8a31;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_4} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE6 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 6 Compressed Luma base address of DPB index 6{has_reset=0} |  |  |  |  |  |

| 1.1.1.293.0 |  |  |  |  |  |  |  | RET_CORE_IDC |  |  |  |  |  |  | 0x3A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 928 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| RET_CORE_IDC{alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_4_signal=2;virtualpairUid=2022-09-0709:27:32T3bc4aa26-bc06-4c40-8c58-616390cd8a31;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_4} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 15:12 | STAGE3_CORE_IDC |  |  | rw | ro |  | 0x0 |  |  | Allocated core idc for stage #3 (package-encoding){has_reset=0} |  |  |  |  |  |
| 11:8 | STAGE2_CORE_IDC |  |  | rw | ro |  | 0x0 |  |  | Allocated core idc for stage #2 (vcore){has_reset=0} |  |  |  |  |  |
| 7:4 | STAGE1_CORE_IDC |  |  | rw | ro |  | 0x0 |  |  | Allocated core idc for stage #1 (prescan-decoding){has_reset=0} |  |  |  |  |  |
| 3:0 | STAGE0_CORE_IDC |  |  | rw | ro |  | 0x0 |  |  | Allocated core idc for stage #0 (seek/prepare){has_reset=0} |  |  |  |  |  |

| 1.1.1.294.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE6 |  |  |  |  |  |  | 0x3A4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 932 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index 6{alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_5_signal=1;virtualpairUid=2022-09-0709:27:32T49c312b9-cbdc-4bf2-9b9e-3a7c259357c2;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_5} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE6 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 6 Compressed Cb and Cr base address of DPB index 6{has_reset=0} |  |  |  |  |  |

| 1.1.1.296.0 |  |  |  |  |  |  |  | RET_DEC_PIC_PARAM |  |  |  |  |  |  | 0x3A4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 932 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Bitstream sequence/picture parameter information{alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_5_signal=2;virtualpairUid=2022-09-0709:27:32T49c312b9-cbdc-4bf2-9b9e-3a7c259357c2;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_5} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:2 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 1:1 | ENABLE_SCREEN_CONT_TOOLS |  |  | rw | ro |  | 0x0 |  |  | AV1: screen contents tools enable{has_reset=0} |  |  |  |  |  |
| 0:0 | ENABLE_INTRABC |  |  | rw | ro |  | 0x0 |  |  | AV1: intrabc tool enable{has_reset=0} |  |  |  |  |  |

| 1.1.1.297.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE6 |  |  |  |  |  |  | 0x3A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 936 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 6{alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_6_signal=1;virtualpairUid=2022-09-0709:27:32T5d80a5d3-1a3b-47c1-99a2-8d18f5271e82;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_6} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE6 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 6{has_reset=0} |  |  |  |  |  |

| 1.1.1.298.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET6 |  |  |  |  |  |  | 0x3A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 936 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 6{alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_6_signal=2;virtualpairUid=2022-09-0709:27:32T5d80a5d3-1a3b-47c1-99a2-8d18f5271e82;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_6} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE6 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB index 6 when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.300.0 |  |  |  |  |  |  |  | RET_ENC_CORE_IDC |  |  |  |  |  |  | 0x3A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 936 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Used core id{alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_6_signal=3;virtualpairUid=2022-09-0709:27:32T5d80a5d3-1a3b-47c1-99a2-8d18f5271e82;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_6} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:4 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 3:0 | ENC_CORE_IDC |  |  | rw | ro |  | 0x0 |  |  | 0x0 : HW not used 0x1 : Core 0 0x2 : Core 1{has_reset=0} |  |  |  |  |  |

| 1.1.1.301.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET6 |  |  |  |  |  |  | 0x3AC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 940 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index6{alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_7_signal=1;virtualpairUid=2022-09-0709:27:32Tb195cde3-469e-4e9d-8393-4049759dd807;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_7} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE6 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 6. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.303.0 |  |  |  |  |  |  |  | RET_HOST_CMD_WARN_INFO |  |  |  |  |  |  | 0x3AC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 940 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Warning Info{alt_mode=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_7_signal=2;virtualpairUid=2022-09-0709:27:32Tb195cde3-469e-4e9d-8393-4049759dd807;alternate_reg=CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_7} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | ret_host_cmd_warn_info |  |  | rw | ro |  | 0x0 |  |  | ret_host_cmd_warn_info{has_reset=0} |  |  |  |  |  |

| 1.1.1.305.0 |  |  |  |  |  |  |  | RET_HOST_CMD_ERR_INFO |  |  |  |  |  |  | 0x3B0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 944 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Error Info{alt_mode=CMD_SET_FB_ADDR_COL6_signal=1;virtualpairUid=2022-09-0709:27:32Tfe313fbe-fbb7-4c8d-b582-265b423d0846;alternate_reg=CMD_SET_FB_ADDR_COL6} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | ret_host_cmd_err_info |  |  | rw | ro |  | 0x0 |  |  | ret_host_cmd_err_info{has_reset=0} |  |  |  |  |  |

| 1.1.1.307.0 |  |  |  |  |  |  |  | RET_HOST_CMD_SUCCESS |  |  |  |  |  |  | 0x3B4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 948 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Host command Reture Value{alt_mode=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE6_signal=1;virtualpairUid=2022-09-0709:27:32T02a09861-a1e4-4112-b489-2444406c2ac4;alternate_reg=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE6} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | ret_host_cmd_success |  |  | rw | ro |  | 0x0 |  |  | ret_host_cmd_success{has_reset=0} |  |  |  |  |  |

| 1.1.1.309.0 |  |  |  |  |  |  |  | RET_ENC_SUM_ME0_X_DIR_LOWER |  |  |  |  |  |  | 0x3B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 952 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| mv x sum lower{alt_mode=CMD_SET_FB_ADDR_LUMA_BASE7_signal=1;virtualpairUid=2022-09-0709:27:32Tc0be92c1-8a02-43a7-939b-e429b687379a;alternate_reg=CMD_SET_FB_ADDR_LUMA_BASE7} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | L0_MV_X_SUIM_LOWER |  |  | rw | ro |  | 0x0 |  |  | L0 FME 32x32 mv_x sum lower{has_reset=0} |  |  |  |  |  |

| 1.1.1.311.0 |  |  |  |  |  |  |  | RET_ENC_SUM_ME0_X_DIR_HIGHER |  |  |  |  |  |  | 0x3BC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 956 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| mv x sum higher{alt_mode=CMD_SET_FB_ADDR_CB_BASE7_signal=1;virtualpairUid=2022-09-0709:27:32T28ee4502-bbf8-412a-a375-8f11be486f94;alternate_reg=CMD_SET_FB_ADDR_CB_BASE7} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | L0_MV_X_SUIM_HIGHER |  |  | rw | ro |  | 0x0 |  |  | L0 FME 32x32 mv_x sum higher{has_reset=0} |  |  |  |  |  |

| 1.1.1.312.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET7 |  |  |  |  |  |  | 0x3C0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 960 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 7{alt_mode=CMD_SET_FB_ADDR_CR_BASE7_signal=1;virtualpairUid=2022-09-0709:27:32T42dd8c3f-a630-4581-8b36-c772020616eb;alternate_reg=CMD_SET_FB_ADDR_CR_BASE7} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE7 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB index 7 when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.314.0 |  |  |  |  |  |  |  | RET_ENC_SUM_ME0_Y_DIR_LOWER |  |  |  |  |  |  | 0x3C0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 960 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| mv y sum lower{alt_mode=CMD_SET_FB_ADDR_CR_BASE7_signal=2;virtualpairUid=2022-09-0709:27:32T42dd8c3f-a630-4581-8b36-c772020616eb;alternate_reg=CMD_SET_FB_ADDR_CR_BASE7} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | L0_MV_Y_SUIM_LOWER |  |  | rw | ro |  | 0x0 |  |  | L0 FME 32x32 mv_y sum lower{has_reset=0} |  |  |  |  |  |

| 1.1.1.316.0 |  |  |  |  |  |  |  | RET_ENC_SUM_ME0_Y_DIR_HIGHER |  |  |  |  |  |  | 0x3C4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 964 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| mv y sum higher{alt_mode=CMD_SET_FB_ADDR_FBC_C_OFFSET7_signal=1;virtualpairUid=2022-09-0709:27:32Te1263344-d0d7-4e61-b4a7-be63dec0fe16;alternate_reg=CMD_SET_FB_ADDR_FBC_C_OFFSET7} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | L0_MV_Y_SUIM_HIGHER |  |  | rw | ro |  | 0x0 |  |  | L0 FME 32x32 mv_y sum higher{has_reset=0} |  |  |  |  |  |

| 1.1.1.318.0 |  |  |  |  |  |  |  | RET_ENC_SUM_ME1_X_DIR_LOWER |  |  |  |  |  |  | 0x3C8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 968 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| mv x sum lower{alt_mode=CMD_SET_FB_ADDR_COL7_signal=1;virtualpairUid=2022-09-0709:27:32T2ff3cdd7-515f-4c23-8dc7-5683463332d1;alternate_reg=CMD_SET_FB_ADDR_COL7} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | L1_MV_X_SUIM_LOWER |  |  | rw | ro |  | 0x0 |  |  | L1 FME 32x32 mv_x sum lower{has_reset=0} |  |  |  |  |  |

| 1.1.1.319.0 |  |  |  |  |  |  |  | RET_DEC_WARN_INFO |  |  |  |  |  |  | 0x3CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 972 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Warning Information {alt_mode=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE7_signal=1;virtualpairUid=2022-09-0709:27:32T52484437-14f9-4dae-acc7-7d9e0fe981dc;alternate_reg=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE7} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DEC_WARN_INFO |  |  | rw | ro |  | 0x0 |  |  | Please refer to <<wave6_error_info>> defining warnings that VPU might have.{has_reset=0} |  |  |  |  |  |

| 1.1.1.321.0 |  |  |  |  |  |  |  | RET_ENC_SUM_ME1_X_DIR_HIGHER |  |  |  |  |  |  | 0x3CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 972 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| mv x sum higher{alt_mode=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE7_signal=2;virtualpairUid=2022-09-0709:27:32T52484437-14f9-4dae-acc7-7d9e0fe981dc;alternate_reg=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE7} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | L1_MV_X_SUIM_HIGHER |  |  | rw | ro |  | 0x0 |  |  | L1 FME 32x32 mv_x sum higher{has_reset=0} |  |  |  |  |  |

| 1.1.1.322.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE8 |  |  |  |  |  |  | 0x3D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 976 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 8{alt_mode=CMD_ENC_SEQ_TILE_PARAM_signal=1;virtualpairUid=2022-09-0709:27:32T1360298c-6df3-414b-9155-fba07f445ad4;alternate_reg=CMD_ENC_SEQ_TILE_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE8 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 8 Compressed Luma base address of DPB index 8{has_reset=0} |  |  |  |  |  |

| 1.1.1.323.0 |  |  |  |  |  |  |  | RET_DEC_ERR_INFO |  |  |  |  |  |  | 0x3D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 976 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Error Information {alt_mode=CMD_ENC_SEQ_TILE_PARAM_signal=2;virtualpairUid=2022-09-0709:27:32T1360298c-6df3-414b-9155-fba07f445ad4;alternate_reg=CMD_ENC_SEQ_TILE_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | ERROR_INFO |  |  | rw | ro |  | 0x0 |  |  | Please refer to <<wave6_error_info>> defining errors that VPU might have.{has_reset=0} |  |  |  |  |  |

| 1.1.1.325.0 |  |  |  |  |  |  |  | RET_ENC_SUM_ME1_Y_DIR_LOWER |  |  |  |  |  |  | 0x3D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 976 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| mv y sum lower{alt_mode=CMD_ENC_SEQ_TILE_PARAM_signal=3;virtualpairUid=2022-09-0709:27:32T1360298c-6df3-414b-9155-fba07f445ad4;alternate_reg=CMD_ENC_SEQ_TILE_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | L1_MV_Y_SUIM_LOWER |  |  | rw | ro |  | 0x0 |  |  | L1 FME 32x32 mv_y sum lower{has_reset=0} |  |  |  |  |  |

| 1.1.1.326.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE8 |  |  |  |  |  |  | 0x3D4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 980 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index 8{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_0_signal=1;virtualpairUid=2022-09-0709:27:32Td46888d7-6650-456d-8787-a10863e9e32d;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_0} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE8 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 8 Compressed Cb and Cr base address of DPB index 8{has_reset=0} |  |  |  |  |  |

| 1.1.1.327.0 |  |  |  |  |  |  |  | RET_DEC_DECODING_SUCCESS |  |  |  |  |  |  | 0x3D4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 980 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| RET_DEC_DECODING_SUCCESS{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_0_signal=2;virtualpairUid=2022-09-0709:27:32Td46888d7-6650-456d-8787-a10863e9e32d;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_0} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 1:0 | QUERY_DEC_SUCCESS |  |  | rw | ro |  | 0x0 |  |  | It indicates that decoding result for enqueued decode command(INIT_SEQ or DEC_PIC) 00: FAIL If the value is not "zero", it means "SUCCESS" 01: SUCCESS 10: SUCCESS_WITH_WARNING (success but there exist some warning) If it returns FAIL, please refer to RET_QUERY_DEC_ERR_INFO. If it returns SUCCESS_WITH_WARNING, please refer to RET_QUERY_DEC_WARN_INFO{has_reset=0} |  |  |  |  |  |

| 1.1.1.329.0 |  |  |  |  |  |  |  | RET_ENC_SUM_ME1_Y_DIR_HIGHER |  |  |  |  |  |  | 0x3D4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 980 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| mv y sum higher{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_0_signal=3;virtualpairUid=2022-09-0709:27:32Td46888d7-6650-456d-8787-a10863e9e32d;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_0} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | L1_MV_Y_SUIM_HIGHER |  |  | rw | ro |  | 0x0 |  |  | L1 FME 32x32 mv_y sum higher{has_reset=0} |  |  |  |  |  |

| 1.1.1.330.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE8 |  |  |  |  |  |  | 0x3D8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 984 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 8{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_1_signal=1;virtualpairUid=2022-09-0709:27:32Td86760b6-c0dc-4f1f-aefc-7038b01b76b1;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE8 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 8{has_reset=0} |  |  |  |  |  |

| 1.1.1.332.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET8 |  |  |  |  |  |  | 0x3D8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 984 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 8{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_1_signal=2;virtualpairUid=2022-09-0709:27:32Td86760b6-c0dc-4f1f-aefc-7038b01b76b1;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE8 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB index 8 when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.334.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET8 |  |  |  |  |  |  | 0x3DC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 988 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index 8{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_2_signal=1;virtualpairUid=2022-09-0709:27:32T7f98f243-8e2e-4e3b-9990-86eb3e75b33a;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_2} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE8 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 8. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.336.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL8 |  |  |  |  |  |  | 0x3E0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 992 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index 8{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_3_signal=1;virtualpairUid=2022-09-0709:27:32T11538bb0-2fe5-4eb3-b027-4cc5140cea15;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_3} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE8 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.338.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE8 |  |  |  |  |  |  | 0x3E4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 996 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 8{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_4_signal=1;virtualpairUid=2022-09-0709:27:32T40f79e2d-57ec-4b30-ba4e-cce3ba4bbd4d;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_4} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SUB_SAMPLED_FB_BASE8 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frame 8{has_reset=0} |  |  |  |  |  |

| 1.1.1.340.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE9 |  |  |  |  |  |  | 0x3E8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1000 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 9{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_5_signal=1;virtualpairUid=2022-09-0709:27:32T17a8747d-0285-45dd-adf6-7acc49d8e24d;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_5} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE9 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 9 Compressed Luma base address of DPB index 9{has_reset=0} |  |  |  |  |  |

| 1.1.1.342.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE9 |  |  |  |  |  |  | 0x3EC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1004 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index 9{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_6_signal=1;virtualpairUid=2022-09-0709:27:32Tdb096d4d-3be3-4b6d-be07-946b70b7fdba;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_6} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE9 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 9 Compressed Cb and Cr base address of DPB index 9{has_reset=0} |  |  |  |  |  |

| 1.1.1.343.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE9 |  |  |  |  |  |  | 0x3F0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1008 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 9{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_7_signal=1;virtualpairUid=2022-09-0709:27:32T047fa116-47bf-4b86-a2de-c192c60f2b4c;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_7} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE9 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 9{has_reset=0} |  |  |  |  |  |

| 1.1.1.345.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET9 |  |  |  |  |  |  | 0x3F0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1008 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 9{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_7_signal=2;virtualpairUid=2022-09-0709:27:32T047fa116-47bf-4b86-a2de-c192c60f2b4c;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_7} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE9 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB index 9 when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.347.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET9 |  |  |  |  |  |  | 0x3F4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1012 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index 9{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_8_signal=1;virtualpairUid=2022-09-0709:27:32Tf9c54489-5ed4-4c4b-a906-db3835ab0db2;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_8} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE9 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 9. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.348.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL9 |  |  |  |  |  |  | 0x3F8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1016 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index 9{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_9_signal=1;virtualpairUid=2022-09-0709:27:32Td4fcad5b-2f06-423a-9745-3760a5a27909;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_9} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE9 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.350.0 |  |  |  |  |  |  |  | RET_ENC_SRC_DEBUG_0 |  |  |  |  |  |  | 0x3F8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1016 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| ret_enc_src_debug_0{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_9_signal=2;virtualpairUid=2022-09-0709:27:32Td4fcad5b-2f06-423a-9745-3760a5a27909;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_9} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SFS_DEBUG_INFO |  |  | rw | ro |  | 0x0 |  |  | SFS debugging info - will be removed{has_reset=0} |  |  |  |  |  |

| 1.1.1.351.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE9 |  |  |  |  |  |  | 0x3FC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1020 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 9{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_10_signal=1;virtualpairUid=2022-09-0709:27:32T9a1cbb1e-be5c-44c1-a571-7de4cda9e2d4;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_10} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SUB_SAMPLED_FB_BASE9 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frame 9{has_reset=0} |  |  |  |  |  |

| 1.1.1.353.0 |  |  |  |  |  |  |  | RET_ENC_SRC_DEBUG_1 |  |  |  |  |  |  | 0x3FC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1020 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| ret_enc_src_debug_1{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_10_signal=2;virtualpairUid=2022-09-0709:27:32T9a1cbb1e-be5c-44c1-a571-7de4cda9e2d4;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_10} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SFS_DEBUG_INFO |  |  | rw | ro |  | 0x0 |  |  | SFS debugging info - will be removed{has_reset=0} |  |  |  |  |  |

| 1.1.1.355.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE10 |  |  |  |  |  |  | 0x400 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1024 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 10{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_11_signal=1;virtualpairUid=2022-09-0709:27:32T10f2c046-67ea-4d24-9c78-8bdd34539237;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_11} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE10 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 10 Compressed Luma base address of DPB index 10{has_reset=0} |  |  |  |  |  |

| 1.1.1.357.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE10 |  |  |  |  |  |  | 0x404 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1028 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index 10{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_12_signal=1;virtualpairUid=2022-09-0709:27:32T90d70f0c-260d-4cb3-b1b2-426e79bb0d87;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_12} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE10 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 10 Compressed Cb and Cr base address of DPB index 10{has_reset=0} |  |  |  |  |  |

| 1.1.1.358.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE10 |  |  |  |  |  |  | 0x408 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1032 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 10{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_13_signal=1;virtualpairUid=2022-09-0709:27:32T12fe2b7b-ff36-48f4-86c1-a87fa5de5654;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_13} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE10 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 10{has_reset=0} |  |  |  |  |  |

| 1.1.1.360.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET10 |  |  |  |  |  |  | 0x408 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1032 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 10{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_13_signal=2;virtualpairUid=2022-09-0709:27:32T12fe2b7b-ff36-48f4-86c1-a87fa5de5654;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_13} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE10 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB index 10 when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.362.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET10 |  |  |  |  |  |  | 0x40C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1036 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index 10{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_14_signal=1;virtualpairUid=2022-09-0709:27:32Td28f0dd7-eb0d-4178-925c-990c0a6f5fcb;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_14} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE10 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 10. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.364.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL10 |  |  |  |  |  |  | 0x410 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1040 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index 10{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_15_signal=1;virtualpairUid=2022-09-0709:27:32T87b02600-3ef8-43e4-ba03-f0c4790825eb;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_15} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE10 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.366.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE10 |  |  |  |  |  |  | 0x414 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1044 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 10{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_16_signal=1;virtualpairUid=2022-09-0709:27:32T769c4fc8-23ff-4381-9642-8fcf771bb334;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_16} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE10 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frame 10{has_reset=0} |  |  |  |  |  |

| 1.1.1.368.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE11 |  |  |  |  |  |  | 0x418 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1048 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 11{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_17_signal=1;virtualpairUid=2022-09-0709:27:32T09dfcbcf-7f7b-4c7a-9def-5bdce41c5daf;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_17} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE11 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 11 Compressed Luma base address of DPB index 11{has_reset=0} |  |  |  |  |  |

| 1.1.1.370.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE11 |  |  |  |  |  |  | 0x41C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1052 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index 11{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_18_signal=1;virtualpairUid=2022-09-0709:27:32T2c52bbb2-3f0f-4c14-89bb-0ca202764cc9;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_18} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE11 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 11 Compressed Cb and Cr base address of DPB index 11{has_reset=0} |  |  |  |  |  |

| 1.1.1.371.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE11 |  |  |  |  |  |  | 0x420 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1056 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 11{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_19_signal=1;virtualpairUid=2022-09-0709:27:32Tbc968102-0e96-48e5-9104-32bcf0ef806d;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_19} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE11 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 11{has_reset=0} |  |  |  |  |  |

| 1.1.1.373.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET11 |  |  |  |  |  |  | 0x420 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1056 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 11{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_19_signal=2;virtualpairUid=2022-09-0709:27:32Tbc968102-0e96-48e5-9104-32bcf0ef806d;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_19} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE11 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB 11 index when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.375.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET11 |  |  |  |  |  |  | 0x424 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1060 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index 11{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_20_signal=1;virtualpairUid=2022-09-0709:27:32T86eeee10-b7f7-44f6-940c-8f5cda97760e;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_20} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE11 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 11. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.377.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL11 |  |  |  |  |  |  | 0x428 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1064 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index 11{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_21_signal=1;virtualpairUid=2022-09-0709:27:32T0090317e-67e6-4cfb-a923-9cf4e9946280;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_21} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE11 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.379.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE11 |  |  |  |  |  |  | 0x42C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1068 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 11{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_22_signal=1;virtualpairUid=2022-09-0709:27:32T0e6243ad-6259-48b7-ae99-1c8547af8ac8;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_22} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE11 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frame 11{has_reset=0} |  |  |  |  |  |

| 1.1.1.381.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE12 |  |  |  |  |  |  | 0x430 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1072 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 12{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_23_signal=1;virtualpairUid=2022-09-0709:27:32T7921316b-dd7e-4994-8406-27b62aa19cc7;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_23} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE12 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 12 Compressed Luma base address of DPB index 12{has_reset=0} |  |  |  |  |  |

| 1.1.1.383.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE12 |  |  |  |  |  |  | 0x434 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1076 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index 12{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_24_signal=1;virtualpairUid=2022-09-0709:27:32T066bab4d-aec7-4629-8032-d7df6e3301fe;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_24} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE12 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 12 Compressed Cb and Cr base address of DPB index 12{has_reset=0} |  |  |  |  |  |

| 1.1.1.384.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE12 |  |  |  |  |  |  | 0x438 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1080 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 12{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_25_signal=1;virtualpairUid=2022-09-0709:27:32Tab553a80-0cf7-4457-b3fb-de6b34b5d9be;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_25} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE12 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 12{has_reset=0} |  |  |  |  |  |

| 1.1.1.386.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET12 |  |  |  |  |  |  | 0x438 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1080 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 12{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_25_signal=2;virtualpairUid=2022-09-0709:27:32Tab553a80-0cf7-4457-b3fb-de6b34b5d9be;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_25} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE12 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB index 12 when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.388.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET12 |  |  |  |  |  |  | 0x43C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1084 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index 12{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_26_signal=1;virtualpairUid=2022-09-0709:27:32T257cb468-b340-48e7-9db0-f6e8f23a2967;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_26} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE12 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 12. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.390.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL12 |  |  |  |  |  |  | 0x440 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1088 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index 12{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_27_signal=1;virtualpairUid=2022-09-0709:27:32T4edac029-23e9-426f-9633-5afabe5deb23;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_27} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE12 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.392.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE12 |  |  |  |  |  |  | 0x444 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1092 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 12{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_28_signal=1;virtualpairUid=2022-09-0709:27:32T6014bd8c-5c14-4197-8d92-8372f5024b4f;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_28} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE12 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frame 12{has_reset=0} |  |  |  |  |  |

| 1.1.1.394.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE13 |  |  |  |  |  |  | 0x448 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1096 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 13{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_29_signal=1;virtualpairUid=2022-09-0709:27:32Tfc71c8cc-02c7-44e0-87ae-dd8fc89f6574;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_29} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE13 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 13 Compressed Luma base address of DPB index 13{has_reset=0} |  |  |  |  |  |

| 1.1.1.396.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE13 |  |  |  |  |  |  | 0x44C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1100 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index 13{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_30_signal=1;virtualpairUid=2022-09-0709:27:32T437bfe73-b8e9-4e66-a6f3-3625e58b004c;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_30} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE13 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 13 Compressed Cb and Cr base address of DPB index 13{has_reset=0} |  |  |  |  |  |

| 1.1.1.397.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE13 |  |  |  |  |  |  | 0x450 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1104 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 13{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_31_signal=1;virtualpairUid=2022-09-0709:27:32T9cb53884-0469-44dd-b18e-5d2f73df6875;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_31} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE13 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 13{has_reset=0} |  |  |  |  |  |

| 1.1.1.399.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET13 |  |  |  |  |  |  | 0x450 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1104 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 13{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_31_signal=2;virtualpairUid=2022-09-0709:27:32T9cb53884-0469-44dd-b18e-5d2f73df6875;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_31} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE13 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB index 13 when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.401.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET13 |  |  |  |  |  |  | 0x454 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1108 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index 13{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_32_signal=1;virtualpairUid=2022-09-0709:27:32T049b1faf-986e-4889-8677-63060f19ffb4;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_32} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE13 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 13. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.403.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL13 |  |  |  |  |  |  | 0x458 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1112 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index 13{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_33_signal=1;virtualpairUid=2022-09-0709:27:32Tb428ec4d-ca90-4fb5-80a5-96d2e3fa9cc4;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_33} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE13 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.405.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE13 |  |  |  |  |  |  | 0x45C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1116 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 13{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_34_signal=1;virtualpairUid=2022-09-0709:27:32Tbed604b2-82ee-46a5-a5d1-169eb283106e;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_34} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE13 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frame 13{has_reset=0} |  |  |  |  |  |

| 1.1.1.407.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE14 |  |  |  |  |  |  | 0x460 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1120 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 14{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_35_signal=1;virtualpairUid=2022-09-0709:27:32T7d9aea98-bc2e-4938-bdd2-6b9dcbd1e477;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_35} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE14 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 14 Compressed Luma base address of DPB index 14{has_reset=0} |  |  |  |  |  |

| 1.1.1.409.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE14 |  |  |  |  |  |  | 0x464 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1124 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index 14{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_36_signal=1;virtualpairUid=2022-09-0709:27:32T5d82eed9-66bd-44a1-b006-3b2724d9125d;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_36} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE14 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 14 Compressed Cb and Cr base address of DPB index 14{has_reset=0} |  |  |  |  |  |

| 1.1.1.410.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE14 |  |  |  |  |  |  | 0x468 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1128 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 14{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_37_signal=1;virtualpairUid=2022-09-0709:27:32T569896bc-be1f-45f2-815e-4ac4c43feeec;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_37} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE14 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 14{has_reset=0} |  |  |  |  |  |

| 1.1.1.412.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET14 |  |  |  |  |  |  | 0x468 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1128 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 14{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_37_signal=2;virtualpairUid=2022-09-0709:27:32T569896bc-be1f-45f2-815e-4ac4c43feeec;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_37} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE14 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB index 14 when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.414.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET14 |  |  |  |  |  |  | 0x46C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1132 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index 14{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_38_signal=1;virtualpairUid=2022-09-0709:27:32T6c48ea0c-cb01-4495-970d-e777b6a99cd2;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_38} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE14 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 14. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.416.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL14 |  |  |  |  |  |  | 0x470 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1136 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index 14{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_39_signal=1;virtualpairUid=2022-09-0709:27:32Tbc1bccac-84b1-4dcb-a5dd-5345bba52a7b;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_39} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE14 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.418.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE14 |  |  |  |  |  |  | 0x474 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1140 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 14{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_40_signal=1;virtualpairUid=2022-09-0709:27:32T204ca053-81fe-457e-b0c7-904ad9458dec;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_40} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE14 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frame 14{has_reset=0} |  |  |  |  |  |

| 1.1.1.420.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE15 |  |  |  |  |  |  | 0x478 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1144 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 15{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_41_signal=1;virtualpairUid=2022-09-0709:27:32T69b791c3-4190-4171-9698-d434faaead23;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_41} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE15 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 15 Compressed Luma base address of DPB index 15{has_reset=0} |  |  |  |  |  |

| 1.1.1.422.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE15 |  |  |  |  |  |  | 0x47C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1148 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index 15{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_42_signal=1;virtualpairUid=2022-09-0709:27:32T0d8e743e-08a4-4b35-a3aa-993499b4ac8c;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_42} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE15 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 15 Compressed Cb and Cr base address of DPB index 15{has_reset=0} |  |  |  |  |  |

| 1.1.1.423.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE15 |  |  |  |  |  |  | 0x480 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1152 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 15{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_43_signal=1;virtualpairUid=2022-09-0709:27:32Tc9a6cf0a-3d81-40f8-9953-57f71605f9b5;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_43} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE15 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 15{has_reset=0} |  |  |  |  |  |

| 1.1.1.425.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_Y_OFFSET15 |  |  |  |  |  |  | 0x480 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1152 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC luma offset base of index 15{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_43_signal=2;virtualpairUid=2022-09-0709:27:32Tc9a6cf0a-3d81-40f8-9953-57f71605f9b5;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_43} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_LUMA_OFFSET_BASE15 |  |  | rw | ro |  | 0x0 |  |  | Compressed Luma offset table base address of DPB index 15 when FBC is used{has_reset=0} |  |  |  |  |  |

| 1.1.1.427.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET15 |  |  |  |  |  |  | 0x484 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1156 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index 15{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_44_signal=1;virtualpairUid=2022-09-0709:27:32T7f0ca812-969c-426f-a8d4-7c827c1970eb;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_44} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE15 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 15. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.429.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL15 |  |  |  |  |  |  | 0x488 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1160 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index 15{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_45_signal=1;virtualpairUid=2022-09-0709:27:32T3e8e4988-7ce4-4be3-8095-4e4d25377299;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_45} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE15 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.432.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE15 |  |  |  |  |  |  | 0x48C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1164 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 15 {alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_46_signal=1;virtualpairUid=2022-09-0709:27:32Tc41719ba-8f58-4d16-905c-4c5dfec84feb;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_46} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE15 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frame 15{has_reset=0} |  |  |  |  |  |

| 1.1.1.434.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_DEFAULT_CDF |  |  |  |  |  |  | 0x494 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1172 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_SET_FB_ADDR_DEFAULT_CDF{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_48_signal=1;virtualpairUid=2022-09-0709:27:32T9c5754e0-c4a0-46d6-b4b2-d6268ad29c2c;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_48} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DEFAULT_CDF_BASE |  |  | rw | ro |  | 0x0 |  |  | Start Address of default cdf table AV1 encoder/decoder only{has_reset=0} |  |  |  |  |  |

| 1.1.1.436.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SEGMAP |  |  |  |  |  |  | 0x498 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1176 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_SET_FB_ADDR_SEGMAP{alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_49_signal=1;virtualpairUid=2022-09-0709:27:32T16080eee-d963-4ecb-a898-fcde367152c7;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_49} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SEGMAP_BUF_BASE |  |  | rw | ro |  | 0x0 |  |  | Start Address of segmap buffer VP9 decoder only{has_reset=0} |  |  |  |  |  |

| 1.1.1.439.0 |  |  |  |  |  |  |  | CMD_SET_FB_DEC_PP_PVRIC_CTRL |  |  |  |  |  |  | 0x49C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1180 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| PVRIC control resigster (Post processing){alt_mode=CMD_ENC_CUSTOM_LAMBDA_DATA_50_signal=1;virtualpairUid=2022-09-0709:27:32T785cc35c-8aa3-4efd-a5a3-1c0143febd40;alternate_reg=CMD_ENC_CUSTOM_LAMBDA_DATA_50} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:10 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 9:9 | PVRIC_REQ_LOSSY |  |  | rw | ro |  | 0x0 |  |  | pvric lossy 0: off 1: on{has_reset=0} |  |  |  |  |  |
| 8:7 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 6:0 | PVRIC_REQ_FMT |  |  | rw | ro |  | 0x0 |  |  | pvric format 0x37 : YVU420, 8bit 0x65: YVU420, 10bit{has_reset=0} |  |  |  |  |  |

| 1.1.1.448.0 |  |  |  |  |  |  |  | CMD_SET_FB_DEC_PP_SCL_PARAM |  |  |  |  |  |  | 0x4A4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1188 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Scaler control paramter (Post processing){alt_mode=CMD_ENC_SEQ_TEMPORAL_LAYER_0_QP_signal=1;virtualpairUid=2022-09-0709:27:32T7a94c0e2-7a19-4ed8-a199-49f70b83944c;alternate_reg=CMD_ENC_SEQ_TEMPORAL_LAYER_0_QP} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:5 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 4:3 | SCL_CHR_FORMAT |  |  | rw | ro |  | 0x0 |  |  | Output chroma format 0 : 420 1 : 422 2 : 444 This field is valid only if SCL_EN is 1.{has_reset=0} |  |  |  |  |  |
| 2:1 | SCL_COEF_MODE |  |  | rw | ro |  | 0x0 |  |  | Scaler interpolation mode 0 : bicubic 0.5 1 : bicubic 1.0 2 : bicubic 1.5 3 : lanczos This field is valid only if SCL_EN is 1.{has_reset=0} |  |  |  |  |  |
| 0:0 | SCL_EN |  |  | rw | ro |  | 0x0 |  |  | This field indicates whether output scaling is enabled or not. 0: Scaling disable 1: Scaling enable This field is valid only if BWB_ENABLE is 1. [NOTE] This is valid only if scaler hardware is included in the product.{has_reset=0} |  |  |  |  |  |

| 1.1.1.450.0 |  |  |  |  |  |  |  | CMD_SET_FB_DEC_PP_CROP_POS |  |  |  |  |  |  | 0x4C4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1220 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Crop start position (Post processing){alt_mode=CMD_ENC_SEQ_SFS_PARAM_signal=1;virtualpairUid=2022-09-0709:27:32Te268da9d-9d30-4380-9920-5097e1165065;alternate_reg=CMD_ENC_SEQ_SFS_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | CROP_START_POS_X |  |  | rw | ro |  | 0x0 |  |  | Crop start x position - position must be 16 align{has_reset=0} |  |  |  |  |  |
| 15:0 | CROP_START_POS_Y |  |  | rw | ro |  | 0x0 |  |  | Crop start y position - position must be 16 align{has_reset=0} |  |  |  |  |  |

| 1.1.1.452.0 |  |  |  |  |  |  |  | CMD_SET_FB_DEC_PP_CROP_SIZE |  |  |  |  |  |  | 0x4C8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1224 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Crop size (Post processing){alt_mode=CMD_ENC_SEQ_CROP_ENABLE_signal=1;virtualpairUid=2022-09-0709:27:32T92b71e80-e251-49e3-bc64-026b3fa4ac42;alternate_reg=CMD_ENC_SEQ_CROP_ENABLE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | CROP_WIDTH |  |  | rw | ro |  | 0x0 |  |  | Crop width - size must be 16 align{has_reset=0} |  |  |  |  |  |
| 15:0 | CROP_HEIGHT |  |  | rw | ro |  | 0x0 |  |  | Crop height - size must be 16 align{has_reset=0} |  |  |  |  |  |

| 1.1.1.457.0 |  |  |  |  |  |  |  | CMD_SET_FB_DEC_PP_AFBC_COMMON |  |  |  |  |  |  | 0x4CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1228 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| AFBC control paramter (Post processing){alt_mode=CMD_ENC_SEQ_CROP_START_POS_signal=1;virtualpairUid=2022-09-0709:27:32T5e862428-201b-4a4e-80db-a7edfdd47190;alternate_reg=CMD_ENC_SEQ_CROP_START_POS} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:17 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 16:16 | AFBC_TZMP_ERR |  |  | rw | ro |  | 0x0 |  |  | AFBC core master setting value{has_reset=0} |  |  |  |  |  |
| 15:15 | AFBC_SINGLE_ID |  |  | rw | ro |  | 0x0 |  |  | AFBC core master setting value{has_reset=0} |  |  |  |  |  |
| 14:12 | AFBC_OUTSTANDING |  |  | rw | ro |  | 0x0 |  |  | AFBC core master setting value{has_reset=0} |  |  |  |  |  |
| 11:9 | AFBC_MAX_LEN |  |  | rw | ro |  | 0x0 |  |  | AFBC core master setting value{has_reset=0} |  |  |  |  |  |
| 8:8 | AFBC_AFBC_DATA_128B |  |  | rw | ro |  | 0x0 |  |  | AFBC core master setting value{has_reset=0} |  |  |  |  |  |
| 7:7 | AFBC_AWWIDE_SB |  |  | rw | ro |  | 0x0 |  |  | AFBC sb_mode{has_reset=0} |  |  |  |  |  |
| 6:6 | AFBC_AWHALF_SB |  |  | rw | ro |  | 0x0 |  |  | AFBC split mode{has_reset=0} |  |  |  |  |  |
| 5:1 | AFBC_FORMAT_IDX |  |  | rw | ro |  | 0x0 |  |  | AFBC superblock format index{has_reset=0} |  |  |  |  |  |
| 0:0 | AFBC_SOLID_COLOR |  |  | rw | ro |  | 0x0 |  |  | AFBC solid color{has_reset=0} |  |  |  |  |  |

| 1.1.1.1.0 |  |  |  |  |  |  |  | CMD_SET_FB_DEC_ADDR_COL_DUAL |  |  |  |  |  |  | 0x4DC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1244 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| {alt_mode=CMD_ENC_SEQ_Y2Y_DATA_1_signal=1;virtualpairUid=2022-09-0709:27:32T93ccfa6e-bf06-4cd1-a09c-df81a8f984dc;alternate_reg=CMD_ENC_SEQ_Y2Y_DATA_1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | ADDR_DUAL_COL |  |  | rw | ro |  | 0x0 |  |  | Required only for vp9 dual and av1 dual{has_reset=0} |  |  |  |  |  |

| 1.1.1.2.0 |  |  |  |  |  |  |  | HOST_GLOBAL_WR |  |  |  |  |  |  | 0x000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 0 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Host can read and write this register but this register does not affect any VPU operation. VPU has internally single register for all 4 VMs instead of each regsiter for each VM. So, reading/writing from/to this register through any VM accesses the same regsiter value. The purpose of this register is that host controls VM using mutex such as test_and_set operation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | HOST_GLOBAL_WR |  |  | rw | ro |  | 0x0 |  |  | host data |  |  |  |  |  |

| 1.1.1.3.0 |  |  |  |  |  |  |  | VCPU_CUR_PC |  |  |  |  |  |  | 0x004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 4 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Current program counter value of V-CPU |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CUR_PC |  |  | ro | ro |  | 0x0 |  |  | [DEBUG] PC value represents the address of instruction which is executed in V-CPU |  |  |  |  |  |

| 1.1.1.4.0 |  |  |  |  |  |  |  | VCPU_CUR_LR |  |  |  |  |  |  | 0x008 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 8 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Current LR (for debugger only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CUR_LR |  |  | ro | ro |  | 0x0 |  |  | [DEBUG] Current LR (Link Register) to find out caller address |  |  |  |  |  |

| 1.1.1.5.0 |  |  |  |  |  |  |  | DBG_MSG_17 |  |  |  |  |  |  | 0x00C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 12 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.6.0 |  |  |  |  |  |  |  | DBG_MSG_18 |  |  |  |  |  |  | 0x010 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 16 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.7.0 |  |  |  |  |  |  |  | DBG_MSG_19 |  |  |  |  |  |  | 0x014 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 20 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.8.0 |  |  |  |  |  |  |  | DBG_MSG_20 |  |  |  |  |  |  | 0x018 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 24 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.9.0 |  |  |  |  |  |  |  | DBG_MSG_21 |  |  |  |  |  |  | 0x01C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 28 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.10.0 |  |  |  |  |  |  |  | VPU_FIO_ADDR |  |  |  |  |  |  | 0x020 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 32 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FIO CRTL, READY, and Address for accessing FIO (FastIO) which is an internal peripheral bus in VPU. By accessing FIO, user can check the internal status of some accelerators in VPU for debugging purpose in some cases. FIO transaction is carried out when host writes this register. CAUTON: Accessing FIO can make unwanted behavior in VPU, please access it using API only. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:31 | READY |  |  | rw | ro |  | 0x0 |  |  | Ready for the transaction When writing, it should be 0. |  |  |  |  |  |
| 16:16 | RW_FLAG |  |  | wo | ro |  | 0x0 |  |  | Read/Write transaction control 0: read 1: write |  |  |  |  |  |
| 15:0 | FIO_ADDR |  |  | wo | ro |  | 0x0 |  |  | FIO Address |  |  |  |  |  |

| 1.1.1.11.0 |  |  |  |  |  |  |  | VPU_FIO_DATA |  |  |  |  |  |  | 0x024 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 36 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FIO data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FIO_DATA |  |  | rw | ro |  | 0x0 |  |  | When writing, FIO data should be written to this register firstly. When reading, # Write data to this register. # Check whether READY flag of VPU_FIO_CTRL_ADDR register is 1. # When READY flag is asserted, VPU reads data from this VPU_FIO_DATA. |  |  |  |  |  |

| 1.1.1.12.0 |  |  |  |  |  |  |  | DBG_MSG_0 |  |  |  |  |  |  | 0x028 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 40 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.13.0 |  |  |  |  |  |  |  | DBG_MSG_1 |  |  |  |  |  |  | 0x02C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 44 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.14.0 |  |  |  |  |  |  |  | DBG_MSG_22 |  |  |  |  |  |  | 0x030 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 48 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.15.0 |  |  |  |  |  |  |  | VPU_VINT_REASON_CLR |  |  |  |  |  |  | 0x034 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 52 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Interrupt Reason Clear This register clears the interrupt reason in ISR when host processor catchs an interrupt. It is to notify that host processor has received the interrupt. If host processor wants to get task done interrupts or so from VPU, they should set the fields of VPU_VINT_ENABLE register as they wish. And when they recieve any of the command done interrupt from VPU (VPU_VINT_REASON_CLR is not zero), host processor should check the interrupt and do the following sequence for safe interrupt clear. 1. Clear the interrupt by setting the relevant bit of this VPU_VINT_REASON_CLR. 2. Set VPU_VINT_CLEAR to 1, which drops the interrupt signal. Then VPU_VPU_INT_STS is automatically cleared. In above sequence, 1 should be done first than 2 because VPU_VINT_CLEAR without VPU_VINT_REASON_CLR leads to interrupt reassurtion. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 15:15 | BSEMPTY_CLR |  |  | wo | ro |  | 0x0 |  |  | Bitstream empty (bitstream feeding request) interrupt clear |  |  |  |  |  |
| 14:14 | RSVD0 |  |  | wo | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 13:13 | RSVD1 |  |  | wo | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 12:12 | RSVD2 |  |  | wo | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 11:11 | RSVD3 |  |  | wo | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 10:10 | RSVD4 |  |  | wo | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 9:9 | CMD9_CLR |  |  | wo | ro |  | 0x0 |  |  | ENC_SET_PARAM command done interrupt clear |  |  |  |  |  |
| 8:8 | CMD8_CLR |  |  | wo | ro |  | 0x0 |  |  | DEC_PIC/ENC_PIC command done interrupt clear |  |  |  |  |  |
| 7:7 | CMD7_CLR |  |  | wo | ro |  | 0x0 |  |  | SET_FRAMEBUFFER command done interrupt clear |  |  |  |  |  |
| 6:6 | CMD6_CLR |  |  | wo | ro |  | 0x0 |  |  | INIT_SEQ command done interrupt clear |  |  |  |  |  |
| 5:5 | CMD5_CLR |  |  | wo | ro |  | 0x0 |  |  | DESTROY_INSTANCE command done interrupt clear |  |  |  |  |  |
| 4:4 | CMD4_CLR |  |  | wo | ro |  | 0x0 |  |  | FLSUH_INSTANCE command done interrupt clear |  |  |  |  |  |
| 3:3 | CMD3_CLR |  |  | wo | ro |  | 0x0 |  |  | CREATE_INSTANCE command done interrupt clear |  |  |  |  |  |
| 2:2 | CMD2_CLR |  |  | wo | ro |  | 0x0 |  |  | SLEEP_VPU command done interrupt clear |  |  |  |  |  |
| 1:1 | CMD1_CLR |  |  | wo | ro |  | 0x0 |  |  | WAKE_VPU command done interrupt clear |  |  |  |  |  |
| 0:0 | CMD0_CLR |  |  | wo | ro |  | 0x0 |  |  | INIT_VPU command done interrupt clear |  |  |  |  |  |

| 1.1.1.16.0 |  |  |  |  |  |  |  | VPU_HOST_INT_REQ |  |  |  |  |  |  | 0x038 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 56 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Interrupt request sent from host processor to VPU for the command and so on. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 0:0 | HINTREQ |  |  | wo | ro |  | 0x0 |  |  | If this is set to 1, an interrupt named HOST interrupt is sent to VPU. |  |  |  |  |  |

| 1.1.1.17.0 |  |  |  |  |  |  |  | VPU_VINT_CLEAR |  |  |  |  |  |  | 0x03C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 60 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Clear VPU interrupt. Host processor can clear the VPU interrupt which has been pending through this register. As mentioned in VPU_VINT_REASON_CLR, this register setting definetely come after VPU_INT_REASON setting. Also, setting this VPU_VINT_CLEAR to 1 forces VPU_VPU_INT_STS INT_STS to be cleared automatically. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 0:0 | VINTCLR |  |  | wo | ro |  | 0x0 |  |  | Clear VPU interrupt. |  |  |  |  |  |

| 1.1.1.18.0 |  |  |  |  |  |  |  | VPU_HINT_CLEAR |  |  |  |  |  |  | 0x040 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 64 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Check Host Command Interrupt is cleared. From host side, this register is not used or checked. VPU can clear the host interrupt which has been pending through this register. When VPU operation for the host interrupt is done, VPU sets this reigster to clear the host interrupt. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 0:0 | HINTCLR |  |  | wo | ro |  | 0x0 |  |  | Check Host Command Interrupt is cleared. |  |  |  |  |  |

| 1.1.1.19.0 |  |  |  |  |  |  |  | VPU_VPU_INT_STS |  |  |  |  |  |  | 0x044 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 68 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Interrupt Status. This has the same value as interrupt output signal from VPU. This is a way of interrupt check by register polling. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 0:0 | VPU_VPU_INT_STS |  |  | ro | ro |  | 0x0 |  |  | Interrupt Status |  |  |  |  |  |

| 1.1.1.20.0 |  |  |  |  |  |  |  | VPU_VINT_ENABLE |  |  |  |  |  |  | 0x048 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 72 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Interrupt Enable for each interrupt reason(source) Interrupt in LSB position shall be handled with higher priority. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 15:15 | BSEMPTY_EN |  |  | rw | ro |  | 0x0 |  |  | Bitstream empty (bitstream feeding request) interrupt clear |  |  |  |  |  |
| 14:14 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 13:13 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 12:12 | RSVD2 |  |  | rw | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 11:11 | RSVD3 |  |  | rw | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 10:10 | RSVD4 |  |  | rw | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 9:9 | CMD9_EN |  |  | rw | ro |  | 0x0 |  |  | ENC_SET_PARAM command done interrupt enable |  |  |  |  |  |
| 8:8 | CMD8_EN |  |  | rw | ro |  | 0x0 |  |  | DEC_PIC/ENC_PIC command done interrupt enable |  |  |  |  |  |
| 7:7 | CMD7_EN |  |  | rw | ro |  | 0x0 |  |  | SET_FRAMEBUFFER command done interrupt enable |  |  |  |  |  |
| 6:6 | CMD6_EN |  |  | rw | ro |  | 0x0 |  |  | INIT_SEQ command done interrupt enable |  |  |  |  |  |
| 5:5 | CMD5_EN |  |  | rw | ro |  | 0x0 |  |  | DESTROY_INSTANCE command done interrupt enable |  |  |  |  |  |
| 4:4 | CMD4_EN |  |  | rw | ro |  | 0x0 |  |  | FLSUH_INSTANCE command done interrupt enable |  |  |  |  |  |
| 3:3 | CMD3_EN |  |  | rw | ro |  | 0x0 |  |  | CREATE_INSTANCE command done interrupt enable |  |  |  |  |  |
| 2:2 | CMD2_EN |  |  | rw | ro |  | 0x0 |  |  | SLEEP_VPU command done interrupt enable |  |  |  |  |  |
| 1:1 | CMD1_EN |  |  | rw | ro |  | 0x0 |  |  | WAKE_VPU command done interrupt enable |  |  |  |  |  |
| 0:0 | CMD0_EN |  |  | rw | ro |  | 0x0 |  |  | INIT_VPU command done interrupt enable |  |  |  |  |  |

| 1.1.1.22.0 |  |  |  |  |  |  |  | VPU_VINT_REASON |  |  |  |  |  |  | 0x04C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 76 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| VPU interrupt reason This register Interrupt reasons are almost the same with the command. Interrupt in LSB position shall be handled with higher priority. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 15:15 | BSEMPTY_INTR |  |  | rw | ro |  | 0x0 |  |  | Bitstream empty (bitstream feeding request) |  |  |  |  |  |
| 14:14 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 13:13 | CMDD_INTR |  |  | rw | ro |  | 0x0 |  |  | Low latency interrupt |  |  |  |  |  |
| 12:12 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 11:11 | RSVD2 |  |  | rw | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 10:10 | RSVD3 |  |  | rw | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 9:9 | CMD9_INTR |  |  | rw | ro |  | 0x0 |  |  | ENC_SET_PARAM command done interrupt |  |  |  |  |  |
| 8:8 | CMD8_INTR |  |  | rw | ro |  | 0x0 |  |  | DEC_PIC/ENC_PIC command done interrupt |  |  |  |  |  |
| 7:7 | CMD7_INTR |  |  | rw | ro |  | 0x0 |  |  | SET_FRAMEBUFFER command done interrupt |  |  |  |  |  |
| 6:6 | CMD6_INTR |  |  | rw | ro |  | 0x0 |  |  | INIT_SEQ command done interrupt |  |  |  |  |  |
| 5:5 | CMD5_INTR |  |  | rw | ro |  | 0x0 |  |  | DESTROY_INSTANCE command done interrupt |  |  |  |  |  |
| 4:4 | CMD4_INTR |  |  | rw | ro |  | 0x0 |  |  | FLSUH_INSTANCE command done interrupt |  |  |  |  |  |
| 3:3 | CMD3_INTR |  |  | rw | ro |  | 0x0 |  |  | CREATE_INSTANCE command done interrupt |  |  |  |  |  |
| 2:2 | CMD2_INTR |  |  | rw | ro |  | 0x0 |  |  | SLEEP_VPU command done interrupt |  |  |  |  |  |
| 1:1 | CMD1_INTR |  |  | rw | ro |  | 0x0 |  |  | WAKE_VPU command done interrupt |  |  |  |  |  |
| 0:0 | CMD0_INTR |  |  | rw | ro |  | 0x0 |  |  | INIT_VPU command done interrupt |  |  |  |  |  |

| 1.1.1.24.0 |  |  |  |  |  |  |  | VCPU_RESTART |  |  |  |  |  |  | 0x058 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 88 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| V-CPU restart request |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 0:0 | VCPU_RESTART |  |  | wo | ro |  | 0x0 |  |  | This register restarts V-CPU from the reset vector without clearing H/W logic. 0: DO NOTHING 1: RESTART REQUEST |  |  |  |  |  |

| 1.1.1.25.0 |  |  |  |  |  |  |  | VPU_REMAP_CTRL |  |  |  |  |  |  | 0x060 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 96 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Remap control register (REMAP REG ADDR / ATTR / SIZE) VPU remaps addresses VCPU uses between the physical memory and virtual memory for efficient address management. VPU works with the virtual memory addresses that are translated to the phsyical addresses. Host processor needs to assign V-CPU code buffer to VPU_REMAP_PADDR and VPU_REMAP_VADDR to 0, so that VPU can start by reading from VPU_REMAP_PADDR considering it is its the base address 0. This register has the configuration fields for remapping. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:31 | REMAP_GLOBEN |  |  | rw | ro |  | 0x0 |  |  | Set 1 if you want to change the [30:12] part of this register Default to be 0 |  |  |  |  |  |
| 30:20 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 19:16 | ENDIAN |  |  | rw | ro |  | 0x0 |  |  | Endianness for memory access Endian control of memory transactions from processor core |  |  |  |  |  |
| 15:12 | REMAP_IDX |  |  | rw | ro |  | 0x0 |  |  | Remap index Only 0 to 3 are valid. |  |  |  |  |  |
| 11:11 | REMAP_PAGE_SIZE_EN |  |  | rw | ro |  | 0x0 |  |  | Set 1 if you want to change the REMAP_PSIZE field Default to be 0 |  |  |  |  |  |
| 10:9 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 8:0 | REMAP_PSIZE |  |  | rw | ro |  | 0x0 |  |  | Remap Page Size (page base should be aligned in 4KB boundary) 0x001: 4K 0x002: 8K 0x004: 16K 0x008: 32K 0x010 : 64K 0x020 : 128K 0x040 : 256K 0x080: 512K 0x100: 1M |  |  |  |  |  |

| 1.1.1.26.0 |  |  |  |  |  |  |  | VPU_REMAP_VADDR |  |  |  |  |  |  | 0x064 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 100 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Remap region base address in virtual address space Virtual address space is an address generated by V-CPU. To setup Remap region, keep the sequence below. # set remap index using REMAP_IDX in VPU_REMAP_CTRL. While setting up, other information should be the same as the previous value # set virtual address using VPU_REMAP_VADDR # set physical address using VPU_REMAP_PADDR CAUTION: In case of CODE section (which has REMAP IDX 0), virtual address should be 0x0, since V-CPU always start booting up from virtual address 0. If not, VPU can not boot-up. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:12 | VPU_REMAP_VADDR |  |  | rw | ro |  | 0x0 |  |  | Remap region base address in virtual address space. The address should be aligned to 4KB boundary. |  |  |  |  |  |

| 1.1.1.27.0 |  |  |  |  |  |  |  | VPU_REMAP_PADDR |  |  |  |  |  |  | 0x068 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 104 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Remap region base address in physical address space |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:12 | VPU_REMAP_PADDR |  |  | rw | ro |  | 0x0 |  |  | Real address(physical address) as a pair of virtual address. It should aligned to 4KB boundary. |  |  |  |  |  |

| 1.1.1.28.0 |  |  |  |  |  |  |  | VPU_REMAP_CORE_START |  |  |  |  |  |  | 0x06C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 108 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| VPU Start Request |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 0:0 | VPU_REMAP_CORE_START |  |  | rw | ro |  | 0x0 |  |  | It starts VPU after initial setting has been done. After setting up remap or initial configuartion, host should set this register to init VPU. |  |  |  |  |  |

| 1.1.1.29.0 |  |  |  |  |  |  |  | VCPU_CMD_BUSY_STATUS |  |  |  |  |  |  | 0x070 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 112 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| V-CPU Busy Status |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 0:0 | VPU_BUSY_STATUS |  |  | rw | ro |  | 0x0 |  |  | Command Reentrance Check [0] 0: Host processor has the ownership for access to host interface. 1: VPU has the ownership for access to host interface. - host should set this flag just before trying to send a command into command-queue - firmware clears this flag when a command is queueued for processing. - for the case of clock_gating or power_down, please use VPU_VCPU_STATUS register to check the status of VPU. |  |  |  |  |  |

| 1.1.1.30.0 |  |  |  |  |  |  |  | VPU_HALT_STATUS |  |  |  |  |  |  | 0x074 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 116 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| For power control, status checking of V-CPU |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 4:4 | VPU_HALT_STATUS |  |  | ro | ro |  | 0x0 |  |  | 1: V-CPU is on the HALT status |  |  |  |  |  |
| 3:0 | VPU_HALT_STATUS_DEBUG |  |  | ro | ro |  | 0x0 |  |  | for debugging - Eventhough V-CPU is on HALT status, for the clock gating or power down, VPU_VCPU_STATUS should be checked to check pending bus operations. |  |  |  |  |  |

| 1.1.1.31.0 |  |  |  |  |  |  |  | VPU_VCPU_STATUS |  |  |  |  |  |  | 0x078 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 120 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| V-CPU bus busy and V-CPU status |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 14:0 | VPU_VCPU_STATUS |  |  | ro | ro |  | 0x0 |  |  | If [31:16] is 0x0000, there is no more bus transaction on V-CPU. If [15:0] is 0x0040, V-CPU is on the halt status. Thus, the value returns 0x40, power for VPU can be turnned-off |  |  |  |  |  |

| 1.1.1.32.0 |  |  |  |  |  |  |  | VPU_BUSY_STATUS |  |  |  |  |  |  | 0x07C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 124 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| VPU Busy Status |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 0:0 | VPU_BUSY_STATUS |  |  | ro | ro |  | 0x0 |  |  | 0 : VPU is idle. Host processor can gate the clock of VPU. 1 : VPU is busy. |  |  |  |  |  |

| 1.1.1.33.0 |  |  |  |  |  |  |  | RET_FIO_STATUS |  |  |  |  |  |  | 0x080 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 128 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | RSVD0 |  |  | ro | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |

| 1.1.1.34.0 |  |  |  |  |  |  |  | DBG_MSG_3 |  |  |  |  |  |  | 0x084 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 132 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.35.0 |  |  |  |  |  |  |  | DBG_MSG_4 |  |  |  |  |  |  | 0x088 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 136 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.36.0 |  |  |  |  |  |  |  | DBG_MSG_5 |  |  |  |  |  |  | 0x08C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 140 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.37.0 |  |  |  |  |  |  |  | RET_PRODUCT_NAME |  |  |  |  |  |  | 0x090 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 144 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| VPU hardware product name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 3:0 | HW_NAME |  |  | ro | ro |  | 0x0 |  |  | It always returns "WAVE" for WAVE6 series IP product. |  |  |  |  |  |

| 1.1.1.38.0 |  |  |  |  |  |  |  | RET_PRODUCT_VERSION |  |  |  |  |  |  | 0x094 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 148 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| VPU hardware product version information. [NOTE] The list is just examples of version code. Version code can be varied without notice |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 3:0 | PRODUCT_NUMBER |  |  | ro | ro |  | 0x0 |  |  | It returns as follows: 0x6270 for WAVE627 |  |  |  |  |  |

| 1.1.1.39.0 |  |  |  |  |  |  |  | RET_VCPU_CONFIG0 |  |  |  |  |  |  | 0x098 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 152 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Reserved for Configuration Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | RSVD0 |  |  | ro | ro |  | 0x0 |  |  | Configuration Information #0 (internal use only) |  |  |  |  |  |

| 1.1.1.40.0 |  |  |  |  |  |  |  | RET_VCPU_CONFIG1 |  |  |  |  |  |  | 0x09C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 156 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Reserved for Configuration Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | RSVD0 |  |  | ro | ro |  | 0x0 |  |  | Configuration Information #1 (internal use only) |  |  |  |  |  |

| 1.1.1.41.0 |  |  |  |  |  |  |  | RET_CODEC_STD |  |  |  |  |  |  | 0x0A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 160 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Reserved for Configuration Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CODEC_STD |  |  | ro | ro |  | 0x0 |  |  | General Configuration Information (internal use only) |  |  |  |  |  |

| 1.1.1.42.0 |  |  |  |  |  |  |  | RET_CONF_DATE |  |  |  |  |  |  | 0x0A4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 164 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Reserved for Configuration Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | HW_DATE |  |  | ro | ro |  | 0x0 |  |  | The date that the hardware has been configured in YYYYmmdd in digit (internal use only) |  |  |  |  |  |

| 1.1.1.43.0 |  |  |  |  |  |  |  | RET_CONF_REVISION |  |  |  |  |  |  | 0x0A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 168 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Reserved for Configuration Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | HW_REVISION |  |  | ro | ro |  | 0x0 |  |  | The revision number when the hardware has been configured (internal use only) |  |  |  |  |  |

| 1.1.1.44.0 |  |  |  |  |  |  |  | RET_CONF_TYPE |  |  |  |  |  |  | 0x0AC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 172 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Reserved for Configuration Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | HW_TYPE |  |  | ro | ro |  | 0x0 |  |  | The define value used in hardware configuration (internal use only) |  |  |  |  |  |

| 1.1.1.45.0 |  |  |  |  |  |  |  | RET_VCORE0_CFG |  |  |  |  |  |  | 0x0B0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 176 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Reserved for Configuration Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CONFIG_VORE0 |  |  | ro | ro |  | 0x0 |  |  | The VCORE0 Configuration Information (internal use only) |  |  |  |  |  |

| 1.1.1.46.0 |  |  |  |  |  |  |  | RET_VCORE1_CFG |  |  |  |  |  |  | 0x0B4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 180 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Reserved for Configuration Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CONFIG_VORE1 |  |  | ro | ro |  | 0x0 |  |  | The VCORE1 Configuration Information (internal use only) |  |  |  |  |  |

| 1.1.1.47.0 |  |  |  |  |  |  |  | RET_VCORE2_CFG |  |  |  |  |  |  | 0x0B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 184 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Reserved for Configuration Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CONFIG_VORE2 |  |  | ro | ro |  | 0x0 |  |  | The VCORE2 Configuration Information (internal use only) |  |  |  |  |  |

| 1.1.1.48.0 |  |  |  |  |  |  |  | RET_VCORE3_CFG |  |  |  |  |  |  | 0x0BC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 188 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Reserved for Configuration Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CONFIG_VORE3 |  |  | ro | ro |  | 0x0 |  |  | The VCORE3 Configuration Information (internal use only) |  |  |  |  |  |

| 1.1.1.49.0 |  |  |  |  |  |  |  | VPU_RET_VCORE_PRESENT |  |  |  |  |  |  | 0x0C0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 192 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Number of VCOREs present (turn on cores) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 3:0 | VCORE_PRESENT |  |  | ro | ro |  | 0x0 |  |  | Each bit represnets turn-on VCORE [0]: VCORE0 [1]: VCORE1 [2]: VCORE2 [3]: VCORE3 |  |  |  |  |  |

| 1.1.1.50.0 |  |  |  |  |  |  |  | DBG_MSG_6 |  |  |  |  |  |  | 0x0C4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 196 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.51.0 |  |  |  |  |  |  |  | DBG_MSG_7 |  |  |  |  |  |  | 0x0C8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 200 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.52.0 |  |  |  |  |  |  |  | DBG_MSG_8 |  |  |  |  |  |  | 0x0CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 204 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.54.0 |  |  |  |  |  |  |  | DBG_MSG_9 |  |  |  |  |  |  | 0x0D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 208 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.55.0 |  |  |  |  |  |  |  | DBG_MSG_10 |  |  |  |  |  |  | 0x0D8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 216 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.56.0 |  |  |  |  |  |  |  | DBG_MSG_11 |  |  |  |  |  |  | 0x0DC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 220 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.57.0 |  |  |  |  |  |  |  | DBG_MSG_12 |  |  |  |  |  |  | 0x0E0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 224 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.58.0 |  |  |  |  |  |  |  | DBG_MSG_13 |  |  |  |  |  |  | 0x0E4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 228 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.59.0 |  |  |  |  |  |  |  | DBG_MSG_14 |  |  |  |  |  |  | 0x0E8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 232 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.60.0 |  |  |  |  |  |  |  | DBG_MSG_15 |  |  |  |  |  |  | 0x0EC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 236 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugging message for HOST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DBG_MSG |  |  | ro | ro |  | 0x0 |  |  | Debugging message |  |  |  |  |  |

| 1.1.1.61.0 |  |  |  |  |  |  |  | VPU_DBG_SW_UART_STATUS |  |  |  |  |  |  | 0x0F0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 240 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| VPU_DBG_SW_UART_STATUS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:4 | RSVD0 |  |  | ro | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 3:0 | VPU_DBG_SW_UART_STATUS |  |  | ro | ro |  | 0x0 |  |  | the status of VPU SW_UART |  |  |  |  |  |

| 1.1.1.62.0 |  |  |  |  |  |  |  | VPU_DBG_SW_UART_TX |  |  |  |  |  |  | 0x0F4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 244 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| VPU_DBG_SW_UART_TX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:4 | RSVD0 |  |  | ro | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 3:0 | VPU_DBG_SW_UART_TX |  |  | ro | ro |  | 0x0 |  |  | TX of VPU SW_UART |  |  |  |  |  |

| 1.1.1.63.0 |  |  |  |  |  |  |  | VPU_DBG_REG_0 |  |  |  |  |  |  | 0x0F8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 248 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| VPU_DBG_REG_0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:4 | RSVD0 |  |  | ro | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 3:0 | VPU_DBG_REG_0 |  |  | ro | ro |  | 0x0 |  |  | Debug register 0 |  |  |  |  |  |

| 1.1.1.65.0 |  |  |  |  |  |  |  | VPU_DBG_REG_1 |  |  |  |  |  |  | 0x0FC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 252 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| VPU_DBG_REG_1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:4 | RSVD0 |  |  | ro | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 3:0 | VPU_DBG_REG_1 |  |  | ro | ro |  | 0x0 |  |  | Debug register 1 |  |  |  |  |  |

| 1.1.1.67.0 |  |  |  |  |  |  |  | VPU_SUB_FRAME_SYNC_CTRL |  |  |  |  |  |  | 0x11C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 284 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | ipu_frame_row_cnt |  |  | ro | ro |  | 0x0 |  |  | Read valid source buffer count (for debugging) |  |  |  |  |  |
| 15:15 | ipu_frame_row_valid |  |  | ro | ro |  | 0x0 |  |  | Read valid source buffer validation (for debugging) |  |  |  |  |  |
| 14:7 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved |  |  |  |  |  |
| 6:2 | current_buffer |  |  | rw | ro |  | 0x0 |  |  | Change current source buffer Valid only if SUB_FRAME_SYNC_TYPE is 1. |  |  |  |  |  |
| 1:1 | new_frame |  |  | rw | ro |  | 0x0 |  |  | Toggle when new frame start. Valid only if SUB_FRAME_SYNC_TYPE is 1. |  |  |  |  |  |
| 0:0 | end_of_row |  |  | rw | ro |  | 0x0 |  |  | Toggle at end of line Valid only if SUB_FRAME_SYNC_TYPE is 1. |  |  |  |  |  |

| 1.1.1.68.0 |  |  |  |  |  |  |  | COMMAND |  |  |  |  |  |  | 0x200 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 512 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Command to run VPU Depending on the value of command, host interface registers from 0x210 mean different things. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COMMAND |  |  | rw | ro |  | 0x0 |  |  | 0x0001 : INIT_VPU 0x0002 : WAKE_VPU 0x0004 : SLEEP_VPU 0x0008 : CREATE_INST 0x0010 : FLUSH_INST 0x0020 : DESTROY_INST 0x0040 : INIT_SEQ 0x0080 : SET_FB 0x0100 : ENC_PIC / DEC_PIC 0x0200 : ENC_SET_PARAM 0x4000 : QUERY 0x8000 : UPDATE_BS{has_reset=0} |  |  |  |  |  |

| 1.1.1.73.0 |  |  |  |  |  |  |  | CMD_DEC_PIC_OPTION |  |  |  |  |  |  | 0x204 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 516 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_DEC_PIC_OPTION{alt_reg=CMD_INIT_SEQ_OPTION,CMD_OPTION,CMD_SET_FB_OPTION,CMD_SET_PARAM_OPTION} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:7 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 6:6 | FORCE_FILM_GRAIN_OFF |  |  | rw | ro |  | 0x0 |  |  | AV1 DEC only 0x0: film grain is not applied to BWB output regardless of syntax value named "apply_grain" 0x1: film grain is applied or not to BWB output in accordance with syntax value named "apply_grain"{has_reset=0} |  |  |  |  |  |
| 5:0 | SKIP_MODE |  |  | rw | ro |  | 0x0 |  |  | [HEVC DEC] 0x00: normal DEC_PIC 0x10: thumbnail mode. It skips non-IRAP pictures w/o registering reference DPB. 0x11: skip non-IRAP. 0x13: skip non-reference picture. 0x20: handle CRA picture as BLA. It skips RASL pictures followd by CRA pictures. [AVC DEC] 0x00: normal DEC_PIC 0x10: thumbnail mode. It skips non-IRAP pictures w/o registering reference DPB. 0x11: skip non-IRAP. 0x13: skip non-reference picture. [VP9 DEC] 0x00: normal DEC_PIC 0x10: thumbnail mode. It skips non-IRAP pictures w/o registering reference DPB. 0x11: skip non-key frame. [AV1 DEC] 0x00: normal DEC_PIC 0x10: thumbnail mode. It skips non-IRAP pictures w/o registering reference DPB. 0x11: skip non-key frame.{has_reset=0} |  |  |  |  |  |

| 1.1.1.74.0 |  |  |  |  |  |  |  | RET_SUCCESS |  |  |  |  |  |  | 0x208 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 520 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Result of the command If the value is not 0x1, check the reason. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 1:0 | RUN_CMD_STATUS |  |  | rw | ro |  | 0x0 |  |  | 00: FAIL 01: SUCCESS 10: SUCCESS_WITH_WARNING{has_reset=0} |  |  |  |  |  |

| 1.1.1.75.0 |  |  |  |  |  |  |  | RET_FAIL_REASON |  |  |  |  |  |  | 0x20C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 524 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Fail reason of the run command |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FAIL_REASON |  |  | rw | ro |  | 0x0 |  |  | Please refer to <<wave6_sys_error>> which provides error code description.{has_reset=0} |  |  |  |  |  |

| 1.1.1.76.0 |  |  |  |  |  |  |  | CMD_INSTANCE_INFO |  |  |  |  |  |  | 0x210 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 528 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Instance information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | CODEC_STD |  |  | rw | ro |  | 0x0 |  |  | Codec standard to run STD_HEVC_DEC = 0x00 STD_HEVC_ENC = 0x01 STD_AVC_DEC = 0x02 STD_AVC_ENC = 0x03 STD_VP9_DEC = 0x16 STD_AV1_DEC = 0x1A STD_AV1_ENC = 0x1B {has_reset=0} |  |  |  |  |  |
| 15:0 | INST_ID |  |  | rw | ro |  | 0x0 |  |  | Instance ID CREATE_INSTANCE command returns instance ID through RET_CREATE_INSTANCE_ID register. Then the following commands for this instance uses the intance ID as input. VPU can encode or decode more than one instance simultaneously. If multiple instances are running, each instance must have its own process index that is assigned by this register. For example, two instances are running simultaneously, the first instance has the process index 0, the second instance has the process index 1. Instance index shall be in the range of 0 to 31, inclusive.{has_reset=0} |  |  |  |  |  |

| 1.1.1.77.0 |  |  |  |  |  |  |  | CMD_QUE_FULL_IDC |  |  |  |  |  |  | 0x214 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 532 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Indicator for which command queue is full. Host can can read this register at any time. Because the reading of this register does not involve interrupt to VCPU, it does not cause any VCPU performance drop. Only valid for CQ FW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CMD_QUEUE_FULLNESS |  |  | rw | ro |  | 0x0 |  |  | For each bit position i, - '1' means the command queue for instance i is full. - '0' means the command queue for instance i is not full.{has_reset=0} |  |  |  |  |  |

| 1.1.1.78.0 |  |  |  |  |  |  |  | RET_QUE_EMPTY_IDC |  |  |  |  |  |  | 0x218 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 536 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Indicator for which command queue is full. Host can can read this register at any time. Because the reading of this register does not involve interrupt to VCPU, it does not cause any VCPU performance drop. Only valid for CQ FW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | RET_QUEUE_EMPTY |  |  | rw | ro |  | 0x0 |  |  | For each bit position i, - '1' means the command queue for instance i is empty. - '0' means the command queue for instance i is not empty.{has_reset=0} |  |  |  |  |  |

| 1.1.1.79.0 |  |  |  |  |  |  |  | CMD_DONE_INST_IDC |  |  |  |  |  |  | 0x21C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 540 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Done interrupt instance idc. Only valid for CQ FW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | DONE_INST_IDC |  |  | rw | ro |  | 0x0 |  |  | done instance idc for que command{has_reset=0} |  |  |  |  |  |

| 1.1.1.81.0 |  |  |  |  |  |  |  | RET_CREATE_INSTANCE_ID |  |  |  |  |  |  | 0x220 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 544 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| return instance id |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | INSTANCE_ID |  |  | rw | ro |  | 0x0 |  |  | return allocation instance id (Valid after create_instance command){has_reset=0} |  |  |  |  |  |

| 1.1.1.82.0 |  |  |  |  |  |  |  | RET_CMD_CQ_IN_TICK |  |  |  |  |  |  | 0x23C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 572 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CMD_CQ_IN_TICK |  |  | rw | ro |  | 0x0 |  |  | Tick when CMD enters CQ{has_reset=0} |  |  |  |  |  |

| 1.1.1.83.0 |  |  |  |  |  |  |  | RET_CMD_FW_RUN_TICK |  |  |  |  |  |  | 0x240 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 576 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CMD_FW_RUN_TICK |  |  | rw | ro |  | 0x0 |  |  | Tick when CMD comes out of CQ (De-queue){has_reset=0} |  |  |  |  |  |

| 1.1.1.84.0 |  |  |  |  |  |  |  | RET_CMD_HW_RUN_TICK |  |  |  |  |  |  | 0x244 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 580 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CMD_HW_RUN_TICK |  |  | rw | ro |  | 0x0 |  |  | Tick when giving pic_run from FW to HW{has_reset=0} |  |  |  |  |  |

| 1.1.1.85.0 |  |  |  |  |  |  |  | RET_CMD_HW_DONE_TICK |  |  |  |  |  |  | 0x248 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 584 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CMD_HW_DONE_TICK |  |  | rw | ro |  | 0x0 |  |  | Tick when receiving pic_done from HW to FW{has_reset=0} |  |  |  |  |  |

| 1.1.1.86.0 |  |  |  |  |  |  |  | RET_CMD_FW_DONE_TICK |  |  |  |  |  |  | 0x24C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 588 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CMD_FW_DONE_TICK |  |  | rw | ro |  | 0x0 |  |  | Tick when report enters RQ{has_reset=0} |  |  |  |  |  |

| 1.1.1.87.0 |  |  |  |  |  |  |  | RET_CMD_RQ_OUT_TICK |  |  |  |  |  |  | 0x250 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 592 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CMD_RQ_OUT_TICK |  |  | rw | ro |  | 0x0 |  |  | Tick when report comes out of RQ{has_reset=0} |  |  |  |  |  |

| 1.1.1.88.0 |  |  |  |  |  |  |  | RET_CMD_FW_RRE_RUN_TICK |  |  |  |  |  |  | 0x254 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 596 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CMD_FW_PRE_RUN_TICK |  |  | rw | ro |  | 0x0 |  |  | Tick when CMD comes out of CQ (De-queue){has_reset=0} |  |  |  |  |  |

| 1.1.1.89.0 |  |  |  |  |  |  |  | RET_CMD_HW_RRE_RUN_TICK |  |  |  |  |  |  | 0x258 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 600 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CMD_HW_PRE_RUN_TICK |  |  | rw | ro |  | 0x0 |  |  | Tick when giving pre_run from FW to HW{has_reset=0} |  |  |  |  |  |

| 1.1.1.90.0 |  |  |  |  |  |  |  | RET_CMD_HW_RRE_DONE_TICK |  |  |  |  |  |  | 0x25C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 604 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CMD_HW_RRE_DONE_TICK |  |  | rw | ro |  | 0x0 |  |  | Tick when receiving pre_run done from HW to FW{has_reset=0} |  |  |  |  |  |

| 1.1.1.92.0 |  |  |  |  |  |  |  | RET_CMD_FW_RRE_DONE_TICK |  |  |  |  |  |  | 0x260 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 608 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CMD_FW_PRE_DONE_TICK |  |  | rw | ro |  | 0x0 |  |  | Tick when giving pic_run from FW to Pre HW{has_reset=0} |  |  |  |  |  |

| 1.1.1.102.0 |  |  |  |  |  |  |  | CMD_BS_RD_PTR |  |  |  |  |  |  | 0x300 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 768 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Bitstream buffer read pointer{alt_reg=CMD_BS_RD_PTR_DEC_PIC,CMD_BS_START,CMD_CREATE_INST_ADDR_WORK_BUF,CMD_ENC_SEQ_SET_PARAM_ENABLE,CMD_SET_FB_COMMON_PIC_INFO,RET_DEC_ADDR_USERDATA_BASE,RET_ENC_RD_PTR,RET_ENC_RD_PTR_QUERY_ENC_FW_STATUS,RET_QUERY_ENC_BS_RD_PTR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | RD_PTR |  |  | rw | ro |  | 0x0 |  |  | Start address of bitstream for handling current command Host processor cannot update this register in the middle of decoding. It is only allowed to update this before decoding has started. VPU also updates this (RET_ADDR_BS_RD_PTR) with end address of a NAL, when a NAL is decoded.{has_reset=0} |  |  |  |  |  |

| 1.1.1.112.0 |  |  |  |  |  |  |  | CMD_BS_SIZE |  |  |  |  |  |  | 0x304 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 772 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Bitstream buffer size{alt_reg=CMD_BS_WR_PTR,CMD_BS_WR_PTR_DEC_PIC,CMD_CREATE_INST_WORK_BUF_SIZE,CMD_ENC_SEQ_SRC_SIZE,CMD_SET_FB_PIC_SIZE,RET_DEC_USERDATA_SIZE,RET_ENC_WR_PTR,RET_ENC_WR_PTR_QUERY_ENC_FW_STATUS,RET_QUERY_ENC_BS_WR_PTR} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | BS_BUF_SIZE |  |  | rw | ro |  | 0x0 |  |  | Size of bitstream buffer It should be aligned in bus width (128bit/16 byte), but we highly recommend to be aligned in 4KB{has_reset=0} |  |  |  |  |  |

| 1.1.1.118.0 |  |  |  |  |  |  |  | CMD_BS_OPTION |  |  |  |  |  |  | 0x308 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 776 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Bitstream buffer option{alt_reg=CMD_BS_OPTIONS,CMD_BS_OPTIONS_ENC_PIC,CMD_ENC_SEQ_CUSTOM_MAP_ENDIAN,CMD_SET_FB_NUM_FB,RET_ENC_NUM_REQUIRED_FB} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:8 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 7:6 | STREAM_FORMAT |  |  | rw | ro |  | 0x0 |  |  | Refer. to STREAM_FORMAT on INIT_SEQ{has_reset=0} |  |  |  |  |  |
| 5:5 | STREAM_END |  |  | rw | ro |  | 0x0 |  |  | Refer. to STREAM_END on INIT_SEQ{has_reset=0} |  |  |  |  |  |
| 4:4 | EXPLICIT_END |  |  | rw | ro |  | 0x0 |  |  | Refer. to EXPLICIT_END on INIT_SEQ{has_reset=0} |  |  |  |  |  |
| 3:0 | BS_ENDIAN |  |  | rw | ro |  | 0x0 |  |  | Endianness of bitstream buffer{has_reset=0} |  |  |  |  |  |

| 1.1.1.124.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_PARAM |  |  |  |  |  |  | 0x30C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 780 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Encoder sequence parameters (MANDATORY){alt_reg=CMD_SET_FB_FBC_STRIDE,CMD_USE_SEC_AXI,CMD_USE_SEC_AXI_ENC_PIC,RET_DEC_BS_RD_PTR,RET_MIN_SRC_BUF_NUM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:31 | USE_DEFAULT_SCALING_LIST |  |  | rw | ro |  | 0x0 |  |  | It enables to apply default scaling list.{has_reset=0} |  |  |  |  |  |
| 30:30 | STILL_PICTURE |  |  | rw | ro |  | 0x0 |  |  | Still_profile indicator (HEVC) 0 : disable still_profile encoding (default) 1 : enable main/main10_still_profile encoding according to bit_depth{has_reset=0} |  |  |  |  |  |
| 29:28 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved {has_reset=0} |  |  |  |  |  |
| 27:27 | USE_STRONG_INTRA |  |  | rw | ro |  | 0x0 |  |  | It enables strong intra smoothing.{has_reset=0} |  |  |  |  |  |
| 26:25 | USE_INTRA_TRANS_SKIP |  |  | rw | ro |  | 0x0 |  |  | It enables transform skip for intra CU.{has_reset=0} |  |  |  |  |  |
| 24:24 | USE_SAO |  |  | rw | ro |  | 0x0 |  |  | It enables sample adaptive offset. (HEVC) It enables wiener. (AV1){has_reset=0} |  |  |  |  |  |
| 23:23 | USE_TMVP |  |  | rw | ro |  | 0x0 |  |  | It enables temporal motion vector prediction. (HEVC) It enables CDEF. (AV1){has_reset=0} |  |  |  |  |  |
| 22:22 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 21:21 | USE_LONG_TERM |  |  | rw | ro |  | 0x0 |  |  | It enables to use longterm reference frame.{has_reset=0} |  |  |  |  |  |
| 20:19 | RSVD2 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 18:14 | BIT_DEPTH |  |  | rw | ro |  | 0x0 |  |  | Bit depth (8 or 10){has_reset=0} |  |  |  |  |  |
| 13:12 | TIER_IDC |  |  | rw | ro |  | 0x0 |  |  | A tier indicator (HEVC only) 0 : main 1 : high{has_reset=0} |  |  |  |  |  |
| 11:3 | LEVEL_IDC |  |  | rw | ro |  | 0x0 |  |  | 0 : VPU decides a proper level value. Non-0 :host input value determines a level value as below: For AVC, host inputs the level value 10. For HEVC, host inputs the level value 30. e.g.) If you want for the level value to be set as 5, for AVC, input 5*10 = 50 as host level value. for HEVC, input 5*30 = 150 as host level value{has_reset=0} |  |  |  |  |  |
| 2:0 | PROFILE_IDC |  |  | rw | ro |  | 0x0 |  |  | Profile indicator (HEVC only) 0 : VPU decides whether the profile is main or main10 according to bit_depth. 1 : main 2 : main10 3 : main still picture (In case of AV1, the profile is always "main". In case of AVC, the profile is decided by VPU according to "USE_ENTROPY_CODING_MODE" and "USE_TRANSFORM_8x8"){has_reset=0} |  |  |  |  |  |

| 1.1.1.131.0 |  |  |  |  |  |  |  | CMD_CREATE_INST_BS_PARAM |  |  |  |  |  |  | 0x310 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 784 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Bitstream buffer option{alt_reg=CMD_ENC_REPORT_PARAM,CMD_ENC_SEQ_PPS_PARAM,CMD_SEQ_CHANGE_ENABLE_FLAG,CMD_SET_FB_ADDR_LUMA_BASE0,RET_DEC_SEQ_PARAM,RET_ENC_PIC_TYPE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:6 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 5:4 | BS_MODE |  |  | rw | ro |  | 0x0 |  |  | 0x0: Line buffer mode{has_reset=0} |  |  |  |  |  |
| 3:0 | BS_ENDIAN |  |  | rw | ro |  | 0x0 |  |  | Endianness of bitstream buffer{has_reset=0} |  |  |  |  |  |

| 1.1.1.136.0 |  |  |  |  |  |  |  | CMD_DEC_SEI_MASK |  |  |  |  |  |  | 0x314 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 788 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| User data dump option{alt_reg=CMD_ENC_SEQ_GOP_PARAM,CMD_SET_FB_ADDR_CB_BASE0,RET_DEC_COLOR_SAMPLE_INFO,RET_ENC_PIC_POC} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:17 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 16:16 | SUFFIX_SEI_COLOUR_REAMPPING_INFO |  |  | rw | ro |  | 0x0 |  |  | HEVC : Enable to report colour_remapping_ info( ) PREFIX_SEI message. AVC : Enable to report colour_remapping_ info( ) SEI message.{has_reset=0} |  |  |  |  |  |
| 15:15 | PREFIX_SEI_CONTENT_LIGHT_LEVEL_INFO |  |  | rw | ro |  | 0x0 |  |  | HEVC : Enable to report content_light_level_ info( ) PREFIX_SEI message. AVC : Reserved{has_reset=0} |  |  |  |  |  |
| 14:14 | PREFIX_SEI_FILM_GRAIN_CHARACTERISTICS_INFO |  |  | rw | ro |  | 0x0 |  |  | HEVC : Enable to report film_grain_ characteristics_ info() message. AVC : Enable to report film_grain_ characteristics_ info() message.{has_reset=0} |  |  |  |  |  |
| 13:13 | PREFIX_SEI_TONE_MAPPING_INFO |  |  | rw | ro |  | 0x0 |  |  | HEVC : Enable to report tone_mapping_info PREFIX_SEI message. AVC : Enable to report tone_mapping_info SEI message.{has_reset=0} |  |  |  |  |  |
| 12:12 | PREFIX_SEI_KNEE_FUNCTION_INFO |  |  | rw | ro |  | 0x0 |  |  | HEVC : Enable to report knee_function_ info( ) PREFIX_SEI message. AVC : Reserved{has_reset=0} |  |  |  |  |  |
| 11:11 | PREFIX_SEI_CHROMA_RESAMPLING_FILTER_HINT |  |  | rw | ro |  | 0x0 |  |  | HEVC : Enable to report chroma_ resampling_filter_ hint( ) PREFIX_SEI message. AVC : Reserved{has_reset=0} |  |  |  |  |  |
| 10:10 | PREFIX_SEI_MASTERING_DISPLAY_COLOUR_VOLUME |  |  | rw | ro |  | 0x0 |  |  | HEVC : Enable to report mastering_display_ colour_volume( ) PREFIX_SEI message. AVC : Reserved{has_reset=0} |  |  |  |  |  |
| 9:9 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 8:8 | SUFFIX_SEI_USER_DATA_UNREGISTERED |  |  | rw | ro |  | 0x0 |  |  | HEVC : Enable to report user_data_ unregistered( ) SUFFIX_SEI message. AVC : Reserved{has_reset=0} |  |  |  |  |  |
| 7:7 | SUFFIX_SEI_USER_DATA_REGISTERED_ITU_T_T35_0 |  |  | rw | ro |  | 0x0 |  |  | HEVC : Enable to report the 1st user_data_ registered_itu_t_ t35( ) SUFFIX_SEI message. AVC : Reserved{has_reset=0} |  |  |  |  |  |
| 6:6 | PREFIX_SEI_USER_DATA_UNREGISTERED |  |  | rw | ro |  | 0x0 |  |  | HEVC : Enable to report user_data_ unregistered( ) PREFIX_SEI message. AVC : Enable to report user_data_unregistered() SEI message{has_reset=0} |  |  |  |  |  |
| 5:5 | PREFIX_SEI_USER_DATA_REGISTERED_ITU_T_T35_0 |  |  | rw | ro |  | 0x0 |  |  | HEVC : Enable to report the 1st user_data_ registered_itu_t_ t35( ) PREFIX_SEI message. AVC : Enable to report the user_data_registered_itu_t_t35() SEI message{has_reset=0} |  |  |  |  |  |
| 4:4 | PREFIX_SEI_PIC_TIMING |  |  | rw | ro |  | 0x0 |  |  | HEVC : Enable to report pic_timing( ) PREFIX_SEI message. AVC : Enable to report pic_timing( ) SEI message.{has_reset=0} |  |  |  |  |  |
| 3:3 | RSVD2 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 2:2 | REPORT_VUI |  |  | rw | ro |  | 0x0 |  |  | HEVC : Enable to report VUI. AVC : Enable to report VUI{has_reset=0} |  |  |  |  |  |
| 1:0 | RSVD3 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |

| 1.1.1.145.0 |  |  |  |  |  |  |  | CMD_CREATE_ADDR_EXT |  |  |  |  |  |  | 0x318 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 792 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_CREATE_ADDR_EXT{alt_reg=CMD_DEC_TEMPORAL_ID_PLUS1,CMD_ENC_MV_HISTO_CLASS0,CMD_ENC_SEQ_INTRA_PARAM,CMD_ENC_SEQ_INTRA_PARAM_AVC,CMD_SET_FB_ADDR_CR_BASE0,CMD_SET_FB_ADDR_FBC_Y_OFFSET0,RET_DEC_ASPECT_RATIO,RET_ENC_PIC_IDX} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:8 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 7:0 | EXT_ADDR |  |  | rw | ro |  | 0x0 |  |  | Primary AXI address extension{has_reset=0} |  |  |  |  |  |

| 1.1.1.151.0 |  |  |  |  |  |  |  | CMD_DEC_FORCE_FB_LATENCY_PLUS1 |  |  |  |  |  |  | 0x31C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 796 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| User define display latency{alt_reg=CMD_ENC_MV_HISTO_CLASS1,CMD_ENC_SEQ_CONF_WIN_TOP_BOT,CMD_SET_FB_ADDR_FBC_C_OFFSET0,RET_DEC_BIT_RATE,RET_ENC_PIC_SLICE_NUM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 4:0 | USER_DEF_DISP_LATENCY_PLUS1 |  |  | rw | ro |  | 0x0 |  |  | Change frame buffer display latency by force. 0x0 : not used 0x1~0x1f : latency + 1 (if this is set to 1, that means latency is 0 - immediate out){has_reset=0} |  |  |  |  |  |

| 1.1.1.156.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_MAP_OPTION_PARAM |  |  |  |  |  |  | 0x320 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 800 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Custom map parameters{alt_reg=CMD_ENC_SEQ_CONF_WIN_LEFT_RIGHT,CMD_SET_FB_ADDR_COL0,RET_DEC_FRAME_RATE_NR,RET_ENC_PIC_SKIP} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:2 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 1:1 | USE_CTU_FORCE_MODE_FLAG |  |  | rw | ro |  | 0x0 |  |  | It enables to force intra or, force skip in block unit.{has_reset=0} |  |  |  |  |  |
| 0:0 | USE_QP_MAP_FLAG |  |  | rw | ro |  | 0x0 |  |  | It enables to use QP map.{has_reset=0} |  |  |  |  |  |

| 1.1.1.161.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_MAP_OPTION_ADDR |  |  |  |  |  |  | 0x324 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 804 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Custom map address{alt_reg=CMD_ENC_SEQ_RDO_PARAM,CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE0,RET_DEC_FRAME_RATE_DR,RET_ENC_PIC_NUM_INTRA} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CUSTOM_MAP_OPTION_ADDR |  |  | rw | ro |  | 0x0 |  |  | Address of custom map{has_reset=0} |  |  |  |  |  |

| 1.1.1.165.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_SLICE_PARAM |  |  |  |  |  |  | 0x328 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 808 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Slice parameters{alt_reg=CMD_SET_FB_ADDR_LUMA_BASE1,RET_DEC_NUM_REQURED_FB,RET_ENC_PIC_NUM_MERGE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:3 | ARGUMENT |  |  | rw | ro |  | 0x0 |  |  | It specifies a size of slice (2^29 - 1) The number of CTU in case of SLICE_MODE = 1{has_reset=0} |  |  |  |  |  |
| 2:0 | SLICE_MODE |  |  | rw | ro |  | 0x0 |  |  | A slice mode 0 - Single slice 1 - CTU based slice 2 - ROW based Slice{has_reset=0} |  |  |  |  |  |

| 1.1.1.170.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_INTRA_REFRESH |  |  |  |  |  |  | 0x32C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 812 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Intra refresh mode{alt_reg=CMD_ENC_SRC_PIC_IDX,CMD_SET_FB_ADDR_CB_BASE1,RET_DEC_NUM_REORDER_DELAY,RET_ENC_PIC_NON_REF_PIC_FLAG} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | INTRA_REFRESH_ARGUMENT |  |  | rw | ro |  | 0x0 |  |  | It specifies an intra CTU refresh interval. Depending on intraRefreshMode, it can mean one of the followings: The number of consecutive CTU rows for IntraCtuRefreshMode of 1 The number of consecutive CTU columns for IntraCtuRefreshMode of 2 A step size in CTU for IntraCtuRefreshMode of 3 The number of Intra CTUs to be encoded in a picture for IntraCtuRefreshMode of 4{has_reset=0} |  |  |  |  |  |
| 15:3 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 2:0 | INTRA_REFRESH_MODE |  |  | rw | ro |  | 0x0 |  |  | An intra refresh mode 0 : No intra refresh 1 : Row 2 : Column 3 : A step size in CTU{has_reset=0} |  |  |  |  |  |

| 1.1.1.177.0 |  |  |  |  |  |  |  | CMD_CREATE_INST_CORE_INFO |  |  |  |  |  |  | 0x330 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 816 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_CREATE_INST_CORE_INFO{alt_reg=CMD_ENC_SEQ_INTRA_MIN_MAX_QP,CMD_ENC_SRC_ADDR_Y,CMD_SET_FB_ADDR_CR_BASE1,CMD_SET_FB_ADDR_FBC_Y_OFFSET1,RET_DEC_SUB_LAYER_INFO,RET_ENC_PIC_NUM_SKIP} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:11 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 10:8 | SET_CQ_DEPTH |  |  | rw | ro |  | 0x0 |  |  | Set command queue depth Minimum : 1 Maximim : 6 Default : 4{has_reset=0} |  |  |  |  |  |
| 7:4 | CORE_IDC |  |  | rw | ro |  | 0x0 |  |  | Core indicator to be used for the instance. 0 : Firmware allocates a core idc for the instance. (default) 1 : Core 0 is used for the instance when NUM_CORE is 1. (only valid for dual core products) 2 : Core 1 is used for the instance when NUM_CORE is 1. (only valid for dual core products){has_reset=0} |  |  |  |  |  |
| 3:0 | NUM_CORE |  |  | rw | ro |  | 0x0 |  |  | Number of cores to be used for the instance. 0 : N/A 1 : Use single-core. 2 : Use dual-cores (a frame parallel processing is used.){has_reset=0} |  |  |  |  |  |

| 1.1.1.183.0 |  |  |  |  |  |  |  | CMD_CREATE_INST_PRIORITY |  |  |  |  |  |  | 0x334 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 820 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_CREATE_INST_PRIORITY{alt_reg=CMD_ENC_SEQ_RC_FRAME_RATE,CMD_ENC_SRC_ADDR_U,CMD_SET_FB_ADDR_FBC_C_OFFSET1,RET_DEC_NOTIFICATION,RET_ENC_PIC_AVG_CTU_QP} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:9 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 8:8 | SECURITY_FLAG |  |  | rw | ro |  | 0x0 |  |  | security flag (0: non-secure instance / 1: secure instance) - AXI AxProt[1] bit is set to an inverted value.{has_reset=0} |  |  |  |  |  |
| 7:5 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 4:0 | PRIORITY_VALUE |  |  | rw | ro |  | 0x0 |  |  | priority value (low priority 0~31 high priority){has_reset=0} |  |  |  |  |  |

| 1.1.1.188.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_RC_TARGET_RATE |  |  |  |  |  |  | 0x338 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 824 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_RC_TARGET_RATE{alt_reg=CMD_ENC_SRC_ADDR_V,CMD_SET_FB_ADDR_COL1,RET_DEC_USERDATA_IDC,RET_ENC_PIC_BYTE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | RC_TARGET_RATE |  |  | rw | ro |  | 0x0 |  |  | A target bitrate for for rate control{has_reset=0} |  |  |  |  |  |

| 1.1.1.193.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_RC_PARAM |  |  |  |  |  |  | 0x33C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 828 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_RC_PARAM{alt_reg=CMD_ENC_SRC_STRIDE,CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE1,RET_DEC_PIC_SIZE,RET_ENC_GOP_PIC_IDX} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:24 | RC_UPDATE_SPEED |  |  | rw | ro |  | 0x0 |  |  | rc update speed{has_reset=0} |  |  |  |  |  |
| 23:20 | RC_INITIAL_LEVEL |  |  | rw | ro |  | 0x0 |  |  | initial rc buffer level{has_reset=0} |  |  |  |  |  |
| 19:14 | RC_INITIAL_QP |  |  | rw | ro |  | 0x0 |  |  | An initial QP If this value is smaller than 0 or larger than 51, firmware decides the initial QP. [NOTE] This parameter cannot be changed within the same sequence.�{has_reset=0} |  |  |  |  |  |
| 13:13 | RC_MODE |  |  | rw | ro |  | 0x0 |  |  | Rate control mode 0 : VBR 1 : CBR{has_reset=0} |  |  |  |  |  |
| 12:7 | PIC_RC_MAX_DELTA_QP |  |  | rw | ro |  | 0x0 |  |  | Max delta QP for picture level RC{has_reset=0} |  |  |  |  |  |
| 6:2 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 1:1 | EN_CU_LEVEL_RC |  |  | rw | ro |  | 0x0 |  |  | It enables CU level rate control. [NOTE] This parameter cannot be changed within the same sequence.{has_reset=0} |  |  |  |  |  |
| 0:0 | EN_RATE_CONTROL |  |  | rw | ro |  | 0x0 |  |  | It enables rate control. [NOTE] This parameter cannot be changed within the same sequence.{has_reset=0} |  |  |  |  |  |

| 1.1.1.198.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_HVS_PARAM |  |  |  |  |  |  | 0x340 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 832 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_HVS_PARAM{alt_reg=CMD_ENC_SRC_FORMAT,CMD_SET_FB_ADDR_LUMA_BASE2,RET_DEC_CROP_TOP_BOTTOM,RET_ENC_USED_SRC_IDX} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:18 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 17:12 | HVS_MAX_DELTA_QP |  |  | rw | ro |  | 0x0 |  |  | Specifies maximum delta QP from CTU QP. (only valid when CULevelRateControl is 1.) (0 ~ 12){has_reset=0} |  |  |  |  |  |
| 11:4 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 3:0 | HVS_QP_SCALE |  |  | rw | ro |  | 0x0 |  |  | Specifies the scale for CU QP derivation. Only available when EnHvsQp is 1. (1 ~ 4){has_reset=0} |  |  |  |  |  |

| 1.1.1.202.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_RC_MAX_BITRATE |  |  |  |  |  |  | 0x344 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 836 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_RC_MAX_BITRATE{alt_reg=CMD_SET_FB_ADDR_CB_BASE2,RET_DEC_CROP_LEFT_RIGHT,RET_ENC_PIC_NUM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | RC_MAX_BITRATE |  |  | rw | ro |  | 0x0 |  |  | A maximum bitrate for for rate control{has_reset=0} |  |  |  |  |  |

| 1.1.1.209.0 |  |  |  |  |  |  |  | CMD_CREATE_INST_ADDR_TEMP_BASE |  |  |  |  |  |  | 0x348 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 840 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Temporal buffer base address{alt_reg=CMD_ENC_SEQ_RC_VBV_BUFFER_SIZE,CMD_ENC_SRC_AXI_SEL,CMD_SET_FB_ADDR_CR_BASE2,CMD_SET_FB_ADDR_FBC_Y_OFFSET2,RET_DEC_AU_START_POS,RET_ENC_VCL_NUT} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | TEMP_BUF_BASE |  |  | rw | ro |  | 0x0 |  |  | Base address of temporal buffer for this frame [NOTE] Each V-CORE should set this field for its own temporal buffer.{has_reset=0} |  |  |  |  |  |

| 1.1.1.215.0 |  |  |  |  |  |  |  | CMD_CREATE_INST_TEMP_SIZE |  |  |  |  |  |  | 0x34C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 844 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Temporal buffer size{alt_reg=CMD_ENC_CODE_OPTION,CMD_ENC_SEQ_INTER_MIN_MAX_QP,CMD_SET_FB_ADDR_FBC_C_OFFSET2,RET_DEC_AU_END_POS,RET_ENC_PIC_PADD_BYTE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | TEMP_BUF_SIZE |  |  | rw | ro |  | 0x0 |  |  | Temporal buffer size for a frame{has_reset=0} |  |  |  |  |  |

| 1.1.1.221.0 |  |  |  |  |  |  |  | CMD_CREATE_INST_ADDR_SEC_AXI |  |  |  |  |  |  | 0x350 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 848 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Secondary AXI base address It is valid when USE_SEC_AXI is not 0. {alt_reg=CMD_ENC_PIC_PARAM,CMD_ENC_SEQ_ROT_PARAM,CMD_SET_FB_ADDR_COL2,RET_DEC_PIC_TYPE,RET_ENC_PIC_DIST_LOW} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SEC_AXI_BASE |  |  | rw | ro |  | 0x0 |  |  | The base address of secondary AXI memory{has_reset=0} |  |  |  |  |  |

| 1.1.1.227.0 |  |  |  |  |  |  |  | CMD_CREATE_INST_SEC_AXI_SIZE |  |  |  |  |  |  | 0x354 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 852 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Seconary AXI memory size It is valid when USE_SEC_AXI is not 0. {alt_reg=CMD_ENC_LONGTERM_PIC,CMD_ENC_SEQ_NUM_UNITS_IN_TICK,CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE2,RET_DEC_PIC_POC,RET_ENC_PIC_DIST_HIGH} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SEC_AXI_MEM_SIZE |  |  | rw | ro |  | 0x0 |  |  | The size of secondary AXI temporal buffer{has_reset=0} |  |  |  |  |  |

| 1.1.1.233.0 |  |  |  |  |  |  |  | CMD_CREATE_INST_ADDR_AR_TABLE |  |  |  |  |  |  | 0x358 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 856 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| adaptive roud base address{alt_reg=CMD_ENC_PREFIX_SEI_NAL_ADDR,CMD_ENC_SEQ_TIME_SCALE,CMD_SET_FB_ADDR_LUMA_BASE3,RET_DEC_RECOVERY_POINT,RET_ENC_MAX_LATENCY_PICTURES} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | AR_TABLE_BASE |  |  | rw | ro |  | 0x0 |  |  | The base address of adaptive round table{has_reset=0} |  |  |  |  |  |

| 1.1.1.237.0 |  |  |  |  |  |  |  | CMD_ENC_PREFIX_SEI_INFO |  |  |  |  |  |  | 0x35C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 860 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_PREFIX_SEI_INFO{alt_reg=CMD_ENC_SEQ_NUM_TICKS_POC_DIFF_ONE,CMD_SET_FB_ADDR_CB_BASE3,RET_DEC_DEBUG_INDEX} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | NAL_SIZE |  |  | rw | ro |  | 0x0 |  |  | The total byte size of the prefix SEI (Max. size = 4096){has_reset=0} |  |  |  |  |  |
| 15:1 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 0:0 | NAL_ENABLE |  |  | rw | ro |  | 0x0 |  |  | It enables to encode the prefix SEI NAL which is given by host. HEVC: prefix sei AVC: sei AV1: metadata{has_reset=0} |  |  |  |  |  |

| 1.1.1.242.0 |  |  |  |  |  |  |  | CMD_ENC_SUFFIX_SEI_NAL_ADDR |  |  |  |  |  |  | 0x360 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 864 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SUFFIX_SEI_NAL_ADDR{alt_reg=CMD_SET_FB_ADDR_CR_BASE3,CMD_SET_FB_ADDR_FBC_Y_OFFSET3,RET_DEC_DECODED_INDEX,RET_ENC_HISTO_CNT_0} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SUFFIX_SEI_NAL_ADDR |  |  | rw | ro |  | 0x0 |  |  | The base address of the SUFFIX SEI NAL buffer (Big endian){has_reset=0} |  |  |  |  |  |

| 1.1.1.246.0 |  |  |  |  |  |  |  | CMD_ENC_SUFFIX_SEI_INFO |  |  |  |  |  |  | 0x364 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 868 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SUFFIX_SEI_INFO{alt_reg=CMD_SET_FB_ADDR_FBC_C_OFFSET3,RET_DEC_DISPLAY_INDEX,RET_ENC_HISTO_CNT_1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | NAL_SIZE |  |  | rw | ro |  | 0x0 |  |  | The total byte size of the suffix SEI (Max. size = 4096){has_reset=0} |  |  |  |  |  |
| 15:1 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 0:0 | NAL_ENABLE |  |  | rw | ro |  | 0x0 |  |  | It enables to encode the suffix SEI NAL which is given by host. (HEVC - suffix sei){has_reset=0} |  |  |  |  |  |

| 1.1.1.249.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL3 |  |  |  |  |  |  | 0x368 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 872 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index 3{alt_reg=RET_DEC_REALLOC_INDEX,RET_ENC_HISTO_CNT_2} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE3 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.253.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_BG_PARAM |  |  |  |  |  |  | 0x36C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 876 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_BG_PARAM{alt_reg=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE3,RET_DEC_DISP_IDC,RET_ENC_HISTO_CNT_3} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:24 | BG_DELTA_QP |  |  | rw | ro |  | 0x0 |  |  | It specifies the difference between the lambda QP value of background and the lambda QP value of foreground. (-16 ~ 15){has_reset=0} |  |  |  |  |  |
| 23:18 | RSVD1 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 17:10 | BG_TH_MEAN_DIFF |  |  | rw | ro |  | 0x0 |  |  | It specifies the threshold of mean difference that is used in s2me block. It is valid when background detection is on. (0 ~ 255){has_reset=0} |  |  |  |  |  |
| 9:1 | BG_TH_MAX_DIFF |  |  | rw | ro |  | 0x0 |  |  | It specifies the threshold of max difference that is used in s2me block. It is valid when background detection is on. (0 ~ 255){has_reset=0} |  |  |  |  |  |
| 0:0 | EN_BG_DETECT |  |  | rw | ro |  | 0x0 |  |  | It enables background detection.{has_reset=0} |  |  |  |  |  |

| 1.1.1.257.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_NON_VCL_PARAM |  |  |  |  |  |  | 0x370 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 880 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_NON_VCL_PARAM{alt_reg=CMD_SET_FB_ADDR_LUMA_BASE4,RET_DEC_NUM_ERR_CTB,RET_ENC_HISTO_CNT_4} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:18 | HRD_RBSP_SIZE |  |  | rw | ro |  | 0x0 |  |  | The bit length of HRD RBSP data (Big endian) (HEVC) (Max size=4096){has_reset=0} |  |  |  |  |  |
| 17:4 | VUI_RBSP_SIZE |  |  | rw | ro |  | 0x0 |  |  | The bit length of VUI RBSP data (Big endian) (HEVC, AVC) (Max size=4096){has_reset=0} |  |  |  |  |  |
| 3:3 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 2:2 | ENCODE_HRD_RBSP |  |  | rw | ro |  | 0x0 |  |  | A flag to encode the HRD syntax RBSP which is given by host into VPS{has_reset=0} |  |  |  |  |  |
| 1:1 | ENCODE_VUI_RBSP |  |  | rw | ro |  | 0x0 |  |  | A flag to encode the VUI syntax RBSP which is given by host{has_reset=0} |  |  |  |  |  |
| 0:0 | ENCODE_AUD |  |  | rw | ro |  | 0x0 |  |  | A flag to encode the AUD NAL (HEVC, AVC){has_reset=0} |  |  |  |  |  |

| 1.1.1.261.0 |  |  |  |  |  |  |  | CMD_ENC_CSC_COEFF_0 |  |  |  |  |  |  | 0x374 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 884 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| RGB to YUV color space conversion coefficient register Coef and offset values can be freely set through the registers below. However, we guarantee only for the 4 format mentioned below. Y = ry_coef*R + gy_coef*G + by_coef*B + y offset Cb = rcb_coef*R + rcb_coef*G + gcb_coef*B + cb offset Cr = rcr_coef*R + gcr_coef*G + bcr_coef*B + cr offset {alt_reg=CMD_ENC_SEQ_VUI_RBSP_ADDR,CMD_SET_FB_ADDR_CB_BASE4,RET_ENC_NUM_TILE_COL} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:20 | CSC_Coeff_RY |  |  | rw | ro |  | 0x0 |  |  | range : -512 ~511 examples: BT.601 : 10'h0x99 BT.601 Studio Swing : 10'h0x84 BT.709 : 10'h0x6d BT.2020 : 10'h0x87{has_reset=0} |  |  |  |  |  |
| 19:10 | CSC_Coeff_GY |  |  | rw | ro |  | 0x0 |  |  | range : -512 ~511 examples: BT.601 : 10'h0x12d BT.601 Studio Swing : 10'h0x102 BT.709 : 10'h0x16e BT.2020 : 10'h0x15b{has_reset=0} |  |  |  |  |  |
| 9:0 | CSC_Coeff_BY |  |  | rw | ro |  | 0x0 |  |  | range : -512 ~511 examples: BT.601 : 10'h0x3A BT.601 Studio Swing : 10'h0x32 BT.709 : 10'h0x25 BT.2020 : 10'h0x1e{has_reset=0} |  |  |  |  |  |

| 1.1.1.266.0 |  |  |  |  |  |  |  | CMD_ENC_CSC_COEFF_1 |  |  |  |  |  |  | 0x378 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 888 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| RGB to YUV color space conversion coefficient register {alt_reg=CMD_ENC_SEQ_HRD_RBSP_ADDR,CMD_SET_FB_ADDR_CR_BASE4,CMD_SET_FB_ADDR_FBC_Y_OFFSET4,RET_ENC_NUM_TILE_ROW} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:20 | CSC_Coeff_RCB |  |  | rw | ro |  | 0x0 |  |  | range : -512 ~511 examples: BT.601 : 10'h0x3aa BT.601 Studio Swing : 10'h0x3b4 BT.709 : 10'h0x3c5 BT.2020 : 10'h0x3b9{has_reset=0} |  |  |  |  |  |
| 19:10 | CSC_Coeff_GCB |  |  | rw | ro |  | 0x0 |  |  | range : -512 ~511 examples: BT.601 : 10'h0x356 BT.601 Studio Swing : 10'h0x36c BT.709 : 10'h0x33b BT.2020 : 10'h0x347{has_reset=0} |  |  |  |  |  |
| 9:0 | CSC_Coeff_BCB |  |  | rw | ro |  | 0x0 |  |  | range : -512 ~511 examples: BT.601 : 10'h0x100 BT.601 Studio Swing : 10'h0x0e0 BT.709 : 10'h0x100 BT.2020 : 10'h0x100{has_reset=0} |  |  |  |  |  |

| 1.1.1.269.0 |  |  |  |  |  |  |  | CMD_ENC_CSC_COEFF_2 |  |  |  |  |  |  | 0x37C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 892 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| RGB to YUV color space conversion coefficient register {alt_reg=CMD_SET_FB_ADDR_FBC_C_OFFSET4,RET_ENC_PIC_WIDTH} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:20 | CSC_Coeff_RCR |  |  | rw | ro |  | 0x0 |  |  | range : -512 ~511 examples: BT.601 : 10'h0x100 BT.601 Studio Swing : 10'h0x0e0 BT.709 : 10'h0x100 BT.2020 : 10'h0x100{has_reset=0} |  |  |  |  |  |
| 19:10 | CSC_Coeff_GCR |  |  | rw | ro |  | 0x0 |  |  | range : -512 ~511 examples: BT.601 : 10'h0x32a BT.601 Studio Swing : 10'h0x344 BT.709 : 10'h0x317 BT.2020 : 10'h0x315{has_reset=0} |  |  |  |  |  |
| 9:0 | CSC_Coeff_BCR |  |  | rw | ro |  | 0x0 |  |  | range : -512 ~511 examples: BT.601 : 10'h0x3d6 BT.601 Studio Swing : 10'h0x3dc BT.709 : 10'h0x3e9 BT.2020 : 10'h0x3eb{has_reset=0} |  |  |  |  |  |

| 1.1.1.273.0 |  |  |  |  |  |  |  | CMD_ENC_CSC_COEFF_3 |  |  |  |  |  |  | 0x380 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 896 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| RGB to YUV color space conversion coefficient register{alt_reg=CMD_ENC_SEQ_QROUND_OFFSET,CMD_SET_FB_ADDR_COL4,ENC_PIC_HEIGHT} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:20 | CSC_Offset_Y |  |  | rw | ro |  | 0x0 |  |  | range: 0 ~ 1023 examples: BT.601 : 10'h0x0 BT.601 Studio Swing : (bit_depth != 8) ? 10'h0x40 : 10'h0x10 BT.709 : 10'h0x0 BT.2020 : 10'h0x0{has_reset=0} |  |  |  |  |  |
| 19:10 | CSC_Offset_Cb |  |  | rw | ro |  | 0x0 |  |  | range: 0 ~ 1023 examples: BT.601 : (bit_depth != 8) ? 10'h0x200 : 10'h0x80 BT.601 Studio Swing : (bit_depth != 8) ? 10'h0x200 : 10'h0x80 BT.709 : (bit_depth != 8) ? 10'h0x200 : 10'h0x80 BT.2020 : (bit_depth != 8) ? 10'h0x200 : 10'h0x80{has_reset=0} |  |  |  |  |  |
| 9:0 | CSC_Offset_Cr |  |  | rw | ro |  | 0x0 |  |  | range: 0 ~ 1023 examples: BT.601 : (bit_depth != 8) ? 10'h0x200 : 10'h0x80 BT.601 Studio Swing : (bit_depth != 8) ? 10'h0x200 : 10'h0x80 BT.709 : (bit_depth != 8) ? 10'h0x200 : 10'h0x80 BT.2020 : (bit_depth != 8) ? 10'h0x200 : 10'h0x80{has_reset=0} |  |  |  |  |  |

| 1.1.1.275.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_QUANT_PARAM_0 |  |  |  |  |  |  | 0x384 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 900 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_QUANT_PARAM_0{alt_reg=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE4} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:24 | DELTA_QP_U_DC |  |  | rw | ro |  | 0x0 |  |  | It specifies the AV1 Qindex offset for DC of Cb component. (-64 ~ 63){has_reset=0} |  |  |  |  |  |
| 23:16 | DELTA_QP_V_DC |  |  | rw | ro |  | 0x0 |  |  | It specifies the AV1 Qindex offset for DC of Cr component. (-64 ~ 63){has_reset=0} |  |  |  |  |  |
| 15:8 | DELTA_QP_U_AC |  |  | rw | ro |  | 0x0 |  |  | It specifies the AV1 Qindex offset for AC of Cb component. (-64 ~ 63){has_reset=0} |  |  |  |  |  |
| 7:0 | DELTA_QP_V_AC |  |  | rw | ro |  | 0x0 |  |  | It specifies the AV1 Qindex offset for AC of Cr component. (-64 ~ 63){has_reset=0} |  |  |  |  |  |

| 1.1.1.277.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_QUANT_PARAM_1 |  |  |  |  |  |  | 0x388 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 904 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_QUANT_PARAM_1{alt_reg=CMD_SET_FB_ADDR_LUMA_BASE5} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:20 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 19:14 | LAMBDA_DELTA_QP_INTER |  |  | rw | ro |  | 0x0 |  |  | It specifies a lambda delta QP for inter pictures. (-32 ~ 31){has_reset=0} |  |  |  |  |  |
| 13:8 | LAMBDA_DELTA_QP_INTRA |  |  | rw | ro |  | 0x0 |  |  | It specifies a lambda delta QP for intra pictures. (-32 ~ 31){has_reset=0} |  |  |  |  |  |
| 7:0 | DELTA_QP_Y_DC |  |  | rw | ro |  | 0x0 |  |  | It specifies the AV1 Qindex offset for Luma DC. (-64 ~ 63){has_reset=0} |  |  |  |  |  |

| 1.1.1.279.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_CUSTOM_GOP_PARAM |  |  |  |  |  |  | 0x38C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 908 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Size of source pictures of custom GOP{alt_reg=CMD_SET_FB_ADDR_CB_BASE5} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:5 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 4:0 | CUSTOM_GOP_SIZE |  |  | rw | ro |  | 0x0 |  |  | The size of cyclic GOP (1 ~ 8) This should be the same as the number of CUSTOM_GOP_PIC_PARAM set by host{has_reset=0} |  |  |  |  |  |

| 1.1.1.282.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_0 |  |  |  |  |  |  | 0x390 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 912 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Parameters for the 0th picture of custom GOP (MANDATORY with CUSTOM_GOP_SIZE){alt_reg=CMD_SET_FB_ADDR_CR_BASE5,CMD_SET_FB_ADDR_FBC_Y_OFFSET5} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:26 | TEMPORAL_ID |  |  | rw | ro |  | 0x0 |  |  | A temporal ID of 0th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 25:20 | REF0_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L0 of 0th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 19:14 | REF1_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L1 of 0th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 13:13 | USE_MULTI_REF_P |  |  | rw | ro |  | 0x0 |  |  | Flag to use multi reference picture for P picture It is valid only if PIC_TYPE is P{has_reset=0} |  |  |  |  |  |
| 12:7 | PIC_QP |  |  | rw | ro |  | 0x0 |  |  | A quantization parameter of 0th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 6:2 | POC_OFFSET |  |  | rw | ro |  | 0x0 |  |  | A poc offset of 0th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 1:0 | PIC_TYPE |  |  | rw | ro |  | 0x0 |  |  | A picture type of 0th picture in the custom GOP 1 : P picture 2 : B picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.285.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_1 |  |  |  |  |  |  | 0x394 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 916 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Parameters for the 1st picture of custom GOP (MANDATORY with CUSTOM_GOP_SIZE){alt_reg=CMD_SET_FB_ADDR_FBC_C_OFFSET5,RET_DEC_NUM_REQURED_COL_FB} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:26 | TEMPORAL_ID |  |  | rw | ro |  | 0x0 |  |  | A temporal ID of 1st picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 25:20 | REF0_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L0 of 1st picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 19:14 | REF1_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L1 of 1st picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 13:13 | USE_MULTI_REF_P |  |  | rw | ro |  | 0x0 |  |  | Flag to use multi reference picture for P picture It is valid only if PIC_TYPE is P{has_reset=0} |  |  |  |  |  |
| 12:7 | PIC_QP |  |  | rw | ro |  | 0x0 |  |  | A quantization parameter of 1st picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 6:2 | POC_OFFSET |  |  | rw | ro |  | 0x0 |  |  | A poc offset of 1st picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 1:0 | PIC_TYPE |  |  | rw | ro |  | 0x0 |  |  | A picture type of 0th picture in the custom GOP 1 : P picture 2 : B picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.287.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_2 |  |  |  |  |  |  | 0x398 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 920 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Parameters for the 2nd picture of custom GOP (MANDATORY with CUSTOM_GOP_SIZE){alt_reg=CMD_SET_FB_ADDR_COL5} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:26 | TEMPORAL_ID |  |  | rw | ro |  | 0x0 |  |  | A temporal ID of 2nd picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 25:20 | REF0_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L0 of 2nd picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 19:14 | REF1_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L1 of 2nd picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 13:13 | USE_MULTI_REF_P |  |  | rw | ro |  | 0x0 |  |  | Flag to use multi reference picture for P picture It is valid only if PIC_TYPE is P{has_reset=0} |  |  |  |  |  |
| 12:7 | PIC_QP |  |  | rw | ro |  | 0x0 |  |  | A quantization parameter of 2nd picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 6:2 | POC_OFFSET |  |  | rw | ro |  | 0x0 |  |  | A poc offset of 2nd picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 1:0 | PIC_TYPE |  |  | rw | ro |  | 0x0 |  |  | A picture type of 0th picture in the custom GOP 1 : P picture 2 : B picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.289.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_3 |  |  |  |  |  |  | 0x39C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 924 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Parameters for the 3rd picture of custom GOP (MANDATORY with CUSTOM_GOP_SIZE){alt_reg=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE5} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:26 | TEMPORAL_ID |  |  | rw | ro |  | 0x0 |  |  | A temporal ID of 3rd picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 25:20 | REF0_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L0 of 3rd picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 19:14 | REF1_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L1 of 3rd picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 13:13 | USE_MULTI_REF_P |  |  | rw | ro |  | 0x0 |  |  | Flag to use multi reference picture for P picture It is valid only if PIC_TYPE is P{has_reset=0} |  |  |  |  |  |
| 12:7 | PIC_QP |  |  | rw | ro |  | 0x0 |  |  | A quantization parameter of 3rd picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 6:2 | POC_OFFSET |  |  | rw | ro |  | 0x0 |  |  | A poc offset of 3rd picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 1:0 | PIC_TYPE |  |  | rw | ro |  | 0x0 |  |  | A picture type of 0th picture in the custom GOP 1 : P picture 2 : B picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.292.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_4 |  |  |  |  |  |  | 0x3A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 928 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Parameters for the 4th picture of custom GOP (MANDATORY with CUSTOM_GOP_SIZE){alt_reg=CMD_SET_FB_ADDR_LUMA_BASE6,RET_CORE_IDC} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:26 | TEMPORAL_ID |  |  | rw | ro |  | 0x0 |  |  | A temporal ID of 4th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 25:20 | REF0_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L0 of 4th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 19:14 | REF1_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L1 of 4th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 13:13 | USE_MULTI_REF_P |  |  | rw | ro |  | 0x0 |  |  | Flag to use multi reference picture for P picture It is valid only if PIC_TYPE is P{has_reset=0} |  |  |  |  |  |
| 12:7 | PIC_QP |  |  | rw | ro |  | 0x0 |  |  | A quantization parameter of 4th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 6:2 | POC_OFFSET |  |  | rw | ro |  | 0x0 |  |  | A poc offset of 4th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 1:0 | PIC_TYPE |  |  | rw | ro |  | 0x0 |  |  | A picture type of 0th picture in the custom GOP 1 : P picture 2 : B picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.295.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_5 |  |  |  |  |  |  | 0x3A4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 932 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Parameters for the 5th picture of custom GOP (MANDATORY with CUSTOM_GOP_SIZE){alt_reg=CMD_SET_FB_ADDR_CB_BASE6,RET_DEC_PIC_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:26 | TEMPORAL_ID |  |  | rw | ro |  | 0x0 |  |  | A temporal ID of 5th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 25:20 | REF0_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L0 of 5th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 19:14 | REF1_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L1 of 5th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 13:13 | USE_MULTI_REF_P |  |  | rw | ro |  | 0x0 |  |  | Flag to use multi reference picture for P picture It is valid only if PIC_TYPE is P{has_reset=0} |  |  |  |  |  |
| 12:7 | PIC_QP |  |  | rw | ro |  | 0x0 |  |  | A quantization parameter of 5th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 6:2 | POC_OFFSET |  |  | rw | ro |  | 0x0 |  |  | A poc offset of 5th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 1:0 | PIC_TYPE |  |  | rw | ro |  | 0x0 |  |  | A picture type of 0th picture in the custom GOP 1 : P picture 2 : B picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.299.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_6 |  |  |  |  |  |  | 0x3A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 936 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Parameters for the 6th picture of custom GOP (MANDATORY with CUSTOM_GOP_SIZE){alt_reg=CMD_SET_FB_ADDR_CR_BASE6,CMD_SET_FB_ADDR_FBC_Y_OFFSET6,RET_ENC_CORE_IDC} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:26 | TEMPORAL_ID |  |  | rw | ro |  | 0x0 |  |  | A temporal ID of 6th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 25:20 | REF0_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L0 of 6th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 19:14 | REF1_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L1 of 6th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 13:13 | USE_MULTI_REF_P |  |  | rw | ro |  | 0x0 |  |  | Flag to use multi reference picture for P picture It is valid only if PIC_TYPE is P{has_reset=0} |  |  |  |  |  |
| 12:7 | PIC_QP |  |  | rw | ro |  | 0x0 |  |  | A quantization parameter of 6th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 6:2 | POC_OFFSET |  |  | rw | ro |  | 0x0 |  |  | A poc offset of 6th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 1:0 | PIC_TYPE |  |  | rw | ro |  | 0x0 |  |  | A picture type of 0th picture in the custom GOP 1 : P picture 2 : B picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.302.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_CUSTOM_GOP_PIC_PARAM_7 |  |  |  |  |  |  | 0x3AC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 940 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Parameters for the 7th picture of custom GOP (MANDATORY with CUSTOM_GOP_SIZE){alt_reg=CMD_SET_FB_ADDR_FBC_C_OFFSET6,RET_HOST_CMD_WARN_INFO} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:26 | TEMPORAL_ID |  |  | rw | ro |  | 0x0 |  |  | A temporal ID of 7th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 25:20 | REF0_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L0 of 7th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 19:14 | REF1_POC |  |  | rw | ro |  | 0x0 |  |  | A poc offset of reference L1 of 7th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 13:13 | USE_MULTI_REF_P |  |  | rw | ro |  | 0x0 |  |  | Flag to use multi reference picture for P picture It is valid only if PIC_TYPE is P{has_reset=0} |  |  |  |  |  |
| 12:7 | PIC_QP |  |  | rw | ro |  | 0x0 |  |  | A quantization parameter of 7th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 6:2 | POC_OFFSET |  |  | rw | ro |  | 0x0 |  |  | A poc offset of 7th picture in the custom GOP{has_reset=0} |  |  |  |  |  |
| 1:0 | PIC_TYPE |  |  | rw | ro |  | 0x0 |  |  | A picture type of 0th picture in the custom GOP 1 : P picture 2 : B picture{has_reset=0} |  |  |  |  |  |

| 1.1.1.304.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL6 |  |  |  |  |  |  | 0x3B0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 944 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index 6{alt_reg=RET_HOST_CMD_ERR_INFO} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE6 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.306.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE6 |  |  |  |  |  |  | 0x3B4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 948 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 6{alt_reg=RET_HOST_CMD_SUCCESS} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SUB_SAMPLED_FB_BASE6 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frame 6{has_reset=0} |  |  |  |  |  |

| 1.1.1.308.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_LUMA_BASE7 |  |  |  |  |  |  | 0x3B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 952 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Luma base of index 7{alt_reg=RET_ENC_SUM_ME0_X_DIR_LOWER} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | LUMA_BASE7 |  |  | rw | ro |  | 0x0 |  |  | Luma base address of DPB index 7 Compressed Luma base address of DPB index 7{has_reset=0} |  |  |  |  |  |

| 1.1.1.310.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CB_BASE7 |  |  |  |  |  |  | 0x3BC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 956 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cb base of index7{alt_reg=RET_ENC_SUM_ME0_X_DIR_HIGHER} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CB_BASE7 |  |  | rw | ro |  | 0x0 |  |  | Cb base address of DPB index 7 Compressed Cb and Cr base address of DPB index 7{has_reset=0} |  |  |  |  |  |

| 1.1.1.313.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_CR_BASE7 |  |  |  |  |  |  | 0x3C0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 960 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Cr base of index 7{alt_reg=CMD_SET_FB_ADDR_FBC_Y_OFFSET7,RET_ENC_SUM_ME0_Y_DIR_LOWER} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | CR_BASE7 |  |  | rw | ro |  | 0x0 |  |  | Cr base address of DPB index 7{has_reset=0} |  |  |  |  |  |

| 1.1.1.315.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_FBC_C_OFFSET7 |  |  |  |  |  |  | 0x3C4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 964 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| FBC chroma offset base of index 7{alt_reg=RET_ENC_SUM_ME0_Y_DIR_HIGHER} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | FBC_CHROMA_OFFSET_BASE7 |  |  | rw | ro |  | 0x0 |  |  | If FBC is used, this is chroma offset base address of DPB index 7. Otherwise, it is ignored.{has_reset=0} |  |  |  |  |  |

| 1.1.1.317.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_COL7 |  |  |  |  |  |  | 0x3C8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 968 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Info base of index 7{alt_reg=RET_ENC_SUM_ME1_X_DIR_LOWER} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | COL_BASE7 |  |  | rw | ro |  | 0x0 |  |  | Codec dedicated information buffer for encoding/decoding HEVC/AVC: MV COL AV1 : CDF{has_reset=0} |  |  |  |  |  |

| 1.1.1.320.0 |  |  |  |  |  |  |  | CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE7 |  |  |  |  |  |  | 0x3CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 972 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Base address of sub-sampled frame buffer for index 7{alt_reg=RET_DEC_WARN_INFO,RET_ENC_SUM_ME1_X_DIR_HIGHER} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | SUB_SAMPLED_FB_BASE7 |  |  | rw | ro |  | 0x0 |  |  | A base address of frame buffer for sub sampled decoded frame 7{has_reset=0} |  |  |  |  |  |

| 1.1.1.324.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_TILE_PARAM |  |  |  |  |  |  | 0x3D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 976 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_TILE_PARAM{alt_reg=CMD_SET_FB_ADDR_LUMA_BASE8,RET_DEC_ERR_INFO,RET_ENC_SUM_ME1_Y_DIR_LOWER} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:8 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 7:4 | TILE_NUM_IN_ROW_M1 |  |  | rw | ro |  | 0x0 |  |  | It specifies the number of tile rows minus 1. (0 ~ 15){has_reset=0} |  |  |  |  |  |
| 3:0 | TILE_NUM_IN_COL_M1 |  |  | rw | ro |  | 0x0 |  |  | It specifies the number of tile columns minus 1. (0 ~ 15){has_reset=0} |  |  |  |  |  |

| 1.1.1.328.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_0 |  |  |  |  |  |  | 0x3D4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 980 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_0{alt_reg=CMD_SET_FB_ADDR_CB_BASE8,RET_DEC_DECODING_SUCCESS,RET_ENC_SUM_ME1_Y_DIR_HIGHER} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.331.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_1 |  |  |  |  |  |  | 0x3D8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 984 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_1{alt_reg=CMD_SET_FB_ADDR_CR_BASE8,CMD_SET_FB_ADDR_FBC_Y_OFFSET8} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.333.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_2 |  |  |  |  |  |  | 0x3DC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 988 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_2{alt_reg=CMD_SET_FB_ADDR_FBC_C_OFFSET8} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.335.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_3 |  |  |  |  |  |  | 0x3E0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 992 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_3{alt_reg=CMD_SET_FB_ADDR_COL8} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.337.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_4 |  |  |  |  |  |  | 0x3E4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 996 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_4{alt_reg=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE8} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.339.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_5 |  |  |  |  |  |  | 0x3E8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1000 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_5{alt_reg=CMD_SET_FB_ADDR_LUMA_BASE9} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.341.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_6 |  |  |  |  |  |  | 0x3EC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1004 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_6{alt_reg=CMD_SET_FB_ADDR_CB_BASE9} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.344.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_7 |  |  |  |  |  |  | 0x3F0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1008 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_7{alt_reg=CMD_SET_FB_ADDR_CR_BASE9,CMD_SET_FB_ADDR_FBC_Y_OFFSET9} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.346.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_8 |  |  |  |  |  |  | 0x3F4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1012 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_8{alt_reg=CMD_SET_FB_ADDR_FBC_C_OFFSET9} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.349.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_9 |  |  |  |  |  |  | 0x3F8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1016 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_9{alt_reg=CMD_SET_FB_ADDR_COL9,RET_ENC_SRC_DEBUG_0} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.352.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_10 |  |  |  |  |  |  | 0x3FC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1020 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_10{alt_reg=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE9,RET_ENC_SRC_DEBUG_1} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.354.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_11 |  |  |  |  |  |  | 0x400 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1024 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_11{alt_reg=CMD_SET_FB_ADDR_LUMA_BASE10} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.356.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_12 |  |  |  |  |  |  | 0x404 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1028 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_12{alt_reg=CMD_SET_FB_ADDR_CB_BASE10} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.359.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_13 |  |  |  |  |  |  | 0x408 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1032 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_13{alt_reg=CMD_SET_FB_ADDR_CR_BASE10,CMD_SET_FB_ADDR_FBC_Y_OFFSET10} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.361.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_14 |  |  |  |  |  |  | 0x40C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1036 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_14{alt_reg=CMD_SET_FB_ADDR_FBC_C_OFFSET10} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.363.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_15 |  |  |  |  |  |  | 0x410 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1040 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_15{alt_reg=CMD_SET_FB_ADDR_COL10} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.365.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_16 |  |  |  |  |  |  | 0x414 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1044 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_16{alt_reg=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE10} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.367.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_17 |  |  |  |  |  |  | 0x418 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1048 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_17{alt_reg=CMD_SET_FB_ADDR_LUMA_BASE11} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.369.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_18 |  |  |  |  |  |  | 0x41C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1052 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_18{alt_reg=CMD_SET_FB_ADDR_CB_BASE11} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.372.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_19 |  |  |  |  |  |  | 0x420 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1056 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_19{alt_reg=CMD_SET_FB_ADDR_CR_BASE11,CMD_SET_FB_ADDR_FBC_Y_OFFSET11} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.374.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_20 |  |  |  |  |  |  | 0x424 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1060 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_20{alt_reg=CMD_SET_FB_ADDR_FBC_C_OFFSET11} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.376.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_21 |  |  |  |  |  |  | 0x428 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1064 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_21{alt_reg=CMD_SET_FB_ADDR_COL11} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.378.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_22 |  |  |  |  |  |  | 0x42C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1068 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_22{alt_reg=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE11} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.380.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_23 |  |  |  |  |  |  | 0x430 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1072 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_23{alt_reg=CMD_SET_FB_ADDR_LUMA_BASE12} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.382.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_24 |  |  |  |  |  |  | 0x434 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1076 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_24{alt_reg=CMD_SET_FB_ADDR_CB_BASE12} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.385.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_25 |  |  |  |  |  |  | 0x438 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1080 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_25{alt_reg=CMD_SET_FB_ADDR_CR_BASE12,CMD_SET_FB_ADDR_FBC_Y_OFFSET12} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.387.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_26 |  |  |  |  |  |  | 0x43C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1084 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_26{alt_reg=CMD_SET_FB_ADDR_FBC_C_OFFSET12} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.389.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_27 |  |  |  |  |  |  | 0x440 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1088 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_27{alt_reg=CMD_SET_FB_ADDR_COL12} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.391.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_28 |  |  |  |  |  |  | 0x444 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1092 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_28{alt_reg=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE12} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.393.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_29 |  |  |  |  |  |  | 0x448 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1096 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_29{alt_reg=CMD_SET_FB_ADDR_LUMA_BASE13} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.395.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_30 |  |  |  |  |  |  | 0x44C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1100 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_30{alt_reg=CMD_SET_FB_ADDR_CB_BASE13} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.398.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_31 |  |  |  |  |  |  | 0x450 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1104 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_31{alt_reg=CMD_SET_FB_ADDR_CR_BASE13,CMD_SET_FB_ADDR_FBC_Y_OFFSET13} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.400.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_32 |  |  |  |  |  |  | 0x454 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1108 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_32{alt_reg=CMD_SET_FB_ADDR_FBC_C_OFFSET13} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.402.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_33 |  |  |  |  |  |  | 0x458 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1112 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_33{alt_reg=CMD_SET_FB_ADDR_COL13} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.404.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_34 |  |  |  |  |  |  | 0x45C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1116 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_34{alt_reg=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE13} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.406.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_35 |  |  |  |  |  |  | 0x460 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1120 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_35{alt_reg=CMD_SET_FB_ADDR_LUMA_BASE14} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.408.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_36 |  |  |  |  |  |  | 0x464 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1124 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_36{alt_reg=CMD_SET_FB_ADDR_CB_BASE14} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.411.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_37 |  |  |  |  |  |  | 0x468 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1128 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_37{alt_reg=CMD_SET_FB_ADDR_CR_BASE14,CMD_SET_FB_ADDR_FBC_Y_OFFSET14} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.413.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_38 |  |  |  |  |  |  | 0x46C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1132 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_38{alt_reg=CMD_SET_FB_ADDR_FBC_C_OFFSET14} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.415.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_39 |  |  |  |  |  |  | 0x470 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1136 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_39{alt_reg=CMD_SET_FB_ADDR_COL14} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.417.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_40 |  |  |  |  |  |  | 0x474 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1140 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_40{alt_reg=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE14} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.419.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_41 |  |  |  |  |  |  | 0x478 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1144 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_41{alt_reg=CMD_SET_FB_ADDR_LUMA_BASE15} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.421.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_42 |  |  |  |  |  |  | 0x47C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1148 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_42{alt_reg=CMD_SET_FB_ADDR_CB_BASE15} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.424.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_43 |  |  |  |  |  |  | 0x480 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1152 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_43{alt_reg=CMD_SET_FB_ADDR_CR_BASE15,CMD_SET_FB_ADDR_FBC_Y_OFFSET15} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.426.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_44 |  |  |  |  |  |  | 0x484 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1156 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_44{alt_reg=CMD_SET_FB_ADDR_FBC_C_OFFSET15} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.428.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_45 |  |  |  |  |  |  | 0x488 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1160 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_45{alt_reg=CMD_SET_FB_ADDR_COL15} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.430.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_46 |  |  |  |  |  |  | 0x48C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1164 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_46{alt_reg=CMD_SET_FB_ADDR_SUB_SAMPLED_FB_BASE15} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.431.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_47 |  |  |  |  |  |  | 0x490 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1168 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_47 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.433.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_48 |  |  |  |  |  |  | 0x494 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1172 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_48{alt_reg=CMD_SET_FB_ADDR_DEFAULT_CDF} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.435.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_49 |  |  |  |  |  |  | 0x498 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1176 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_49{alt_reg=CMD_SET_FB_ADDR_SEGMAP} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.437.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_50 |  |  |  |  |  |  | 0x49C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1180 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_50{alt_reg=CMD_SET_FB_DEC_PP_PVRIC_CTRL} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.438.0 |  |  |  |  |  |  |  | CMD_ENC_CUSTOM_LAMBDA_DATA_51 |  |  |  |  |  |  | 0x4A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1184 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_CUSTOM_LAMBDA_DATA_51 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:21 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 20:7 | LAMBDA_SSD |  |  | rw | ro |  | 0x0 |  |  | lambda_ssd data{has_reset=0} |  |  |  |  |  |
| 6:0 | LAMBDA_SAD |  |  | rw | ro |  | 0x0 |  |  | lambda_sad data{has_reset=0} |  |  |  |  |  |

| 1.1.1.440.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_TEMPORAL_LAYER_0_QP |  |  |  |  |  |  | 0x4A4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1188 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_TEMPORAL_LAYER_0_QP{alt_reg=CMD_SET_FB_DEC_PP_SCL_PARAM} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:18 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 17:12 | B_QP |  |  | rw | ro |  | 0x0 |  |  | A QP value of B Pictures{has_reset=0} |  |  |  |  |  |
| 11:6 | P_QP |  |  | rw | ro |  | 0x0 |  |  | A QP value of P Pictures{has_reset=0} |  |  |  |  |  |
| 5:0 | I_QP |  |  | rw | ro |  | 0x0 |  |  | A QP value of I Pictures{has_reset=0} |  |  |  |  |  |

| 1.1.1.441.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_TEMPORAL_LAYER_1_QP |  |  |  |  |  |  | 0x4A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1192 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_TEMPORAL_LAYER_1_QP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:18 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 17:12 | B_QP |  |  | rw | ro |  | 0x0 |  |  | A QP value of B Pictures{has_reset=0} |  |  |  |  |  |
| 11:6 | P_QP |  |  | rw | ro |  | 0x0 |  |  | A QP value of P Pictures{has_reset=0} |  |  |  |  |  |
| 5:0 | I_QP |  |  | rw | ro |  | 0x0 |  |  | A QP value of I Pictures{has_reset=0} |  |  |  |  |  |

| 1.1.1.442.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_TEMPORAL_LAYER_2_QP |  |  |  |  |  |  | 0x4AC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1196 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_TEMPORAL_LAYER_2_QP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:18 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 17:12 | B_QP |  |  | rw | ro |  | 0x0 |  |  | A QP value of B Pictures{has_reset=0} |  |  |  |  |  |
| 11:6 | P_QP |  |  | rw | ro |  | 0x0 |  |  | A QP value of P Pictures{has_reset=0} |  |  |  |  |  |
| 5:0 | I_QP |  |  | rw | ro |  | 0x0 |  |  | A QP value of I Pictures{has_reset=0} |  |  |  |  |  |

| 1.1.1.443.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_TEMPORAL_LAYER_3_QP |  |  |  |  |  |  | 0x4B0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1200 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_TEMPORAL_LAYER_3_QP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:18 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 17:12 | B_QP |  |  | rw | ro |  | 0x0 |  |  | A QP value of B Pictures{has_reset=0} |  |  |  |  |  |
| 11:6 | P_QP |  |  | rw | ro |  | 0x0 |  |  | A QP value of P Pictures{has_reset=0} |  |  |  |  |  |
| 5:0 | I_QP |  |  | rw | ro |  | 0x0 |  |  | A QP value of I Pictures{has_reset=0} |  |  |  |  |  |

| 1.1.1.444.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_SCL_SRC_SIZE |  |  |  |  |  |  | 0x4B4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1204 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_SCL_SRC_SIZE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | PIC_HEIGHT |  |  | rw | ro |  | 0x0 |  |  | A height of picture in pixel (128 ~ 8192) (before scaler){has_reset=0} |  |  |  |  |  |
| 15:0 | PIC_WIDTH |  |  | rw | ro |  | 0x0 |  |  | A width of picture in pixel (256 ~ 8192) (before scaler){has_reset=0} |  |  |  |  |  |

| 1.1.1.445.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_SCL_PARAM |  |  |  |  |  |  | 0x4B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1208 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_SCL_PARAM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:3 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | RSVD{has_reset=0} |  |  |  |  |  |
| 2:1 | SCL_COEF_MODE |  |  | rw | ro |  | 0x0 |  |  | Scaler coeff mode 0:bicubic 0.5 1:bicubic 1.0 2:bicubic 1.5 3:lanczos{has_reset=0} |  |  |  |  |  |
| 0:0 | SCL_EN |  |  | rw | ro |  | 0x0 |  |  | Scaler enable flag (0 : off, 1: on){has_reset=0} |  |  |  |  |  |

| 1.1.1.446.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_Y2Y_PARAM |  |  |  |  |  |  | 0x4BC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1212 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_Y2Y_PARAM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:2 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | RSVD{has_reset=0} |  |  |  |  |  |
| 1:0 | Y2Y_MODE |  |  | rw | ro |  | 0x0 |  |  | Y2Y mode 0: off 1: Y only 2: Chroma only 3: Y/Chorma{has_reset=0} |  |  |  |  |  |

| 1.1.1.447.0 |  |  |  |  |  |  |  | CMD_SET_FB_DEC_PP_CROP_PARAM |  |  |  |  |  |  | 0x4C0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1216 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Crop control parameter (Post processing) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:1 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 0:0 | CROP_EN |  |  | rw | ro |  | 0x0 |  |  | Crop enable flag 0 : disable 1 : enable{has_reset=0} |  |  |  |  |  |

| 1.1.1.449.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_SFS_PARAM |  |  |  |  |  |  | 0x4C4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1220 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_SFS_PARAM{alt_reg=CMD_SET_FB_DEC_PP_CROP_POS} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:2 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 1:1 | SUB_FRAME_SYNC_TYPE |  |  | rw | ro |  | 0x0 |  |  | 0 : signal(wired) based, 1: register based{has_reset=0} |  |  |  |  |  |
| 0:0 | SUB_FRAME_SYNC_ON |  |  | rw | ro |  | 0x0 |  |  | 0 : off, 1: on{has_reset=0} |  |  |  |  |  |

| 1.1.1.451.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_CROP_ENABLE |  |  |  |  |  |  | 0x4C8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1224 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_CROP_ENABLE{alt_reg=CMD_SET_FB_DEC_PP_CROP_SIZE} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:1 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | reserved{has_reset=0} |  |  |  |  |  |
| 0:0 | CROP_EN |  |  | rw | ro |  | 0x0 |  |  | Crop enable flag (0 : off, 1: on){has_reset=0} |  |  |  |  |  |

| 1.1.1.453.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_CROP_START_POS |  |  |  |  |  |  | 0x4CC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1228 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_CROP_START_POS{alt_reg=CMD_SET_FB_DEC_PP_AFBC_COMMON} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | CROP_START_X |  |  | rw | ro |  | 0x0 |  |  | start X position for crop (AFBCD or Crop ){has_reset=0} |  |  |  |  |  |
| 15:0 | CROP_START_Y |  |  | rw | ro |  | 0x0 |  |  | start Y position for crop (AFBCD or Crop ){has_reset=0} |  |  |  |  |  |

| 1.1.1.455.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_CROP_SRC_SIZE |  |  |  |  |  |  | 0x4D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1232 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_CROP_SRC_SIZE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:16 | PIC_HEIGHT |  |  | rw | ro |  | 0x0 |  |  | A height of picture in pixel (128 ~ 8192) (before crop){has_reset=0} |  |  |  |  |  |
| 15:0 | PIC_WIDTH |  |  | rw | ro |  | 0x0 |  |  | A width of picture in pixel (256 ~ 8192) (before crop){has_reset=0} |  |  |  |  |  |

| 1.1.1.456.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_Y2Y_DATA_0 |  |  |  |  |  |  | 0x4D8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1240 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_Y2Y_DATA_0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:20 | LUMA_OUT_MIN |  |  | rw | ro |  | 0x0 |  |  | Y2Y luma output min value 8bit: 0 ~ 255 10 bit : 0 ~ 1023{has_reset=0} |  |  |  |  |  |
| 19:10 | LUMA_IN_MAX |  |  | rw | ro |  | 0x0 |  |  | Y2Y luma input max value 8bit: 0 ~ 255 10 bit : 0 ~ 1023{has_reset=0} |  |  |  |  |  |
| 9:0 | LUMA_IN_MIN |  |  | rw | ro |  | 0x0 |  |  | Y2Y luma input min value 8bit: 0 ~ 255 10 bit : 0 ~ 1023{has_reset=0} |  |  |  |  |  |

| 1.1.1.458.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_Y2Y_DATA_1 |  |  |  |  |  |  | 0x4DC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1244 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_Y2Y_DATA_1{alt_reg=CMD_SET_FB_DEC_ADDR_COL_DUAL} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:30 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 29:20 | CHR_IN_MAX |  |  | rw | ro |  | 0x0 |  |  | Y2Y Chroma input max value 8bit: 0 ~ 255 10 bit : 0 ~ 1023{has_reset=0} |  |  |  |  |  |
| 19:10 | CHR_IN_MIN |  |  | rw | ro |  | 0x0 |  |  | Y2Y Chroma input min value 8bit: 0 ~ 255 10 bit : 0 ~ 1023{has_reset=0} |  |  |  |  |  |
| 9:0 | LUMA_OUT_MAX |  |  | rw | ro |  | 0x0 |  |  | Y2Y luma output max value 8bit: 0 ~ 255 10 bit : 0 ~ 1023{has_reset=0} |  |  |  |  |  |

| 1.1.1.459.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_Y2Y_DATA_2 |  |  |  |  |  |  | 0x4E0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1248 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_Y2Y_DATA_2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:20 | RESERVED0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 19:10 | CHR_OUT_MAX |  |  | rw | ro |  | 0x0 |  |  | Y2Y Chroma output max value 8bit: 0 ~ 255 10 bit : 0 ~ 1023{has_reset=0} |  |  |  |  |  |
| 9:0 | CHR_OUT_MIN |  |  | rw | ro |  | 0x0 |  |  | Y2Y Chroma output min value 8bit: 0 ~ 255 10 bit : 0 ~ 1023{has_reset=0} |  |  |  |  |  |

| 1.1.1.461.0 |  |  |  |  |  |  |  | CMD_ENC_SEQ_INPUT_RINGBUF |  |  |  |  |  |  | 0x4E4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1252 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| CMD_ENC_SEQ_INPUT_RINGBUF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 0:0 | INPUT_RINGBUF_EN |  |  | rw | ro |  | 0x0 |  |  | 0 : off, 1 : on{has_reset=0} |  |  |  |  |  |

| 1.1.1.462.0 |  |  |  |  |  |  |  | VPU_PDBG_CTRL |  |  |  |  |  |  | 0x600 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1536 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugger control (for debugger only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:4 | RSVD0 |  |  | wo | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 3:3 | IMMBRK |  |  | wo | ro |  | 0x0 |  |  | Immediate break It forces to stop V-CPU and enter in a debug mode to analyze hang-up situation. V-CPU cannot resume from the break point.{has_reset=0} |  |  |  |  |  |
| 2:2 | STABLEBRK |  |  | wo | ro |  | 0x0 |  |  | Stable break It is an external break request when V-CPU is in stable (breakable) status.{has_reset=0} |  |  |  |  |  |
| 1:1 | RESUME |  |  | wo | ro |  | 0x0 |  |  | Resume It resumes from the breakpoint.{has_reset=0} |  |  |  |  |  |
| 0:0 | STEP |  |  | wo | ro |  | 0x0 |  |  | Step It runs V-CPU in step instruction mode.{has_reset=0} |  |  |  |  |  |

| 1.1.1.463.0 |  |  |  |  |  |  |  | VPU_PDBG_IDX_REG |  |  |  |  |  |  | 0x604 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1540 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| V-CPU debugger index register (For debugger only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:10 | RSVD0 |  |  | wo | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 9:9 | RDDBG |  |  | wo | ro |  | 0x0 |  |  | Read Operation Request RDDBG and WRDBG cannot be 1 at the same time.{has_reset=0} |  |  |  |  |  |
| 8:8 | WRDBG |  |  | wo | ro |  | 0x0 |  |  | Write Operation Request RDDBG and WRDBG cannot be 1 at the same time.{has_reset=0} |  |  |  |  |  |
| 7:0 | DBGIDX |  |  | wo | ro |  | 0x0 |  |  | Debug Index Debugger Register Index{has_reset=0} |  |  |  |  |  |

| 1.1.1.464.0 |  |  |  |  |  |  |  | VPU_PDBG_WDATA_REG |  |  |  |  |  |  | 0x608 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1544 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| V-CPU debugger write data register (For debugger only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | VPU_PDBG_WDATA_REG |  |  | wo | ro |  | 0x0 |  |  | To write data to the debugger, # Write some data to this register. # Write VCPU_PDBG_IDX_REG with WRDBG as 1 and DBGIDX as this register address. # After writing is completed, VPU_PDBG_WDATA_REG will be cleared.{has_reset=0} |  |  |  |  |  |

| 1.1.1.465.0 |  |  |  |  |  |  |  | VPU_PDBG_RDATA_REG |  |  |  |  |  |  | 0x60C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1548 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| V-CPU debugger read data register (For debugger only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:0 | VPU_PDBG_RDATA_REG |  |  | ro | ro |  | 0x0 |  |  | To read data to the debugger, # Write VCPU_PDBG_IDX_REG with RDDBG as 1 and DBGIDX as the register address to be read. # Read from the specified register to this register.{has_reset=0} |  |  |  |  |  |

|  |  |  |  |  |  |  |  | VPU_PDBG_STEP_MASK |  |  |  |  |  |  | 0x610 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| offset |  | 1552 | external |  |  |  |  |  | size |  | 32 |  |  |  |  |
| Debugger control (for debugger only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bits | name |  |  | s/w | h/w |  | default |  |  | description |  |  |  |  |  |
| 31:1 | RSVD0 |  |  | rw | ro |  | 0x0 |  |  | Reserved{has_reset=0} |  |  |  |  |  |
| 0:0 | STEP_MASK_ENABLE |  |  | rw | ro |  | 0x0 |  |  | Interrupt Disable at step for debugger{has_reset=0} |  |  |  |  |  |

End RegGroup

End RegGroup